Sean Leach (2004817)

University of Wales Trinity Saint David

Assignment 1

ABCA 5010

Mr Doe

6A Prospect Pl,

Swansea,

SA1 1QP

01 March 2022

Dear Mr Doe,

I hope you are well. I am writing to you in response to your previous letter.

Throughout this letter I will discuss our standards for Building Information Modelling (BIM) for this project, with a focus on level 2 maturity.  I will highlight the measures that are being taken by our team working on this project to ensure level 2 BIM maturity requirements are being met with the relevant standards stated.

Building information modelling, more commonly referred to as BIM, is a process for creating and managing information on a construction project through its whole life cycle. A digital description is made for every aspect of the building and, it is very likely that, included in the digital description are 3D models and associated data such as product and hand over information. A detailed video can be found using the reference website link to give you a greater understanding of BIM in general, (NBS, 2016).

To ensure BIM level 2 maturity is being complied with, during the preconstruction stage we will follow the CIC BIM protocol.  This ensures that there is a contractual obligation on both the Employer and Project Team Members in connection to Project Information and sets out each parties’ rights and liabilities.

To outline a foundational framework and to ensure successful deployment of advanced digital technologies and modelling, a BIM Execution Plan (BEP) will be produced.  Included in the BEP are:

- Clearly defined roles and responsibilities for each team/party.

- Project Information

- Planning and Documentation

- Standards, Methods and Procedures used

- IT Solutions

For more detail, please see a copy of the draft BEP plan attached below to the back of this letter.

As proposed in the CIC BIM protocol, a Common Data Environment (CDE) will be created by the appointed Information Manager. The CDE shall be used by all relevant members working on the project.  All construction project information shall be housed here and be accessible in accordance with BS EN ISO 19650. A central project server shall be used to operate the CDE. Although all files of information will be centralised and accessible for all, the party who submitted the information will retain ownership and be responsible for it. This means all individual models have clear authorship and remain separate, ensuring liabilities remain with the original creators.  This does not change even if their model is used in a federated model.

All parties contributing to the CDE will comply with file naming conventions covered in BS EN ISO 19650-1:2018. The appointed Information Manager shall ensure agreed protocols are followed by all parties, maintaining BIM level 2 compliance.

The structure to be used for information being admitted into the CDE can been seen in the illustration below which has been taken from BS EN ISO 19650-1 pt12,

(Image taken from BS EN ISO 19650-1 pt12)

- Individual parties create a Work in Progress (WIP) within their own data environment. This is then signed off by the lead designer after relevant checks have been done, and if compliant admitted to the CDE.

- Once checked and admitted to the CDE other parties will be able to use this data to reference with their own designs.

- Once all information has been input and completed, it is then submitted to the client shared data area to be authorised. After being checked by the client or their representative for compliance with the EIR and any questions answered, it can then be signed off and published.

- Published information can then be used for construction. When no longer needed the information can be digitally archived and kept as a record for key milestones.

BS EN ISO 19650-1 pt12 states the requirements in further detail with relevant sub paragraphs on all stages shown in the image above. (McPartland, 2016) and (B.S, 2022) are good sources of information for covering the common data environment.

PAS 1192-5:2015 outlines details for the approach to applying appropriate measures to manage the security risks that affect built assets, asset data and asset information.  This is free to view online; however, BS EN ISO 19650 has since superseded it, but this is not free to view. The appointed CDE Information Manager will be responsible for all ensuring that the CDE security approach is in line with BS EN ISO 19650-5:2020.

Specifications of products and materials used throughout the design and build of the project will be specified within the 3D models where possible. However, all product, material information and specifications will be recorded in the Construction Operations Building Information Exchange (COBie). This is a centralised spreadsheet where all non-graphical data will be stored. BS 1192-4:2014 is the code of practice that outlines the methodology to transfer information relating to facilities, including buildings and structures. A company policy to meet any Exchange Information Requirements (EIR) you set out is that we ensure information stored in the COBie will be concise, unambiguous, and easily accessible.  When the construction of the project is complete and handed over to the clients, the COBie will hold all the information required for the maintenance and life cycle of the building.  For example if a light needed changing, you would be able to find all information and specifications for that light on the COBie.  Maintenance schedules, product data sheets, spare parts list and warranties are a few other examples of information that will be stored on the COBie. (wbgd, 2022) and (Hamil, 2018) were good sources of information for this.

As part of the UK BIM framework and in line with the UK Government Soft Landing (GSL) policy a period of after care will be given in line with the code of practice BS 8536-1. This will ensure a smooth hand over to the clients and provide the users with the best possible experience. You can find further information of GSL at the reference provided, (UK BIM Framework, 2019).

I hope this clarifies the steps we are taking to ensure BIM level 2 maturity Is being met.  For any further information or questions on any of the points raised please do get in touch.

Yours sincerely,

Sean Leach

Lead Consultant

Empire Consultants

2004817@student.uwtsd.ac.uk

Appendix 1

Draft BIM Execution Plan, See below.

Draft BIM Execution Plan

Ysgol Bae Abertawe

Contents

1	Introduction	7

1.1	Project Information	7

1.2	Successful Information Management	8

1.3	Our Strategic BIM Goals	9

1.4	Information required by the EIR	9

2	Project Management	10

2.1	Management Contacts	10

2.2	Management of BIM Objectives	11

2.3	Major Project Milestones	11

2.4	Model Quality Management	12

2.5	Model Control Flow Diagram	13

3	Planning and Documentation	14

3.1	Agreed project processes for collaboration	14

3.2	Clash detection software	14

3.3	Agreed Authority	15

3.4	Example - Matrix of responsibility	16

3.5	LOD – Level of Detail	16

4	Standards, Methods and Procedures used	17

4.1	File Naming Standard	18

4.2	Coordinate System	18

5	IT Solutions	19

5.1	Software Solutions	19

5.2	Exchange Formats	19

5.3	Specified Hardware	20

# 1	Introduction

This BIM Execution Plan (BEP) intends to define a foundational framework that ensures design technologies are successfully deployed on this project.

## 1.1	Project Information

Table 1 - Project information

| Project Title | Ysgol Bae Abertawe |
| --- | --- |
| Project Description | 1 form entry infant school for 90 pupils, with 48 PT nursey provision. |
| Project Number | 5010 |
| Project Design Start Date: | 26/04/2022 |
| Project Construction Start Date: | 21/10/2022 |
| Project Completion: | 19/02/2024 |
| Project Location | SA1 |
| Project Language | English (UK) |

## 1.2	Successful Information Management

A successful Project requires a smart plan, lean scope, collaborative processes, team agreement, supporting technology and solid tracking, control and verification workflows.

Our project execution planning ensures that all parties are clearly aware of the opportunities and responsibilities associated with the incorporation of Building Information Modelling (BIM) into our project.

Our Plan details the correct processes for executing BIM throughout the project’s lifecycle.

On this project we will follow these four steps to ensure that all teams are meeting the project's Exchange Information Requirements:

- Identify high value Model/BIM Uses that will support Exchange Information Requirements

- Design and document optimal project execution processes

- Define the BIM deliverables in the form of information exchanges

- Develop the infrastructure in the form of contracts, communication procedures, technology and quality control to support the implementation

To gain the maximum benefits from our BIM implementation, once the Plan and Scope have been defined and appointed, the Task Teams will Schedule their own tasks.  Our team will Track the progress of assigned tasks and Verify task completeness before sharing the model for the described use case.

(Plannerly, 2022)

## 1.3	Our Strategic BIM Goals

Strategic goals for the use of BIM during the design and construction phases:

- Create a standardised data structure that allows data to be reused, and eliminates redundant and conflicting information

- Decrease waste by utilising a virtual building process to simulate design & construction activities

- Reduce capital expenditure costs with better informed decision making, and greater coordination & collaboration amongst project teams

- Allow the Design and Construction models to be used for building operations without adding significant costs or time to the Design & Construction process

- Visual communication of site conditions for planning and health/safety/risk assessment

- More accurate project phasing and timelines

- More accurate budgeting and cost estimating, ensuring the project is constructed with the least variations / clashes as possible

- Ensure that there is coordinated, quality design documentation to estimate the build cost

- Fully clash detected models that reduce RFI’s requesting data on object location, with an emphasis on clash avoidance

- Utilise 3D models for reporting, review and decision making

- Ensure that the constructability and methodology of the design have been assessed before documents are issued for construction

(Plannerly, 2022)

## Information required by the EIR

BS EN ISO 19650-1 2018 pt12 outlines the standards procedure for Exchange Information Requirements. Once we have received from the client their EIR, more detailed information can be input in the BEP.

We aim to produce good quality information that can be exchanged, integrated and interrogated during project delivery and following Handover.

The EIR will contain strategy, Key Performance Indicators, BIM Uses and desired outcomes from the use of BIM on our project.

The EIR will contain sections completed by the client or their representative, Advisor or Consultant that they have appointed to identify our project specific requirements.

# 2	Project Management

This section outlines key roles and responsibilities undertaken on this project with relevant contact details. It outlines key tables and roles required in BS EN 19650-1 2018 pt7.

## 2.1	Management Contacts

Table 2 - Management contacts

| Responsibility | Company | Name | Email | Number |
| --- | --- | --- | --- | --- |
| Main Contractor | Kier | John | John@kier.com | 07123456785 |
| Lead Architect | DP Architecture | Dwaine | Dwaine@dp.com | 07123456786 |
| Project Manager | Kier | Archie | Archie@Kier.com | 07890987656 |
| Construction Manager | Kier | Daniel | Daniel@Kier.com | 07777477774 |
| Structural Engineer | Kier | Dave | Dave@kier.com | 07123456787 |
| Mechanical Engineer | Kier | Paul | Paul@kier.com | 07123456788 |
| Electrical Engineer | RDM | Steve | Steve@RDM.com | 07123456789 |
| Fire Protection | AFPS | Louise | Louise@AFPS.com | 07123456784 |
| Lead Consultant | Empire | Sean | Sean@empire.com | 07834679178 |
| BIM Manager | Kier | Kevin | Kevin@kier.com | 07834564133 |
| Client | Swansea Council | Basil | Basil@swanseacouncil.com | 07654567876 |
| Facilities Manager` | Kier | Bryan | Bryan@kier.com | 07098789142 |

## 2.2	Management of BIM Objectives

Table 3 – Management of BIM objectives

| Objective | Delivery mechanism | Responsible Party |
| --- | --- | --- |
| 3D visualisation & Stakeholder engagement | Community engagement with the 3D Model | Design Team/ Employer |
| Improved design coordination | Through coordination and clash detection utilising 3D models. | Design Team/ Contractor |
| Cost & risk reduction | Via informed information available through 3D models and information in COBie | Design Team/ Contractor |
| Asset management |  |  |
| Task management | Via informed data driven decisions. | Employer |

## Major Project Milestones

Table 4 – Major Project Milestones

| Start Date | Design Completion | Construction Starts | Construction Complete | As Built Models, Documents and Data | Handover |
| --- | --- | --- | --- | --- | --- |
| 26/04/2022 | 01/10/2022 | 21/10/2023 | 15/01/2024 | 05/02/2024 | 19/02/2024 |

Only major milestones throughout the project have been listed here. A more detailed project plan is to be published later after consultation with relevant stakeholders.

## 2.4	Model Quality Management

Table 5 – Model Quality Management

| Check | Definition | Responsible Party | Software Used | Frequency |
| --- | --- | --- | --- | --- |
| Visual | Ensure there are no unintended model components, and the design intent has been followed | Lead Architect | Revit | Weekly |
| Clash | Detect problems in the model where two building components are clashing including soft and hard | BIM Manager | Revit | TBC |
| Standards | Ensure that the BIM and AEC CADD Standard have been followed (fonts, dimensions, line styles, levels/layers, etc) | Team Leaders/ BIM Manager | Revit | TBC |
| Integrity | Describe the QC validation process used to ensure that the Project Facility Data set has no undefined, incorrectly defined, or duplicated elements and the reporting process on non-compliant elements and corrective action plans | BIM Manager | TBC | TBC |

## 2.5	Model Control Flow Diagram

The diagram explains the process of admitting work into the CDE that has been approved correctly as stated in BS EN ISO 19650-1 2018: pt12

It shows stages, WIP, Shared, Published and Archived.

(Image taken from BS EN ISO 19650 -1 pt 12)

- Individual parties create a Work in Progress (WIP) within their own data environment. This is then signed off by the lead designer after relevant checks have been done, and if compliant, admitted to the CDE.

- Once checked and admitted to the CDE other parties will be able to use this data to reference with their own designs.

- Once all information has been input and completed, it is then submitted to the client shared data area to be authorised. After being checked by the client or their representative for compliance with the EIR and any questions answered, it can then be signed off and published.

- Published information can then be used for construction. When no longer needed the information can be digitally archived and kept as a record for key milestones.

# Planning and Documentation

## 3.1	Agreed project processes for collaboration

Table 6 - Agreed project process for collaboration and information modelling.

| Discipline | Use | Network | Software | Version |
| --- | --- | --- | --- | --- |
| All | BIM Management Platform | 4Projects | Plannerly | Most current version |
| Common Data Environment (CDE) | File Sharing | 4Projects | 4Projects | 2022 |
| Architecture | Design | 4Projects | Revit | 2021 |
| Structure | Design | 4Projects | Revit | 2021 |
| Civil | Design | 4Projects | Revit | 2021 |
| Mechanical | Design | 4Projects | Revit | TBC |
| Clash Detection | All Parties | 4Projects | Solibri | V10. |
| Electrical | Design | TBC | TBC | TBC |
| Plumbing | Design | TBC | TBC | TBC |
| Fire Protection | Design | TBC | TBC | TBC |

## 3.2	Clash detection software

Clash detection software to be used for all models.

Table 7 – Clash detection

| Clash Viewer | Version |
| --- | --- |
| Solibri | V10. |

## 3.3	Agreed Authority

Agreed authority for managers from each company. Authority covers security, extranet access, and authority to distribute documents.

Table 8 – Agreed Authority

| Company/Team | Authorised Manager | Authority |
| --- | --- | --- |
| Kier – Site Team | Archie | Uploads, Downloads and changes to personal documents only. |
| DP - Architect | Dwaine | Uploads, Downloads and changes to personal documents only. |
| Empire – Consultant | Sean | Uploads, Downloads and changes to personal documents only. |
| AFPS – Fire Protection Officer | Louise | Uploads, Downloads and changes to personal documents only. |
| RDM – Electrical | Steve | Uploads, Downloads and changes to personal documents only. |

## 3.4	Example - Matrix of responsibility

Below is an example of what the matrix of responsibility will consist of once we receive a defined EIR.

Table 9 – Example of the Matrix of Responsibility

|  |  |  | Plan of Works |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model Authoring | Native Format | Exchange Format | 1 Brief | 2 Concept | 3 Definition | 4 Design | 5 Build & Commission | 6 Handover & Closeout | 7 Operation |
| Architecture Model | RVT | RVT/ICF | LOD1 | LOD2 | LOD3 | LOD3 | LOD4 | LOD4 | LOD4 |
| Structural Design Model | RVT | RVT/ICF |  | LOD1 | LOD2 | LOD3 | LOD4 | LOD4 | LOD4 |
| HVAC Design Model | RVT | RVT/ICF |  |  | LOD2 | LOD3 | LOD4 | LOD4 | LOD4 |
| Building Services Model | RVT | RVT/ICF |  |  | LOD2 | LOD3 | LOD4 | LOD4 | LOD4 |
| Lighting Design Model | RVT | RVT/ICF |  |  | LOD2 | LOD3 | LOD4 | LOD4 | LOD4 |
| Interior Layout and Design Model | RVT | RVT/ICF | LOD1 | LOD2 | LOD3 | LOD3 | LOD4 | LOD4 | LOD4 |
| Site and Existing Buildings | RVT | RVT/ICF |  | LOD1 | LOD1 | LOD1 | LOD1 | LOD1 | LOD1 |

## 3.5	LOD – Level of Detail

Table 10- Levels of Detail Defined

| LOD | Description | Content |
| --- | --- | --- |
| 1 | Brief | Block model with performance requirements, site constraints |
| 2 | Concept | Concept or massing model including basic areas and volumes, orientation, and cost. In the RIBA Plan of Work, this is equivalent to stage 2 |
| 3 | Developed Design | Generalised systems with approximate quantities, size, shape, location and orientation. Equivalent to RIBA stage 3 |
| 4 | Production | Technical design model, equivalent to RIBA stage 4. Accurate and coordinated modelled elements that can be used to estimate costs and check regulatory compliance. |
| 5 | Installation | Model suitable for fabrication and assembly, with accurate requirements and specific components |
| 6 | As Constructed | Details of how the project has been constructed, for use in operations and maintenance |
| 7 | In Use | Asset information model for operations, maintenance and ongoing monitoring |

# 4	Standards, Methods and Procedures used

Below is a table of standards to be used throughout this project.

Table 11 - Standards

| Function | Standard | Description |
| --- | --- | --- |
| Information management | BS EN ISO 19650 Series | Collaborative production of architectural, engineering and construction information. Organisation and digitisation of information about buildings and civil engineering works, including building information modelling (BIM) |
| Means of structuring and classifying information | # BS EN 17412-1:2020 | Building Information Modelling - Level of Information Need - Part 1: Concepts and principles |
| Accuracy and Tolerance | # BS 5606:2022 | When capturing the existing asset information, the geometry being reproduced shall consider the construction tolerances outlined within BS:5606:2022 |
| Space Numbering | BS EN ISO 4157-2 | Space numbering convention for room names and numbers |
| Naming of Containers | BS EN ISO 19650 | The agreed convention for the naming of information container identification |
| Type and Component Naming | BS EN ISO 4157-1 | The agreed type and component naming convention. All primary designations shall be written in full |
| Document Management | # BS EN ISO 7200:2004 | All exchanged information containers shall comply with BS EN ISO 7200:2004 by including, as a minimum, all of the specified mandatory data fields in title blocks and document headers |
| Management of Assets | BS EN ISO 55000:2014 Series | Guidance on the factors to be considered by an organisation when managing its assets |
| Facilities Maintenance | BS 8210 | Detailed guidance on facilities maintenance management and code of practice |
| Briefing for design and construction | BS 8536 | Recommendations for the briefing for design and construction to ensure that the design takes account of the expected performance of the asset/facility in use over its planned operational life. |

## 4.1	File Naming Standard

Example: 5010-EC-xx-zz-D-A-0001-Site-SO-PO1

Table 12 – File Name Example

| Project | Originator | Functional Break down | Spatial Break down | Form | Discipline | Number | Description | Status | Revision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5010 | EC | XX | ZZ | D | A | 0001 | SITE | SO | PO1 |

The file naming standard is in line with the standards set out in BS EN ISO 19650 Part 1 and will apply to all parties working on this project.

## 4.2	Coordinate System

All models of the building will use the Location at Latitude: 192364°E, Longitude: 265976.9N° in the Architectural model as a shared base point that must be used by the entire project team for building coordination purposes.

A .dwg file named "GRIDS" will locate the building structural grid lines relative to the project origin.

All models of site utilities will use a coordinate system separate from the building coordinate system and will be based on the Civil Site drawings.

The General Contractor will coordinate the placement of this point first and all other models will follow.

Each model will be aligned and rotated so that when exporting to the various shared formats, they will align without requiring the exports to be moved or rotated.

This process will be worked out as the coordination begins and published in a document that can be accessed by the entire team. This will allow for all points in the models to be spatially located in the correct location. Additionally, this will allow for the sharing and use of coordinate point data between all trades for field location and installation.

Note to all Revit users – The origin marker, “Base Point”, and “Shared Coordinate” should all be in the same place in your models.

(Plannerly, 2022)

# 5	IT Solutions

This section aims to define IT software systems to be used by relevant parties throughout this project.

## 5.1	Software Solutions

Below is a table of agreed CAD and modelling software versions to be used throughout the project.

Table 13 – Software

| Discipline | Use | Software | Version |
| --- | --- | --- | --- |
| All | BIM Management Platform | Plannerly | Most current version |
| Common Data Environment (CDE) | File Sharing | 4Projects | 2022 |
| Architecture | Design | Revit | 2021 |
| Structure | Design | Revit | 2021 |
| Civil | Design | Revit | 2021 |
| Clash Detection | All Parties | Solibri | V10. |
| Sub-contractor 1 | Design | TBC | TBC |
| Sub-contractor 2 | Design | TBC | TBC |

## 5.2	Exchange Formats

Table 14 – Exchange Formats

|  | RVT | IFC | PDF | DWG | Excel |
| --- | --- | --- | --- | --- | --- |
| Models | X | X |  |  |  |
| Drawings |  |  | X | X |  |
| Final drawing format |  |  | X | X |  |
| Schedules or spreadsheets |  |  | X |  | X |

## 5.3	Specified Hardware

Table 15 - Hardware

| BIM Use and Owner | Hardware | Image | Specification |
| --- | --- | --- | --- |
| BIM Manager | TBC | TBC | TBC |
| Department Managers | TBC | TBC | TBC |
| Site Team for model viewing | Ipad Pro Series |  | Wi-Fi + Cellular 128GB iPad Pro with UAG Case |

# References

(B.S, 2022) - bimportal.scottishfuturestrust. (2022). Task: Overview of the Common Data Environment (CDE) - BIM Level 2 Guidance. [online] Available at: https://bimportal.scottishfuturestrust.org.uk/level2/stage/1/task/22/overview-of-the-common-data-environment-cde.

App.plannerly.com. (2022). Plannerly. [online] Available at: https://app.plannerly.com/projects/bd814650-c14d-41b3-80a4-83bf417a2e3e/plan [Accessed 10 Mar. 2022].

BSI (2020). Little book of BIM 2020 international edition Includes ISO 19650 standards. [online] Available at: https://www.bsigroup.com/globalassets/localfiles/en-us/brochures/bim/little-book-bim.pdf [Accessed 15 Mar. 2022].

Building Design. (2022). CPD 5 2017: BIM levels of detail. [online] Available at: https://www.bdonline.co.uk/cpd/cpd-5-2017-bim-levels-of-detail/5086956.article. [Accessed 10 Mar. 2022].

Dr Stephen Hamil (2018). What is COBie? [online] NBS. Available at: https://www.thenbs.com/knowledge/what-is-cobie.

Eastman, C.M., Teicholz, P.M., Sacks, R. and Lee, G. (2011). BIM handbook : A guide to building information modeling for owners, managers, designers, engineers and contractors. Hoboken, New Jersey: Wiley.

McPartland, R. (2016). What is the Common Data Environment (CDE)? [online] NBS. Available at: https://www.thenbs.com/knowledge/what-is-the-common-data-environment-cde.

NBS (2014). BIM Levels explained. [online] NBS. Available at: https://www.thenbs.com/knowledge/bim-levels-explained. [Accessed 10 Mar. 2022].

UK BIM Framework (2019). Government Soft Landings. [online] ukbimframewor. Available at: https://ukbimframework.org/wp-content/uploads/2019/11/GSL_Report_PrintVersion.pdf [Accessed 15 Mar. 2022].

wbdg (2022). Construction-Operations Building Information Exchange (COBie) | WBDG - Whole Building Design Guide. [online] Available at: https://www.wbdg.org/bim/cobie.