EXAMINATIONS 2018 / 2019

ARHOLIADAU  2018  / 2019

FACULTY OF: Architecture, Computing & Engineering

| NAME OF COURSE/PROGRAMME: | BSc Engineering Programmes |
| --- | --- |
| YEAR OR LEVEL: | Level 4 |
| NAME OF MODULE/UNIT/EXAM PAPER: | Engineering Science |
| MODULE NO: | M4X02192 |
| DATE OF EXAMINATION: | 03/05/2019 |
| EXAMINATION TIMES: |  |
| START TIME: | 11:00 |
| FINISH TIME: | 13:00 |
| TIME ALLOWED: | 2 hours |

INSTRUCTIONS TO CANDIDATES:

Answer all questions.

g = 9.81m/s2

Give answers to 2.d.p.

You are allowed a scientific calculator and a dictionary.

1.

Find the temperature difference of a water to air intercooler that is 600mm by 465mm, given that the convective heat transfer coefficient of 1100W/m2K and heat transfer of 55kW.

[8 Marks]

Q = h A (T2 – T1)

l=100mm

2.

Fixed Edge

x =2mm

25mm

250N

400mm

Figure 1.

Figure 1 shows an experiment to determine the modulus of rigidity of a sample of copper, a rectangular block of the material, measuring 400 x 100 x 25mm, was firmly fastened, so that it projected a distance of 100mm from a vertical wall.   It was found that when a downward load of 250N was applied to the free vertical edge, there was a vertical deflection of 2mm.  Calculate:

- The shear stress, ,   							     [5 marks]

- The shear strain, ,   							     [4 marks]

- The experimental value of the modulus of rigidity of this copper sample. 	     [2 marks]

=FA 		=Xl

3.

A solid section drive shaft that is 0.75m long is to be subjected to a torque of 1200Nm, which results in 0.0005 radians of rotation, calculate the shaft diameter given the modulus of rigidity of the shaft is 210GN/m2.									  	   [10 marks]

J=D432

4.

An experiment to determine the Young’s modulus of Elasticity of a sample of titanium consisted of a load of 273N being applied to the end of a rod of the material, this resulted in an extension of 0.0035mm.  Given that the rod was 0.25m long and had a diameter of 15mm calculate the value of E.                                                                                                                                                  		     [10 marks]

σ=FA                             ε=xl                      E=σε