Types of Munition of Potential Interest

Mk 83 air dropped munition

- Charge Mass – 225 kg

- Mass of case – 229 kg

- AUW – 454 kg

- Explosive – Comp B

- Average fragment area - 0.48 cm2

- Average fragment mass – 0.115 kg

- Frag drag coefficient – 0.82

- Density of air 1.23 kg m-3

- Frag Vi = 1950 ms-1

- Frag Vf = 1911 ms-1 at 92 m

M-67 grenade

- Charge mass – 185 g

- Mass of case – 213 g

- AUW – 398 g

- Explosive – Comp B

- Average frag area – 1.0 cm2

- Average frag mass – 2g

- Frag drag coefficient – 0.5

- Density of air – 1.29 kg m-3

- Frag V @ 5m -1674 ms-1

- Frag V @ 15m – 1424 ms-1

- Frag V @ 230 m – 44 ms-1

RGD-5 grenade

- Charge mass – 110 g

- Mass of case – 200 g

- AUW – 310 g

- Explosive – TNT

- Average frag area – 1.0 cm2

- Average frag mass – 0.12 - 0.46g

- Frag drag coefficient – 0.5

- Density of air – 1.29 kg m-3

- Frag Vi = 1567 ms-1

- Number of fragments 350

81 mm Mortar AR-M81

- Charge Mass – 0.76 kg

- Mass of case – 1.334 kg

- AUW – 4.15 kg

- Explosive – TNT

- Frag drag coefficient – 1.6

- Density of air 1.23 kg m-3

- Frag Vi = 1734 ms-1

- Frag V @ 1m – 1700 ms-1

- Frag V @ 4m – 1600 ms-1

- Frag V @ 20m – 1150 ms-1

- Frag V @ 32m – 900 ms-1

- Frag V @ 77m – 357 ms-1

RPG PG-7VL

- Charge mass – 730 g

- Mass of case – 1600 g (including rocket motor)

- AUW – 1975 g

- Explosive – RDX/TNT - 95% HMX 5% wax

- Average frag mass – 0.1 g

- Density of air – 1.29 kg m-3

- Frag Vi -1831.5 ms-1

155 mm

- Charge mass – 1150 g

- Mass of case – 3105 g

- AUW – 4255 g

- Explosive – Comp B

- Average frag mass – 2.1g

- Density of air – 1.29 kg m-3

- Frag Vi -1551 ms-1

Pipe Bomb

- Charge mass – 264 g

- Mass of case – 417 g

- AUW – 681 g

- Explosive – PE 8

- Average frag mass – 0.83 g

- Frag drag coefficient – 1.6

- Density of air – 1.29 kg m-3

- Frag Vi = 2047 ms-1

- Frag V@ 1m = 2022 ms-1

- Frag V@ 20m = 1602 ms-1