- A spring is held vertically and has a 0.7 kg mass suspended from the lower end. If the spring constant is 500 Nm-1 what is the period of oscillation? Give your answer to 2 D.P.

Solution:

Angular frequency  ω =√(k/m)	= √(500/0.7) = 26.73  rad s-1

But ω = 2πf    therefore  f= ω/(2π)   = 4.25 Hz

Period T = 1/f  = 0.24 s

- A body is rotating at a speed of 7 revolutions per minute.  Determine to 2 D.P. the angular velocity in radians per sec. Give your answer to 2 D.P.

Solution:

7 revs per minute =  7 x (2π)  radians/minute

= 7 x (2π) /60  radians/sec

=0.73 rad s-1

- Find the resultant force for the system of forces shown in Figure 1. Give your answers to 2 D.P.

20 N

35°

15 N

25°

50°

6N

Resolving forces in horizontal direction:

FH      	= 20 cos(55) + 15cos(25) + 6 cos(40)

= 11.47  + 13.59  + 4.60

FH	= 29.66 N

Resolving forces in vertical direction:

FV     = 20 cos(35)  + 15 cos(65)  - 6 cos(50)

= 16.38 +  6.34 -  3.86

FV	= 18.86 N

Therefore resultant

FR     = √ ( 29.662 + 18.862 )

FR	=35.15 N

And   from  diagram , the angle FR   makes with positive  horizontal axis is θ then

Tanθ = 18.86/29.66

Therefore θ = tan-1(18.86/29.66)

Θ =32.45 degrees

- An ideal gas is at an initial thermodynamic state 1 given by:

P1  =  7 x 106 Pa

V1 =  0.4 m3

If the gas is compressed isothermally to a volume V2 =   0.002 m3   , what is the final pressure P2 ?

Solution:

Isothermal change of state therefore PV  is a constant

i.e.   P1V1 = P2V2

we know everything except V2.

Therefore V2 	= P1V1/P2

= 7 x 106  x 0.4  /0.002

=1.4 x 109 Pa

- A body of mass 500 g is dropped from a vertical height of 5.2 m. Calculate the speed at impact on ground.. Express your answer to two decimal places.

Potential Energy (mgh) is converted entirely into  kinetic energy (0.5mv2) at ground impact

mgh = 0.5 mv2

v2  = gh/0.5  = 9.8 x 5.2/0.5  = 101.92

therefore v = 10.10   ms-1 to 2 DP

- A projectile of mass 30 g penetrates a target to a depth of 0.3 m. If the speed at initial impact is 700 ms-1 determine the resistive force that the body exerts on the projectile, assuming uniform resistance

Solution:

Kinetic energy of projectile is  converted into  work done by resistive medium

0.5 mv2   =   F x distance of penetration

Where F is resistive force of medium

Therefore  F = 0.5 x 0.03 x 7002   / 0.3

F = 24500 N

- An applied force stretches a uniform wire. If the change in length is 7x10-3 m and the strain was 0.05, calculate the original length of the wire.

Solution:

Strain =  extension/original length

0.05 = 7x10-3 / L

L= 7x10-3  /0.05

L= 0.14 m

- A vertical uniform wire has Modulus of Elasticity 220 x 109 pascals, cross-sectional area 0.5 cm2 and natural length 3.2 m. Calculate the stress and strain values when a mass of 120 kg is suspended from one end. Express your answer to two decimal places.	 Assume acceleration due to gravity g = 9.8 ms-2.

Solution:

Convert area into  S.I. units      0.5 cm2   =0.5 x 10-4 m2

Stress = Force/Area   = mg/area =  120  x 9.8 / (0.5 x 10-4)

Stress = 23.52 MPa  or 23.52 x 106 Pa

To calculate Strain we use Hooke’s Law:

E =  Stress/Strain

Therefore  Strain=  Stress /E  =    23.52 x 106 /220x109

Strain = 1.07 x 10-4

- How much energy is required to raise the temperature of a body with specific heat 400 J kg-1 K-1 and mass 0.7  kg through a temperature increase of 20 K? Express your answer to two decimal places.

From definition of specific heat

It takes 400  Joules of heat energy to  raise 1 Kg through 1 degree

Therefore it takes 400  x 0.7  x 20   Joules to  raise 0.7 kg through 20  degrees

i.e.  Q =  m C  ΔT

= 5600.00  J

- A laser pulse of total energy 7 J is applied for 0.05 ms. If the pulse has a rectangular temporal profile, what is the power?

For a rectangular power profile Energy is  under curve

Energy =  P  x t  therefore P   =  Energy/t   = 7/0.05  = 140  Js-1

- Calculate the Moment of Inertia of a rectangular block of length 5m  and total mass 2kg if its pivot point is at one end.    r2

I  =

Where r is the distance form the pivot point  and dm is an elemental mass

Work out elemental mass  dm over an elemental length dr from the pivot point.

- 720 J of heat energy is supplied to 260 g of gas that has a specific heat of

290 J kg-1 K-1 . If the gas does work of 110 J in expanding, what is the change in temperature of the gas?

From First law of Thermodynamics:

dQ   =            dU                 +                  dW

Remember that the change in Internal Energy of the system is  m C ΔT  where m is mass, C specific heat and ΔT is increase in temperature.

Therefore   dQ = m C ΔT    +    dW

Substituting in values:

720  = 0.26*290* ΔT  + 110

610 = 0.26*290* ΔT

Therefore  ΔT  = 610/(0.26*290)

ΔT = 8.09 K

- A body of surface area 4.5 m2 with surface emissivity 0.8 is at a uniform temperature of 700 °C. Calculate the total radiant power emitted by this body

From Stefan’s Law:

q= εσT4      W/m2      where q is the radiant energy  emitted per second per unit area of the emitter that has an emissivity of ε.   Stefan- Boltzmann constant is    σ.

Substituting in values and remembering that T is in Kelvin  we get

q  =  0.8  * 5.67× 10−8   9734    watts  per m2

= 40,655.98  watts per m2

Therefore by simple proportions in  4.5 m2 we must have

40,655.98  * 4.5  =  182,951.90  Watts

- A metal bar of length 70 cm is at a uniform temperature of 45 °C . If the Coefficient of Linear Expansion is 6 x 10-4 K-1 , what is the length of the bar in S.I. units at 110 °C .

L(T)=L(To)[1+α(T-To)]   where α is linear coefficient of thermal expansion

Where L(T) is length at temperature T  and L(T0)  is Length at temperature T0

Substituting in values:

L(110)  = L(45)  [ 1 + α(65) ]

= 0.70  [1 +  6 x 10-4   * 65]

= 0.70[1.039]

= 0.73 m