| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2020 | IONAWR 2020 |

MODULE CODE: M5X02068 / M5X00406

Thermofluid Mechanics 1

SOLUTIONS

- Question One: Heat Transfer Conduction

A furnace wall consists of 200 mm layer of refractory bricks, 6 mm layer of steel plate, and a 100 mm layer of insulation bricks. The maximum temperature of the wall is 1150C on the furnace side and the minimum temperature is 40C on the outermost side of the wall. An accurate energy balance over the furnace shows that the heat loss from the wall is 400 W/m2. It is known that there is a layer of air between the layers of refractory bricks and the steel plate. Thermal conductivities for the three layers are 1.52, 45, and 0.138 W/m.C respectively. Determine,

- To how many millimetres of insulation bricks is the air layer equivalent.

- The temperature of the outer surface of the steel plate.

Steel plate

ks = 45 W/m.C

Schematic:

40C

Q = 400 W/m2

1150C

Insulation bricks

ki = 0.138 W/m.C

100

mm

xa

200 mm

6

mm

Air gap equivalent to xa mm of insulation bricks

Refractory bricks

krb = 1.52 W/m.C

Assumptions:

- Steady-state conditions.

- Property of layers to be considered:

krb = 1.52 W/m.C (Refractory bricks)

ka = ki  (Air gap)

ks= 45 W/m.C (Steel plate)

ki = 0.138 W/m.C (Insulation bricks)

SOLUTION:

- Heat transfer through the composite wall

Q=U∙A∙∆T

∴   400=U∙1∙1150-40

Where overall heat transfer coefficient U is,

1U=1Urb+1Ua+1Us+1Ui

∴  1U=∆xrbkrb+∆xaki+∆xsks+∆xiki

∴  1U=0.21.52+∆xa0.138+0.00645+0.10.138=0.8563499619+∆xa0.138

(5 marks)

Thus,

1U=1150-40400    ∴  U=4001110=0.360 Wm2℃

∴  0.21.52+∆xa0.138+0.00645+0.10.138=1110400

∴  ∆xa0.138=1110400-0.21.52+0.00645+0.10.138=1.91865

∴  ∆xa=1110400-0.21.52+0.00645+0.10.138×0.138

∴  ∆xa=264.77 mm

(10 marks)

- Temperature of the outer surface of the steel plate Ts is,

Q=U∙A∙∆T

∴   400=ki∆xi∙1∙Ts-40

∴   Ts=400×∆xiki+40=400×0.10.138+40=329.86℃

(10 marks)

- Question Two: Heat Transfer Convection + Radiation (25 marks)

The power of an electronic component is dissipated through a rectangular horizontal heat sink by convection and radiation into surroundings at 25C. The total rate of heat transfer through the heat sink is equal to the power dissipated by the component which is equivalent to 1.35 W. The surface temperature of the heat sink is measured to be 55C. The component has a width W = 2 cm and length L = 5 cm. The length L is to be considered as the significant length. An incorporated fan is generating an air flow velocity of 5 m/s. Determine the surface emissivity of the heat sink.

Schematic:

QTotal = QConvection + QRadiation

Where QTotal = Power dissipated by component through heat sink = 1.35 W

Heat sink temperature

Ts = 55C

Air temperature

Tf =  25C

uf = 5m/s

W = 2cm

L = 5cm

Assumptions:

- Steady-state conditions.

- Constant properties.

- Heat sink thickness is negligible.

- Properties of air at 40C:  = 1.128 kg/m3,  = 0.019×103 kg/m.s, k = 0.0272 W/mK, cp = 1.01 kJ/kg.K

SOLUTION:

Qconvection=h∙A∙Ts-Tf

Qradiation=56.7×103∙ε∙A∙T210004-T110004

Qtotal=Qconvection+Qradiation

Therefore,

Qtotal=h∙A∙Ts-Tf+56.7×103∙ε∙A∙T210004-T110004

∴1.35=h∙0.05×0.02∙55-25+56.7×103∙ε∙0.05×0.02∙55+27310004-25+27310004

(4 marks)

∴    1.35=h×0.001×30+56.7×103×ε×0.001∙32810004-29810004

- Determination of h:

Re=u∙l∙ρμ=5×0.05×1.1280.019×10-3

∴   Re=14842.11

(5 marks)

Re < 50×103 therefore the region is laminar and,

Nu=0.664∙Re0.5∙Pr0.3

With,

Pr=μ∙cpk=0.019×10-3×1.01×1030.0272=0.71

(3 marks)

Thus,

Nu=0.664×14842.110.5×0.710.3=72.86

(5 marks)

Hence,

Nu=h∙lk

∴   h=Nu∙kl=72.86×0.02720.05=39.63 Wm2K

(3 marks)

Therefore,

1.35=39.63×0.001×30+56.7×103×ε×0.001∙32810004-29810004

∴  1.35=1.189+56.7×103×ε×0.001∙32810004-29810004

∴       ε=1.35-39.63×0.001×3056.7×103×0.001∙32810004-29810004=0.77

(5 marks)

- Question Three: Fluid Flow

Water flows down a sloping pipe, which has one end 2.3 m above the other. The pipe section tapers from 95 cm diameter at the top end A1 to 400 mm diameter at the lower end A2. The flow of water is 8.5 tonnes/min. If the frictional losses are equivalent to a head of 120 cm of water, determine the difference in pressure head between A1 and A2.

Schematic:

D1 = 0.95 m

m=8.5 tonnes/min

hf=1.2 m

Water

2.3 m

D2 = 0.4 m

SOLUTION:

Applying Bernoulli’s equation:

P1ρ.g+v122.g+h1=P2ρ.g+v222.g+h2+hf

∴     P2ρ.g-P1ρ.g=v122.g+h1-v222.g+h2+hf

∴     P2ρ.g-P1ρ.g=v122×9.81+2.3-v222×9.81+0+1.2

or     P1-P2ρ.g=v222×9.81+0+1.2-v122×9.81+2.3

- Applying continuity equation:

Q= A1∙v1 = A2∙v2

Where volumetric flow rate Q is,

m=8.5×10360 = 141.6kgs

∴   Q=141.6 ls  because 1 litre weigh 1 kg for water

∴   Q=0.1416 m3s

(5 marks)

Thus,

Q= A1∙v1 = A2∙v2

∴   v1=QA1=0.1416π×0.9524=0.2 m/s

(5 marks)

∴   v2=QA2=0.1416π×0.424=1.13 m/s

(5 marks)

Therefore,

∴     P2ρ.g-P1ρ.g=0.222×9.81+2.3-1.1322×9.81+1.2

∴     P2ρ.g-P1ρ.g=1.04 m

(10 marks)

- Question Four: Thermodynamics, Open system

An indirect contact (no mixing of fluids) chilled-water heat-exchange unit is designed to cool 5 m3/s of air at 100 kPa and 30C to 100kPa and 18C by using water at 5C. The heat exchanger is to be considered well insulated. Determine the maximum water outlet temperature when the mass flow rate of the water is 1.653 kg/s.

Schematic:

mwater=1.653kgs

T4 = ?

Sat. liquid water

T3 = 5C

Sat. liquid water

4

3

CHILLED WATER

HEAT EXCHANGER

Air

P2 = 100 kPa

T2 = 18C

Air

P1 = 100 kPa

T1 = 30C

V1=5m3s

2

1

Assumptions:

- Steady-flow process.

- Kinetic and potential energy changes are negligible.

- There are no work interactions.

- Heat loss from the device to the surroundings is negligible and thus heat transfer from the hot fluid is equal to the heat transfer to the cold fluid.

- The properties of saturated water can be found in Tables A-4.

- Air is an ideal gas, therefore equations P.v = R.T and ∆h=cp.∆T can be used.

- The properties of ideal gases can be found in Tables A-2.

SOLUTION:

| System | Mass balance | Energy balance |
| --- | --- | --- |
| Entire heat exchanger | m1=m2=ma and m3=m4=mw=1.653kgs | Qhe=m2.h2+m4.h4-m1.h1-m3.h3 ∴Qhe=mah2-h1+mwh4-h3 |
| Water pipe | m3=m4=mw=1.653kgs | Qw→a=m4.h4-m3.h3 ∴ Qw→a=mwh4-h3 |
| Air pipe | m1=m2=ms | Qa→w=m2.h2-m1.h1 ∴ Qa→w=mah2-h1 |

Using the mass and energy balance from the entire heat exchanger:

0=mah2-h1+mwh4-h3

∴ mah1-h2=mwh4-h3

∴ ma∆ha=mwh4-h3

(5 marks)

Now we need to determine the enthalpies and mass flow rate of air,

T3=5°Csat. liquid   h3=21.02 kJ/kg 	(Table A-4)

(4 marks)

∆ha=cp∙∆T=1.005×30-18	(Table A-2)

∴  ∆ha=12.06 kJ/kg

(4 marks)

V1=ma∙v1=5m3s

According to ideal gas equation,

P.v = R.T   ∴    v=R∙TP

∴    v1=0.287×30+273100=0.86961 m3kg

(2 marks)

Thus,

ma=V1v1=50.86961=5.75 kg/s

(2 marks)

Transposing and substituting in energy balance, we find:

ma∆ha=mwh4-h3

∴      ma∆ha=mwh4-mwh3

∴      h4=ma∆ha+mwh3mw

∴      h4=5.75×12.06+1.653×21.021.653=62.97 kJ/kg

(4 marks)

According to Table A-4:

h4=62.97 kJ/kgsat. liquid   T4≈15℃

(4 marks)

Diwedd/End