EXAMINATION SOLUTIONS

(to accompany the examination paper sent to the external examiner

but NOT to be seen by students)

| Programme: | BSc/HND Motorsport Engineering BSc/HND Automotive Engineering | Level: | 5 |
| --- | --- | --- | --- |
| Module: | Industrial Placement and Team Management |  |  |
| Author: | Mark Sandford | Page: |  |

| Q 1 | Question | Marks |
| --- | --- | --- |
| 1a) 1b) 1c) 1d) 1e) | Candidates are required to calculate the break-even point. Break-even point = Fixed costsSelling price per unit-Variable costs per unit Break-even point = £500,000£20- £7 Break-even point = 38,462 units Candidates are required to draw a break-even chart that shows the firms current position Sales revenue Profit Variable costs Costs £ Break-even Loss 500,000 Fixed costs 38,462 Output (units) (3 marks) for labelling lines (1 mark) for adding figures (fixed costs and output at break-even point) (1 mark) for labelling axis Candidates are required to calculate the margin of safety at 45,000 units. Margin of safety = Current output – Output at break-even point Margin of safety = 45,000 – 38,462 Margin of safety = 6,538 units Candidates are required to calculate the profit if output falls to 75% Output at 75% = 60,000 x 0.75 = 45,000 units Now calculate the contribution and profit using the following formula Contribution = Sales revenue – Variable costs Profit = Contribution – Fixed costs Contribution = 45,000 x £20 = 45,000 x £7 Contribution = £900,000 - £315,000 Contribution = £585,000 Profit = Contribution – Fixed costs Profit = £585,000 - £500,000 Profit = £85,000 Candidates are required to discuss the limitations of using break-even analysis - It depends on the quality and accuracy of the data used to construct cost and revenue functions. If the data is poor and inaccurate, the conclusions drawn on the basis of the data are flawed. - It assumes that all output is sold and no stocks are held. Many businesses hold stocks of finished goods to cope with changes in demand - The break-even chart is drawn for a given set of conditions. It cannot cope with a sudden increase in wages and prices or changes in technology - Throughout the analysis it has been assumed that the total cost and total revenue functions are linear. This may not always be the case. | 4 marks 5 marks 4 marks 2 marks 3 marks 2 marks 5 marks |

| Q2 | Question | Marks |
| --- | --- | --- |
| 2 a | - Generate an RFQ for the autoclave. Briefly explain the reasoning behind the inclusion of each of your criteria. Physical internal size – to ensure the biggest panel can fit or batches of panels can be accommodated External dimensions and weight for shipping Ex works and delivery charge Warranty terms and conditions Operational software Software maintenance and support facilities Power consumption/efficiency Pressure & heat up / depressurize times Additional service equipment supplied or required Service contract conditions. Sketty sports cars are concerned about the initial purchase price and operating costs for the autoclave. | 8 marks |
| 2 b | - Explain the concept of life style costing. (2 marks) The student should define/discuss life cycle costing - The practice of obtaining over their lifetime the best use of the physical assets at the lowest cost to the organisation. This is achieved through a combination of management, financial, engineering and other disciplines. - Discuss the advantages Accurate comparison of competing options (broad options buy v lease days) Good awareness of total costs (ref total cost of project) Accurate forecasting of cost patterns (cash flow esp, running costs) Allows assessment of performance trade off against cost - Outline the main disadvantage of not really knowing what will happen in the future. | 2marks |
| 2 c | - Critically evaluate the factors that should be considered to generate a life style cost for the Autoclave. (10 marks) The students should analyse important factors, which could include: - LCC gives two main costs - Acquisition costs - Initial cost - Transports costs (i.e. ex works) - Installation / commissioning costs, - esp building changes, floor strengthening etc - provision of electrical power - Cost of necessary peripherials - Cooling towers - Special freezers for material storage etc - Spares packages - Training costs - The time cost of money - Running costs - Operation, fill time, cycletime. - Energy costs - Material/consumable costs - Insurance – pressure vessel - Servicing - Raw material availability. - Maintenance and inspection costs, - Disposal Costs or Residual Value - Lifetime prediction, deprecation. - Disposal – residual value (depreciation factors) or costs of removal and scrapping. | 10 marks |

| Q3 | Question | Marks |
| --- | --- | --- |
| 3 a | Explain the concept of Total Quality Management The students should discuss: Fitness for Purpose or use, (Dr. Joseph Juran) Or Meeting (exceeding)the Customer’s Requirements Requirements include:- - Reliability - Cost - Function - Delivery - Life - Aesthetics Customers are both internal and external to an organistion Total Quality Embraces - BS/ISO – Quality accreditation - Company Culture - Individual Attitudes - Continuous Improvement - Commitment – ownership of problems - Relationships (with suppliers and customers) And applies to all parts of the organisation “Quality is both thinking why something is done, and why it is done that way: Then thinking differently to improve” Mr Ohno, Chairman of Toyota. | 5 marks |
| 3 b | Outline a total Quality Management Strategy. Refer to Juran’s 10 steps. The student should discuss: - TQM Must be owned by senior management - TQM requires continuous improvement - TQM involves everybody - TQM applicable to all parts of the company design, production, purchase, finance - TQM means satisfying your customers Juran’s 10 steps - Build awareness of the need and opportunity for improvement - Set goals for improvement - Organise to meet goals - Promote training - Carry out projects to solve problems - Report progress - Give recognition - Communicate results - Keep score - Maintain momentum. Make annual improvement part of regular systems of company continuous improvement | 6 marks |
| 3 c | What are the potential effects of not having a robust engineering quality system? Cost arises from - Error - Eg. Scrap, rework, waste in any form - Policing - Loss of reputation - Loss of market share Costs can be as high as 40% of sales turnover, and are typically as high as 15-25% of sales. 80-85% of quality problems originate in areas other than manufacture. Examples of waste: - Service/warranty costs FRACAS reporting/investgations - Customer dissatisfaction – loss of reputation, leading to loss of market share. - Redesign - Shortages - Incorrect deliveries - Material handling damage - Lack of information. Unclear communication - Rework - Excess inventory - Time wasting | 3 marks |
| 3 d | Many engineering organizations utilize Fault Reporting And Corrective Action Systems (FRACAS). Imagine that you are working for a volume motorcycle engineering manufacturer. It has been reported that some telescope damper fork seals are leaking regularly on a recently released motorcycle. (see below) You have been asked to identlfy any possible causes of this failiure. Create an Ishikawa (herring bone) diagram of possible causes. Explain significant design error potential causes that may require more detail investigation. Use diagrams as required. Check design intent, usage and life. Velocity and fluid pressures The back bone (or trunk) is the error state eg leaking seal, branches and twigs off that. Design branch Limits and fits (identify critical dimensions) esp seal housing Surface finish esp shaft. Dimensional tolerances etc Material selection, fluid compatibility, shore values Lifing, recommended service intervals etc. Environmental branch Ingress IP rating, fluid and grit etc Temp, humidity Vibration Manufacturing branch Seal insertion, process, mandrels and tooling, lubrication System assy procedure, Training. Material management branch. Purchase systems, identification of critical items. Supplier quality assessment. Strorage, time . temp humidity, stock control etc. | 11 marks |

| Q4 | Question | Marks |
| --- | --- | --- |
| 4 a | The candidate should discuss the potential adoption these technologies: The governing body – automotive relevance, interest from Vehicles Manufacturers, greenification of motorsport. The business - advantages to teams, publicity, costs, potential sponsorship The technology – size, relevance, safety and complexity. | 4 marks |
| 4 b | The candidate should perform a PEST analysis on the marco environment. The list bellows is not exhaustive but provides examples. Political Supporting the political agenda Seen to be green Supporting government agendas Green pressure groups Emissions legislation CO2, noxious gasses Economic Expensive to design and develop Extra spares required Extra safety regulations Potential new sponsorship avenues. Additional staff & facilities required. Social Consumer buying patterns Satisfying consumers green agenda Fashion for performance upgrades Fashion for vehicle downsizing Increased use of KERS / hybridisation again for marketing and sales Technological Effect brake stability Effect on vehicle mass Effect on servicing requirements and safety Ease of installation OEM’s offering more base vehicles with hybrid capability – technology transfer OEM’s providing more ‘niche’ vehicles Competing technologies such as flywheels batteries, supercaps. Improving vehicle stage performance (especially for restricted engines) Possibilities to ‘down size’ IC engine. Ever more complex donor vehicle engine management systems making KERS/hybrid integration more complex | 16marks |
| 4 c | An open question for the candidate, suggestions could include: Improve the green credentials Target for smaller engines, promote concept of downsizing More potential sponsorship Probably need market survey, sound out VM’s etc Technical and cost impact on teams – study required Safety considerations – study required. Diversify within technology and equipment What other markets could be addressed with similar technology? What spin offs into road vehicle technology? Consider maintaining entertainment spectacle. | 5 marks |