University of Wales Trinity Saint David

School of Engineering

2D Transient Heat Equation Solved with the Explicit Finite Difference Method

Computational Methods

(ACMA6012)

Tia Ball

2220258

January 2026

# Abstract

This report models two-dimensional transient heat conduction in a 10 m × 10 m aluminium plate using an Explicit Finite Difference Method (EFDM) with a forward-time central-space (FTCS) scheme on a uniform grid. Dirichlet boundary conditions are applied with the north edge at 60 °C, the remaining boundaries at 0 °C, and an initial interior temperature of 0 °C. Numerical results are validated against an analytical steady-state solution obtained via separation of variables and evaluated as a truncated Fourier sine series. Agreement is quantified using the maximum absolute error and RMSE. Sensitivity studies using a 12 m × 12 m aluminium plate assess the influence of grid spacing, time step, and convergence tolerance under the Fourier-number stability constraint. Results show that the stopping criterion strongly controls the reported “equilibrium” field and can dominate trends with Δt, producing higher validation errors for smaller time steps unless the tolerance is tightened. A parameter set is identified that balances computational cost and agreement with the analytical solution.

Contents

Abstract	2

List of Figures	6

1	Introduction	8

1.1	Context and Background	8

1.2	Aim and Objectives	8

1.2.1	Aim	8

1.2.2	Objectives	8

1.3	Methodology Overview	9

1.4	Report Structure	9

2	Governing Equation	10

2.1	Background and Importance of the Heat Equation	10

2.2	2D Heat Equation with no Heat Source Term	10

2.3	Definition of Terms and Material Properties	10

3	Explicit finite difference form of the governing equation	12

3.1	Taylor Series and Finite Difference Approximations	12

3.2	Discretisation of the Spatial and Temporal Domains	12

3.3	Formulation of the EFDM (FTCS scheme):	13

3.4	Fourier Number and Stability Criterion:	13

4	The Initial, Boundary Conditions and Analytical Solution.	14

4.1	Initial Condition	14

4.2	Boundary Conditions and Justification	14

4.3	Analytical Solution at Thermal Equilibrium	14

5	Codes’ Details and Explanations	16

5.1	Workspace Setup	16

5.2	Material Properties and Thermal Diffusivity	16

5.3	User Input and Validation for X, Y and h	16

5.4	Grid Sizing	17

5.5	Initialisation of Temperature Matrix T1	17

5.6	Apply Boundary Conditions	17

5.7	Time Step Selection and Stability Check	17

5.8	Initialise T2, Convergence Variables, and Plotting Grid	18

5.9	Live Temperature Contour Plot During Iteration	18

5.10	EFDM Update Loop and Convergence Check	18

5.11	Display Convergence Results	19

5.12	Analytical Solution Setup	19

5.13	Analytical Steady-State Solution (Series Summation)	19

5.14	Plot the Analytical Temperature Field	19

5.15	Error Map and Validation Metrics	20

5.16	Time to Thermal Equilibrium Calculation	20

6	Validation of the Code.	21

6.1	Validation Approach	21

6.2	Validation Case Selected for Presentation (CC = 0.001)	21

6.3	Comparison Validation Case (CC = 0.1)	23

6.4	Validation Conclusion	24

7	Sensitivity Analysis of the Code.	25

7.1	Purpose and Method	25

7.2	Influence of grid spacing h(fixed Δt=150s, fixed CC=0.001)	25

7.3	Influence of time step Δt(fixed h=0.75m, fixed CC=0.0001)	26

7.4	Influence of thermal equilibrium tolerance CC(fixed h=1m, fixed Δt=500s)	27

7.5	Sensitivity Summary	28

8	Limitations of the Numerical Model.	29

8.1	Limitations of the Numerical Method (EFDM)	29

8.2	Limitations of the Coding Implementation (Evidence Linked)	29

8.3	Limitations of the Analytical Reference Solution	30

8.4	Summary of Limitations	30

9	Suggested Improvements and Future Developments.	31

9.1	Improved Equilibrium (Steady-State) Detection	31

9.2	More Efficient Time Integration	31

9.3	Grid Convergence and Uncertainty Reporting	31

9.4	Alternatives for Complex Geometries and Unstructured Meshes	32

10	Conclusion.	33

11	References	34

12	Appendix	36

12.1	MATLAB Code	36

# List of Figures

Listing 1 - Script Initialisation	16

Listing 2 - Aluminium Material Properties	16

Listing 3 - User Input Prompts & Validation	16

Listing 4 - Grid Size Calculation and Node Count Definition (NR, NC)	17

Listing 5 - Initial Temperature Field Initialisation (T1)	17

Listing 6 - Application of Dirichlet Boundary Condition on the North Edge (T0)	17

Listing 7 - Stable Time-Step Selection Enforcing Fo ≤ 1/4	17

Listing 8 - Initialise T2, Convergence Variables, and Plotting Grid	18

Listing 9 - Temperature Contour Plot	18

Listing 10 - EFDM Iteration Loop	18

Listing 11 - Convergence Check and Iteration Counter Update	19

Listing 12 - Analytical Solution Setup	19

Listing 13 - Analytical Steady-State Series Solution Assembly	19

Listing 14 - Analytical Solution Contour Plot Generation for TA	19

Listing 15 - Validation Error Field Calculation and Absolute Error Contour Plot	20

Listing 16 - Error Metrics (max error, RMSE) and Equilibrium Time Reporting	20

Table 1 - Thermal Equilibrium Time Results for the Grid Spacing Sensitivity Study on a 12 m × 12 m Plate with Δt=150s and CC=0.001.	25

Table 2 - Thermal Equilibrium Time Results for the Time Step Sensitivity Study on a 12 m × 12 m Plate with h=0.75m and CC=0.0001	26

Table 3 - Thermal Equilibrium Time Results for the Tolerance Sensitivity Study on a 12 m × 12 m Plate with h=1m and Δt=500s	27

Figure 1 - EFDM Steady-State Temperature Field	21

Figure 2 - Analytical Steady-State Temperature Field	22

Figure 3 - Absolute Error Field	22

Figure 4 - EFDM Steady-State Temperature Field	23

Figure 5 - Analytical Steady-State Temperature Field	24

Figure 6 - Thermal Equilibrium Time as a Function of Grid Spacing h for a 12 m × 12 m Plate with Δt=150s and CC=0.001	26

Figure 7 - Thermal Equilibrium Time as a Function of Time Step Δtfor a 12 m × 12 m Plate with h=0.75m and CC=0.0001	27

Figure 8 - Thermal Equilibrium Time as a Function of Equilibrium Tolerance CC for a 12 m × 12 m Plate with h=1m and Δt=500s	28

# Introduction

## Context and Background

Transient heat conduction in solids is a fundamental problem in engineering, underpinning thermal management in structures, manufacturing processes, and electronic systems. In many practical applications, temperature evolves over time due to imposed boundary temperatures, and spatial temperature gradients can drive thermal stresses or affect performance. While analytical solutions exist for a limited class of geometries and boundary conditions, many engineering problems require numerical methods to predict transient and steady temperature fields on spatial domains. Explicit finite difference methods are widely taught and used because they are straightforward to implement on structured grids, though they require adherence to stability constraints and careful interpretation of numerical convergence. (Morton & Mayers, 2005)

## Aim and Objectives

### Aim

To model two-dimensional transient heat conduction in an aluminium plate using the Explicit Finite Difference Method (EFDM) and to validate the resulting equilibrium temperature field against an analytical steady-state solution.

### Objectives

- To formulate the governing 2D transient heat equation for conduction in a homogeneous plate and define thermal diffusivity from material properties (Incropera, DeWitt, Bergman, & Lavine, 2011).

- To derive and implement the EFDM (forward-time, central-space) discretisation for the 2D heat equation and identify the order of accuracy and truncation error sources (Smith, 1985).

- To apply the explicit stability condition using the Fourier number and ensure stable time-marching for all simulation runs (Morton & Mayers, 2005).

- To compute an analytical steady-state reference solution using a truncated separation-of-variables Fourier series appropriate to the prescribed boundary conditions (Carslaw & Jaeger, 1959).

- To validate the EFDM equilibrium field against the analytical solution using contour plots, the maximum absolute error and the root mean square error (RMSE) (Press, Teukolsky, Vetterling, & Flannery, 2007).

- To perform sensitivity analysis on grid spacing h, time step Δt, and convergence criterion CC, and to interpret trade-offs between accuracy and computational cost. (LeVeque, 2007).

## Methodology Overview

The plate domain is discretised using a uniform Cartesian grid and advanced in time using an explicit FTCS finite difference scheme. A user-specified time step is restricted by the explicit stability requirement Fo≤1/4, where Fo=αΔt/h2. The simulation advances until a numerical convergence criterion based on the maximum change between successive temperature fields falls below a specified tolerance. The final numerical field is then compared to an analytical steady-state solution computed using a truncated Fourier series, and quantitative error metrics are reported. (Morton & Mayers, 2005).

## Report Structure

Chapter 2 introduces the governing equation and defines all parameters and units. Chapter 3 presents the EFDM formulation, including the Fourier number and stability considerations. Chapter 4 specifies the initial and boundary conditions and outlines the analytical steady-state reference solution. Chapter 5 summarises the MATLAB implementation. Chapter 6 provides validation against the analytical solution using contour plots and error metrics. Chapter 7 reports sensitivity analysis for grid spacing, time step and convergence tolerance. Chapters 8 and 9 discuss limitations and propose improvements, respectively, and Chapter 10 presents the conclusions.

# Governing Equation

## Background and Importance of the Heat Equation

The heat equation is a fundamental partial differential equation (PDE) describing how temperature evolves over time due to heat conduction within a material. It is widely used in engineering to model heating and cooling processes, support thermal management, and predict temperature gradients that may lead to thermal stresses or performance changes. (Incropera, DeWitt, Bergman, & Lavine, 2011). The model follows directly from energy conservation combined with Fourier’s law of conduction, making it central to heat transfer and diffusion analysis (Bergman, Lavine, Incorpera, & DeWitt, 2011).

## 2D Heat Equation with no Heat Source Term

The general transient heat conduction equation in two dimensions with a volumetric heat generation term q(W/m3) is:

ρcp∂T∂t=k∂2T∂x2∂2T∂y2+q

For the case considered in this report, there is no internal heat source, so q=0. Dividing by ρcp gives the standard 2D heat equation in diffusion form:

∂T∂t=α∂2T∂x2∂2T∂y2

(Incropera, DeWitt, Bergman, & Lavine, 2011).

## Definition of Terms and Material Properties

In the governing equation:

- T(x,y,t) is temperature (°C).

- t is time (s).

- x and y are spatial coordinates (m).

- ∂T∂t is the temporal rate of change of temperature (°C/s).

- ∂2T∂x2 and  ∂2T∂y2 are the spatial second derivatives representing curvature of the temperature field (°C/m2).

- α is thermal diffusivity (m2/s), defined as:

α=kρcp

where:

- k is thermal conductivity (W/m °C).

- ρ is density (kg/m3).

- cp is specific heat capacity (J/kg °C).

Thermal diffusivity characterises how rapidly temperature disturbances diffuse through a material, combining conductivity with thermal storage capacity (Bergman, Lavine, Incorpera, & DeWitt, 2011).

For aluminium, the MATLAB implementation used k=143W/m °C, ρ=2800kg/m3, and cp=795J/kg °C, giving α=6.42×10-5m2/s, consistent with the computed output from the code (Incropera, DeWitt, Bergman, & Lavine, 2011).

# Explicit finite difference form of the governing equation

## Taylor Series and Finite Difference Approximations

Finite difference methods approximate derivatives by expanding T about a point using a Taylor series (Richtmyer & Morton, 1967). For a sufficiently smooth function, expansions about xi give:

T(xi+1)=T(xi)+hT'(xi)+h22T''(xi)+O(h3)
T(xi-1)=T(xi)-hT'(xi)+h22T''(xi)+O(h3)

Adding these expansions eliminates odd derivative terms and yields the standard second-derivative central difference approximation:

T''(xi)≈Ti+1-2Ti+Ti-1h2

with truncation error O(h2), meaning the spatial approximation is second-order accurate (Smith, 1985). Similarly, the forward difference approximation for the time derivative:

∂T∂t≈Tn+1-TnΔt

has a truncation error O(Δt), making the temporal approximation first-order accurate (Morton & Mayers, 2005). The resulting FTCS EFDM is therefore first-order accurate in time and second-order accurate in space, giving a combined local truncation error of O(Δt)+O(h2) (Smith, 1985).

## Discretisation of the Spatial and Temporal Domains

The rectangular domain 0≤x≤X, 0≤y≤Y is divided into a uniform mesh with spacing h in both directions, so h=Δx=Δy. Grid nodes are indexed ij, where idenotes the y-direction (row) and jdenotes the x-direction (column). Time is discretised into levels tn=nΔt, where n is the time-step index (Smith, 1985).

## Formulation of the EFDM (FTCS scheme):

Applying forward difference in time and central differences in space to the 2D heat equation gives:

Ti,jn+1-Ti,jnΔt=αTi+1,jn-2Ti,jn+Ti-1,jnh2Ti,j+1n-2Ti,jn+Ti,j-1nh2

Rearranging yields the explicit update:

Ti,jn+1=Ti,jn+FoTi+1,jn+Ti-1,jn+Ti,j+1n+Ti,j-1n-4Ti,jn

where the Fourier number Fo is defined below.

## Fourier Number and Stability Criterion:

The Fourier number is introduced as:

Fo=αΔth2

This nondimensional parameter controls the proportion of diffusion applied per time step relative to the mesh spacing. A larger Fo generally accelerates the approach to equilibrium but can destabilise explicit schemes if it exceeds the stability limit (Morton & Mayers, 2005).

For the 2D FTCS diffusion scheme on a uniform grid, von Neumann stability analysis shows that stability requires:

Fo≤14

This criterion constrains the maximum permissible time step for a chosen h, linking spatial and temporal discretisation choices directly (Smith, 1985). In the MATLAB code, Δtmax=h2/(4α) is computed and user input is restricted to satisfy this requirement.

# The Initial, Boundary Conditions and Analytical Solution.

## Initial Condition

The plate is initialised at a uniform temperature of 0 °C throughout the domain, such that:

T(x,y,0)=0

This represents a uniform starting condition before heat diffuses into the plate from the imposed boundary temperatures.

## Boundary Conditions and Justification

Dirichlet boundary conditions are applied as follows:

- North boundary: T(x,Y,t)=T0=60°C

- South boundary: T(x,0,t)=0°C

- West boundary: T(0,y,t)=0°C

- East boundary: T(X,y,t)=0°C

In the MATLAB implementation, the north boundary is applied to the top row, excluding corner nodes, i.e. T1(1,2:NC-1)=T0, while the remaining boundary nodes retain the initialised value of 0 °C.

These boundary conditions were chosen because they match the analytical steady-state solution provided in the Coursework 1 brief. Since closed-form analytical solutions are only available for limited boundary-condition configurations, this choice enables direct validation by comparing the EFDM equilibrium field with the analytical reference solution.

## Analytical Solution at Thermal Equilibrium

At thermal equilibrium, ∂T/∂t=0, and the governing equation reduces to Laplace’s equation:

∂2T∂x2+∂2T∂y2=0

For the specified Dirichlet boundary conditions on a rectangular domain, separation of variables yields a Fourier sine-series solution for the steady-state temperature field TAx,y (Carslaw & Jaeger, 1959). In the MATLAB code, this analytical series is evaluated numerically using a finite truncation (10,000 terms) to produce the reference field used for validation.

# Codes’ Details and Explanations

## Workspace Setup

Listing 1 - Script Initialisation

Listing 1 clears the workspace and closes figures to ensure a clean run. This prevents old variables/plots from affecting results.

## Material Properties and Thermal Diffusivity

Listing 2 - Aluminium Material Properties

Listing 2 defines aluminium properties and calculates thermal diffusivity. This is needed for Fo, stability, and time marching.

## User Input and Validation for X, Y and h

Listing 3 - User Input Prompts & Validation

Listing 3 prompts the user for plate dimensions and grid spacing, then validates inputs. This ensures inputs are positive and the grid aligns with boundaries.

Listing 4 - Grid Size Calculation and Node Count Definition (NR, NC)

Grid Sizing

Listing 4 converts physical dimensions into grid node counts. “+1” includes both boundaries in each direction.

Listing 5 - Initial Temperature Field Initialisation (T1)

Initialisation of Temperature Matrix T1

Listing 5 initialises the temperature field. This implements the initial condition T(x,y,0)=0°C.

Listing 6 - Application of Dirichlet Boundary Condition on the North Edge (T0)

Apply Boundary Conditions

Listing 6 applies the north edge Dirichlet boundary condition. The above sets the top boundary to T0while other edges remain 0 °C.

## Time Step Selection and Stability Check

Listing 7 - Stable Time-Step Selection Enforcing Fo ≤ 1/4

Listing 7 computes the maximum stable time step and enforces Fo≤1/4. This prevents instability by restricting Δt.

## Initialise T2, Convergence Variables, and Plotting Grid

Listing 8 - Initialise T2, Convergence Variables, and Plotting Grid

Listing 8 prepares the second temperature array and convergence tracking. This sets up the solver state and grid vectors for plotting.

## Live Temperature Contour Plot During Iteration

Listing 9 - Temperature Contour Plot

Listing 9 creates the initial contour plot for the transient solution. This visualises temperature evolution during time marching.

Listing 10 - EFDM Iteration Loop

EFDM Update Loop and Convergence Check

Listing 10 updates interior nodes using EFDM and checks for convergence. This applies the 5-point stencil until max⁡∣Tn+1-Tn∣≤CC.

## Display Convergence Results

Listing 11 - Convergence Check and Iteration Counter Update

Listing 11 prints the converged numerical field and stopping statistics. This reports the final numerical field and convergence performance.

## Analytical Solution Setup

Listing 12 - Analytical Solution Setup

Listing 12 creates the grid for the analytical series evaluation. This prepares arrays for the analytical reference solution.

## Analytical Steady-State Solution (Series Summation)

Listing 13 - Analytical Steady-State Series Solution Assembly

Listing 13 evaluates the truncated Fourier series for TA(x,y). This produces the analytical steady-state temperature field for validation.

## Plot the Analytical Temperature Field

Listing 14 - Analytical Solution Contour Plot Generation for TA

Listing 14 plots the analytical solution as a contour map. The above allows direct visual comparison with the numerical result.

## Error Map and Validation Metrics

Listing 15 - Validation Error Field Calculation and Absolute Error Contour Plot

Listing 15 computes absolute error, plots it, and reports max error and RMSE. This quantifies agreement between EFDM and analytical results.

Listing 16 - Error Metrics (max error, RMSE) and Equilibrium Time Reporting

Time to Thermal Equilibrium Calculation

Listing 16 converts step count into physical time and prints it neatly. This reports the simulated time needed to satisfy the chosen convergence tolerance.

# Validation of the Code.

## Validation Approach

Validation compared the EFDM equilibrium field T2 with the analytical steady-state field TA on the same grid to quantify numerical accuracy for the specified boundary conditions (Ozisik, 1955). Agreement was assessed using T2 and TA contour plots, the absolute error map E=∣T2-TA∣, the maximum absolute error, and the RMSE (Press, Teukolsky, Vetterling, & Flannery, 2007).

## Validation Case Selected for Presentation (CC = 0.001)

The primary validation case compares the EFDM thermal-equilibrium (steady-state) temperature field T2 with the analytical steady-state field TA for X=10m, Y=10m, h=0.5m, T0=60°C and Δt=900s, giving Fo=0.2313, with CC = 0.001. The run converged in 492 steps and reported a thermal equilibrium time of 5.12 days. The validation metrics were:

- max⁡∣T2-TA∣=0.4360°C.

- RMSE(T2-TA)=0.0732°C.

Figure 1 - EFDM Steady-State Temperature Field

Figure 1 shows the EFDM equilibrium temperature field T2for the CC = 0.001 case.

Figure 2 - Analytical Steady-State Temperature Field

Figure 2 shows the analytical steady-state field TA on the same grid.

Figure 3 shows the absolute error map E=∣T2-TA∣for CC = 0.001.

Figure 3 - Absolute Error Field

Interpretation: The numerical and analytical contours are expected to show the same diffusion-dominated structure (isotherms smoothing from the heated north boundary into the interior). The low RMSE indicates strong overall agreement across the plate, while the maximum error highlights the worst-case deviation (Smith, 1985).

## Comparison Validation Case (CC = 0.1)

To demonstrate the effect of the stopping tolerance, a comparison run was performed with identical X, Y, h, T0, and Δt, but with CC = 0.1. This run converged in 103 steps and reported a thermal equilibrium time of 1.07 days. The validation metrics were:

- max⁡∣T2-TA∣=7.6524°C.

- RMSE(T2-TA)=3.5779°C.

Figure 4 shows the EFDM equilibrium temperature field T2 for CC = 0.1.

Figure 4 - EFDM Steady-State Temperature Field

Figure 5 shows the absolute error map E=∣T2-TA∣for CC = 0.1. (The analytical field is unchanged from Figure 6.2 and is therefore not repeated.)

Figure 5 - Analytical Steady-State Temperature Field

Interpretation: The substantially larger errors confirm that a loose tolerance can terminate iterations while residual transient behaviour remains. This is consistent with convergence criteria based on successive iterate differences: a larger CC allows the algorithm to stop earlier at a state that is less close to the steady solution (Press, Teukolsky, Vetterling, & Flannery, 2007).

## Validation Conclusion

Accuracy is strongly dependent on the equilibrium tolerance CC: a strict tolerance (CC=0.001) gives close agreement with the analytical steady state, whereas a loose tolerance (CC=0.1) reduces computation time but produces noticeably larger errors (Morton & Mayers, 2005). For CC=0.001, agreement is strong, with maximum error below 0.5 °C and RMSE below 0.1 °C

# Sensitivity Analysis of the Code.

## Purpose and Method

This chapter assesses sensitivity to grid spacing h, time step Δt and equilibrium tolerance CC using the reported thermal equilibrium time, defined as the first time at which max⁡∣Tn+1-Tn∣≤CC (Morton & Mayers, 2005). Stability is enforced using the Fourier number Fo=αΔt/h2, and all runs satisfy the explicit stability requirement (Smith, 1985). One parameter is varied at a time while the others are held constant. All sensitivity tests used the same north boundary temperature, with T0 = 60° applied throughout.

## Influence of grid spacing h(fixed Δt=150s, fixed CC=0.001)

Table 1 - Thermal Equilibrium Time Results for the Grid Spacing Sensitivity Study on a 12 m × 12 m Plate with Δt=150s and CC=0.001.

Table 1 summarises the grid spacing sensitivity study for a 12 m × 12 m plate, with Δt=150s and CC=0.001 held constant.

Figure 6 - Thermal Equilibrium Time as a Function of Grid Spacing h for a 12 m × 12 m Plate with Δt=150s and CC=0.001

Figure 6 shows the reported thermal equilibrium time plotted against grid spacing h using the results in Table 1.

Figure 6 shows that equilibrium time changes for coarse grids but then plateaus as his reduced. This indicates that, with fixed Δt and tolerance, the stopping criterion increasingly controls termination rather than mesh refinement (Press, Teukolsky, Vetterling, & Flannery, 2007). Once per-step updates fall below the threshold, further reductions in hhave little effect on the reported equilibrium time (LeVeque, 2007).

## Influence of time step Δt(fixed h=0.75m, fixed CC=0.0001)

Table 2 - Thermal Equilibrium Time Results for the Time Step Sensitivity Study on a 12 m × 12 m Plate with h=0.75m and CC=0.0001

Table 2 summarises the time-step sensitivity study for a 12 m × 12 m plate, with h=0.75m and CC=0.0001 held constant.

Figure 7 shows the reported thermal equilibrium time plotted against Δt using the results in Table 2.

Figure 7 - Thermal Equilibrium Time as a Function of Time Step Δtfor a 12 m × 12 m Plate with h=0.75m and CC=0.0001

Figure 7 shows that the reported equilibrium time decreases as Δt decreases. Because equilibrium is detected using a successive-difference criterion, smaller Δt reduces the update magnitude and can cause max⁡∣Tn+1-Tn∣ to fall below the tolerance earlier in physical time (Press, Teukolsky, Vetterling, & Flannery, 2007). Therefore, equilibrium time trends across Δt reflect both diffusion behaviour and the stopping-rule definition (Morton & Mayers, 2005).

## Influence of thermal equilibrium tolerance CC(fixed h=1m, fixed Δt=500s)

Table 3 - Thermal Equilibrium Time Results for the Tolerance Sensitivity Study on a 12 m × 12 m Plate with h=1m and Δt=500s

Table 3 summarises the tolerance sensitivity study for a 12 m × 12 m plate, with h=1m and Δt=500s held constant.

Figure 8 - Thermal Equilibrium Time as a Function of Equilibrium Tolerance CC for a 12 m × 12 m Plate with h=1m and Δt=500s

Figure 8 shows the reported thermal equilibrium time plotted against tolerance CC using the results in Table 3.

Figure 8 shows that the equilibrium time increases markedly as CC is tightened, because smaller tolerances require more steps for successive fields to converge (Morton & Mayers, 2005). This confirms that CC strongly controls runtime and how close the final field is to steady conditions, so comparisons are only meaningful when CC is consistent across runs (LeVeque, 2007).

## Sensitivity Summary

Across the sensitivity studies, CC has the strongest effect on reported equilibrium time, indicating that the stopping criterion dominates the measured “equilibrium” behaviour relative to changes in h and Δt (Press, Teukolsky, Vetterling, & Flannery, 2007). This motivates using more robust equilibrium measures (e.g., residual-based criteria) and alternative time-integration methods, as discussed in Chapters 8 and 9 (LeVeque, 2007).

# Limitations of the Numerical Model.

## Limitations of the Numerical Method (EFDM)

The explicit FTCS diffusion scheme is conditionally stable and requires Fo≤1/4 in 2D. Because Δtmax∝h2, grid refinement rapidly reduces the allowable time step and increases computational cost, which constrains practical parameter selection in the sensitivity studies (Morton & Mayers, 2005).

The scheme is first-order accurate in time and second-order accurate in space, with truncation error scaling O(Δt)+O(h2) (Smith, 1985). Although reducing h and Δt should reduce discretisation error in principle, the trends in Tables 1–2 are not strongly monotonic, indicating that the stopping criterion can dominate the reported outcomes and mask truncation-error behaviour (Smith, 1985).

Round-off error can accumulate over many iterations, but it is expected to be secondary here because MATLAB uses double precision and the largest discrepancies are associated with changes in the stopping tolerance (Press, Teukolsky, Vetterling, & Flannery, 2007). Round-off becomes more relevant for very large grids, extremely small tolerances, or long runtimes with high iteration counts (Press, Teukolsky, Vetterling, & Flannery, 2007).

Finally, EFDM on a structured Cartesian grid is best suited to simple rectangular geometries and boundary conditions (Lach & et al, 2025); complex geometries typically require boundary approximations or alternative formulations such as FEM or FVM (LeVeque, 2007).

## Limitations of the Coding Implementation (Evidence Linked)

Thermal equilibrium is defined using the successive-difference criterion max⁡∣Tn+1-Tn∣≤CC, which measures per-step change rather than the residual of the steady-state equation. This means termination depends on update magnitude, which scales with Fo=αΔt/h2 (Morton & Mayers, 2005).

This limitation is supported by the sensitivity results: in the time-step study (Table 2), reducing Δt (and hence Fo) led to earlier reported equilibrium times while not consistently improving agreement with the analytical solution, indicating that smaller updates can satisfy the tolerance sooner without reaching the same physical proximity to steady state (Press, Teukolsky, Vetterling, & Flannery, 2007). A similar effect is observed across discretisation’s, where the stopping rule can cause “equilibrium” to represent different physical states unless termination is defined consistently (Press, Teukolsky, Vetterling, & Flannery, 2007).

Therefore, results are only directly comparable when CC is controlled to achieve similar proximity to steady state, as demonstrated by Table 3 where tightening CC from 0.1 to 0.001 reduced RMSE from 3.5779 °C to 0.0732 °C.

## Limitations of the Analytical Reference Solution

The analytical steady-state solution is evaluated using a truncated Fourier series. Truncation introduces an approximation error that decreases with increasing term count, but convergence can be slower near boundaries when the derivative is discontinuous, meaning some residual analytical approximation may remain (Ozisik, 1955). Although 10,000 terms were used to reduce truncation effects, the reference solution remains an approximation and contributes a small uncertainty floor for validation.

## Summary of Limitations

The EFDM scheme is stable under the Fourier-number constraint, but efficiency is limited by conditional stability, and the reported accuracy is strongly influenced by the equilibrium tolerance. Sensitivity results show that the stopping rule can dominate trends across h and Δt, so robustness requires a residual-based criterion or a fixed physical end time (LeVeque, 2007).

# Suggested Improvements and Future Developments.

## Improved Equilibrium (Steady-State) Detection

Robustness can be improved by defining equilibrium using a criterion tied to the steady-state equation rather than successive-iterate differences. A residual-based stop can be formed from the discrete Laplacian,

Ri,j=Ti+1,j+Ti-1,j+Ti,j+1+Ti,j-1-4Ti,j,

with termination when max⁡∣R∣ falls below a chosen tolerance, directly indicating satisfaction of Laplace’s equation at interior nodes (LeVeque, 2007). Alternatively, runs can be compared at a fixed physical time to isolate the influence of Δt on temporal error and avoid premature termination driven by per-step tolerances (Press, Teukolsky, Vetterling, & Flannery, 2007).

## More Efficient Time Integration

The explicit method requires a small Δt for stability, particularly on refined grids, whereas implicit schemes (backward Euler) and Crank–Nicolson relax this restriction for linear diffusion and allow larger stable time steps (Crank, 1975). Crank–Nicolson also provides second-order temporal accuracy, improving transient predictions (Morton & Mayers, 2005). More stable time-integration approaches are therefore commonly considered for heat-conduction simulations to reduce the practical limitations of explicit schemes (Jalghaf, Omle, & Kovacs, 2022). For structured 2D grids, ADI methods reduce computational cost by replacing a full 2D implicit solve with two 1D implicit steps (Peaceman & Rachford, 1955).

## Grid Convergence and Uncertainty Reporting

A more defensible numerical study would include a grid convergence assessment where h is reduced while using either:
(i) a residual-based equilibrium criterion, or
(ii) a consistent physical end time,
to ensure all cases represent a comparable physical state. With this in place, observed changes in max error and RMSE can be attributed to discretisation effects rather than termination effects (Smith, 1985).

Uncertainty reporting could also be strengthened by providing relative error metrics (e.g., normalised RMSE divided by T0) and reporting where the maximum error occurs spatially (e.g., near corners or mid-domain) (Press, Teukolsky, Vetterling, & Flannery, 2007).

## Alternatives for Complex Geometries and Unstructured Meshes

EFDM is most efficient on structured Cartesian grids and simple rectangular domains, whereas complex geometries and local refinement are typically handled using FVM or FEM. FVM enforces conservation over control volumes and is well-suited to unstructured meshes and irregular boundaries (Versteeg & Malalasekera, 2007). FEM approximates the solution using basis functions over elements and is effective for complex geometries, spatially varying properties and locally refined meshes (Zienkiewicz & Taylor, 2000). These methods improve geometric fidelity and enable targeted refinement near boundaries or high gradients without refining the entire domain (LeVeque, 2007).

# Conclusion.

This report modelled 2D transient heat conduction in an aluminium plate using an explicit FTCS finite difference scheme with stability enforced by Fo≤1/4. Validation against a truncated Fourier-series analytical steady state showed strong dependence on the equilibrium tolerance: for X=10m, Y=10m, h=0.5m and Δt=900s, tightening CC from 0.1 to 0.001 reduced RMSE from 3.5779 °C to 0.0732 °C and the maximum absolute error from 7.6524 °C to 0.4360 °C. Sensitivity results indicated that the successive-difference stopping rule can dominate trends across h and Δt, producing earlier termination and non-robust “equilibrium” times as update magnitudes decrease. Overall, the EFDM implementation was validated, and the dominant reliability controls were identified; future work should adopt residual-based equilibrium criteria and consider implicit or Crank–Nicolson time integration to improve robustness and efficiency.

# References

Bergman, T., Lavine, A., Incorpera, F., & DeWitt, D. (2011). Fundamentals of Heat and Mass Transfer. Hoboken: Wiley.

Carslaw, H., & Jaeger, J. (1959). Conduction of Heat in Solids. Oxford: Clarendon Press.

Crank, J. (1975). The Mathematics of Diffusion. Oxford: Claredon Press.

Incropera, F., DeWitt, D., Bergman, T., & Lavine, A. (2011). Introduction to Heat Transfer. Hoboken: Wiley.

Jalghaf, H., Omle, I., & Kovacs, E. (2022). A Comprehensive Study of Explicit and Stable Tim Integration Schemes for Heat Conduction in an Insulated Wall. Buildings, 824. doi:10.3390

Lach, L., & et al. (2025). Advances in Numerical Modeling for Heat Transfer and Thermal Management: A Review of Computational Approaches and Environemntal Impacts. Oxford: Energies.

LeVeque, R. (2007). Finite Difference Methods for Ordinary and Partial Differential Equations: Steady-State and Time-Dependant Problems. Philadelphia: SIAM.

Morton, K., & Mayers, D. (2005). Numerical Solution of Partial Differential Equations: An Introduction. Cambridge: Cambridge University Press. Retrieved January 20, 2025

Ozisik, M. (1955). Boundary Value Problems of Heat Conduction. New York: Dover Publications.

Peaceman, D., & Rachford, H. (1955). The Numerical Solution of Parabolic and Elliptic Differential Equations. Journal of the Society for Industrial and Applied Mathematics(3), 28-41. doi:10.1137/0103003

Press, W., Teukolsky, S., Vetterling, W., & Flannery, B. (2007). Numerical Recipes: The Art of Scientific Computing. Cambridge: Cambridge University.

Richtmyer, R., & Morton, K. (1967). Difference Methods for Initial-Value Problems. New York: Interscience.

Smith, G. (1985). Numerical Solution of Partial Differential Equations: Difference Methods. Oxford: Oxford University Press. Retrieved January 20, 2025

Versteeg, H., & Malalasekera, W. (2007). An Introduction to Computational Fluid Dynamics: The Finite Volume Method. Harlow: Pearson.

Zienkiewicz, O., & Taylor, R. (2000). The Finite Element Method: Volume 1 - The Basis. Oxford: Butterworth-Heinemann.

# Appendix

## MATLAB Code

clc

clear

close all

%Material Properties for Aluminium

k = 143;   % W/m*C    Conductivity

p = 2800;  % kg/m^3   Density

c = 795;   % J/kg*C   Specific Heat

%Thermal Diffusivity

alpha = k / (p * c);  % m^2/s

fprintf('Thermal diffusivity alpha = %0.2e m^2/s\n',alpha)

%Prompt and validate X, Y, h

while true

X = input('Enter plate X dimension (positive scalar): ');

Y = input('Enter plate Y dimension (positive scalar): ');

h = input('Enter grid spacing h (positive scalar): ');

% basic checks

if isempty(X) || isempty(Y) || isempty(h)

fprintf('Empty input detected — please provide numeric values.\n');

continue;

elseif any([X,Y,h] <= 0)

fprintf('All inputs must be positive values. Try again.\n');

continue;

elseif rem(X,h)~0 || rem(Y,h)~=0 || h>=Xdim || h>=Ydim

fprintf('Grid spacing h must divide both X and Y (within tolerance). Try again.\n');

continue;

else

break;

end

end

%Calculate total number of rows NR and columns NC

NR = round(Y/h) + 1;

NC = round(X/h) + 1;

%Initialise Matrix T1

T1=zeros(NR,NC);

%Add the boundary conditions

T0=input('Please enter the North boundary condition temperature: ');

T1(1,2:NC-1)=T0;

%User input for temporal consideration (Stability check: Fo <= 1/4)

dT_max = h^2/(4*alpha);

TS = input(sprintf('Enter time step dT (<= %.6g s): ', dT_max));

while TS <= 0 || TS > dT_max

fprintf('Invalid dT. Must be > 0 and <= %.6g s\n', dT_max);

TS = input(sprintf('Enter time step dT (<= %.6g s): ', dT_max));

end

Fo = alpha*TS/h^2;

fprintf('Accepted: Fo = %.4f\n', Fo);

%Initialise T2 and keep Dirichlet boundary conditions constant

T2 = T1;                % start T2 from T1

%Convergence Criterion (CC) and explicit update until convergence

CC = 0.1;

maxDiff = CC+2;

step = 0;

%

% %Live display of numerical temperature at each time step

x = 0:h:X;

y = 0:h:Y; % row 1 corresponds to y = Y (top boundary)

figure;

hImg = contourf(x, y, flip(T2));

set(gca,'YDir','normal');

colorbar;

xlabel('X (m)');

ylabel('Y (m)');

while maxDiff > CC

step = step + 1;

% Explicit Finite Difference update (interior nodes only)

for i = 2:NR-1

for j = 2:NC-1

T2(i,j) = T1(i,j) + Fo*( ...

T1(i+1,j) + T1(i-1,j) + T1(i,j+1) + T1(i,j-1) - 4*T1(i,j) );

end

end

% Convergence check (max difference anywhere)

maxDiff = max(abs(T2(:) - T1(:)));

% Update live plot

hImg = contourf(x, y, flip(T2));

title(sprintf('Temperature T2 (°C) at t = %.2f s', step*TS));

drawnow;

T1 = T2;

end

disp('T2 ='); disp(T2)

fprintf('Converged in %d steps. Max difference = %.4f\n', step, maxDiff);

%Analytical solution (matrix TA)

%Using assignment formula with: Xdim = X, Ydim = Y, k = 2n+1

[Xg, Yg] = meshgrid(x, y);

Nterms = 10000;          % number of terms in the series (increase for accuracy)

TA = zeros(NR, NC);

for n = 0:Nterms

kterm = 2*n + 1;   % k = 2n+1 (odd terms)

num = exp( (kterm*pi*(Yg - Y))/X ) - exp( (-kterm*pi*(Yg + Y))/X );

den = 1 - exp( (-2*kterm*pi*Y)/X );

TA = TA + (1/kterm) * sin(kterm*pi*Xg/X) .* (num ./ den);

end

TA = (4*T0/pi) * TA;

%Plot temperature distribution TA

figure;

contourf(x, y,(TA));

set(gca,'YDir','normal');

colorbar;

title('Temperature distribution TA (°C)');

xlabel('X (m)');

ylabel('Y (m)');

disp('TA ='); disp(flipud(TA))

%Code validation: error map (single figure)

E = abs(T2 -flipud(TA));   % absolute error

x = linspace(0, X, NC);

y = linspace(Y, 0, NR);   % row 1 is top boundary (y = Y)

figure;

contourf(x, y,E);

set(gca,'YDir','normal');

colorbar;

title('Validation: Absolute error |T2 - TA| (°C)');

xlabel('X (m)');

ylabel('Y (m)');

disp('E =');  disp(E)

fprintf('\n--- Code Validation (T2 vs TA) ---\n');

fprintf('Max |T2 - TA| = %.4f °C\n', max(E(:)));

fprintf('RMSE(T2 - TA) = %.4f °C\n', sqrt(mean(E(:).^2)));

%Time to Thermal Equilibrium

t_eq = step * TS;   % time to reach thermal equilibrium (s)

if t_eq < 60

fprintf('Time to thermal equilibrium: %.2f s\n', t_eq);

elseif t_eq < 3600

fprintf('Time to thermal equilibrium: %.2f min\n', t_eq/60);

elseif t_eq < 86400

fprintf('Time to thermal equilibrium: %.2f h\n', t_eq/3600);

else

fprintf('Time to thermal equilibrium: %.2f days\n', t_eq/86400);

end