| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |

MODULE CODE: M5X02201

MECHANICAL PRINCIPLES

Phase test 1 RESIT

TIME:  2 HOURS

answer ALL questions

Equipment or materials required – Calculator

A ‘formula sheet’ is provided for reference.

Assume ‘g’  =  9.81 m/s2.

and  1 bar  =  105 N/m2 (Pa)

ANSWER ALL QUESTIONS

QUESTION ONE

At a certain point in a material there is a strain in the ‘x’ direction of -12 x 10-6 and a strain in the ‘y’ direction of +16 x 10-6. The material has a Poisson’s ratio of 0.25 and a Modulus of Elasticity of 163 GPa.

- Calculate the corresponding stresses in each direction at this point in the material and state whether they are compressive or tensile.

(5 marks)

- If at a second point in the material the stresses are found to be 25MPa (tensile) in the ‘x’direction and -20 MPa (compressive) in the ‘y’ direction, calculate the corresponding strains.

(5 marks)

QUESTION TWO

A cube, made from an Isotropic material, of side length 300mm is subjected to an external fluid pressure of 55 MPa.

- If the modulus of elasticity of the cube material is 163.2 GPa and the Poisson’s ratio is 0.3, calculate the bulk modulus

(3 marks)

- Calculate the volumetric strain.

(3 marks)

- Calculate the change in volume.

(2 marks)

- Explain what is meant by the term ‘orthotropic material’ and give an example material which exhibits orthotropic characteristics.

(2 marks)

QUESTION THREE

The diagram below shows a simply supported beam with a single point load and two uniformly distributed loads. If the reaction forces RA and RB are 1312.9N and 1387.1N respectively, by applying Macaulay’s method

- Write down an equation for the bending moment at any point along the beam.

(2 marks)

- Write down an equation for the slope at any point along the beam.

(2 marks)

- Write down an equation for the deflection at any point along the beam

(1 mark)

- Apply boundary conditions and calculate the constants of integration.

(3 marks)

- Calculate the deflection at the centre on the beam if the flexural stiffness is 1.00MPa.

(2 marks)

2.0 kN

600 N/m

100 N/m

1.0 m

1.0 m

0.6 m

0.5 m

RB

RA

QUESTION FOUR

A hydrogen storage tank for a vehicle may be modelled as a thin walled pressure vessel. The cylinder has a diameter of 280mm, is 600mm in length and is made from sheet steel, The longitudinal and circumferential joint efficiencies are 55% and 65% respectively. If the cylinder is to withstand an internal pressure of 14.6 Bar with the maximum stress within the material not exceeding 150MPa calculate,

- Calculate the minimum thickness based on the circumferential stress

(5 marks)

- Calculate the minimum thickness based on the longitudinal stress

(4 marks)

- The minimum thickness.

(1 mark)

QUESTION FIVE

Assuming that the brake master cylinder shown above may be modelled as a perfect cylinder with an external diameter of 48mm and the internal diameter is 38mm, determine the circumferential ‘hoop’ stress at the outer surface when the fluid pressure reaches 145 Bar.

- State the boundary conditions to be applied

(3 Marks)

- Calculate the constants, a and b, in Lame’s Equations for this application

(5 Marks)

- Calculate the corresponding circumferential stresses at the inner and outer radii

(2 marks)

Diwedd/End

Mechanical Principles Formula Sheet 1

Two-dimensional stress and strain equations:

Three-dimensional strain equations:

Volumetric Strain:

Relationships between elastic constants:

Stress and strains in thin walled pressure vessels:

Cylinders					Spheres

Stresses in thick walled cylinders: (Lame’s Equations)

Where a and b are found by applying boundary conditions

Loaded beams : Standard Cases

Cantilever Beams:

Simply Supported Beams: