| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2023 | mai 2023 |

HND Electrical and Electronic Engineering

MODULE CODE: GOEE4002

Dynamics Control and Instrumentation

TIME:  2 HOURS

answer all Four questions

Equipment or materials required – Calculators are allowed

Adnoddau neu ddefnyddiau sydd eu hangen -

Question One (20 marks)

- A 50 N applied force drags a 20 kg log to the right across a horizontal surface. What is the acceleration of the log if the force of friction is 10 N?

5 Marks.

Answer:

Fnet=ma

Fa-Ff=ma

50-10=10a

a=4020=2.0 m/s2

- A body travelling at an initial velocity of 25m/s is acted upon by a linear acceleration of 10m/s2 .

5 Marks

Calculate:

a	Velocity after 5 seconds

b	Total distance travelled

Answer:

S=u+at	S=25+(10)(5)	S = 75m/s

S=u+v2t	S=25+7525		S = 250m

- Using sketches to illustrate your answer, compare and contrast an Open Loop and a Closed Loop system.

4 Marks

Answer:

Sketches

- Mason’s Gain Rule is used to reduce the block diagram to a single transfer function as shown:

TF(s)=1∆inPi∆i

Give the definition of each variable in the formal above.

6 Marks

Answer:

Δ = The determinant of the system

n = Number of forward paths

Pi= The ith forward-path gain

Δi = The determinant of the ith forward path

Δ = 1 - (Sum of all individual loop gains) + (sum of products of gains of all pairs of non-touching loops) – (sum of products of gains of all three non-touching loops)

Δi = 1 if no non-touching loops on the ith path OR if removing the ith path breaks all the loops.

Question Two (10 Marks)

The figure below shows a step input:

Draw a system time/response graph for the step input shown. Define the following terms and show clearly on the graph:

- Rise time

- Settling time

- Steady state error

- Maximum overshoot

- Delay time

Answer:

Question Two (20 marks, 5 marks each)

Three particles A, B and C have a motion which can be described by the following expressions:-

For particle A   Sa = 3t2 + 10t - 8

For particle B    Sb = 7t3 + 4t2 + 4

For particle C    Sc = 10t3 - 30t + 2

Where S = displacement in m, t = time in seconds.

Determine:

- The time when the velocity of particle B is three times that of particle A to 2 decimal places?

Answer:

For particle A  Va=dSadt=6t + 10

For particle B   Vb=dSbdt=21t2 + 8t

The velocity of particle B is 3 times that of particle A, i.e Vb=3Va

21t2 + 8t=3(6t + 10)

21t2 + 8t=18t + 30

21t2- 10t-30=0

Then t=-0.9 ignore,

or t=1.46s   Answer

- The time when the acceleration  of particle B  is equal to that of particle C to 3 decimal places

Practice B a= 42t + 8

Practice C a = 60t

42t + 8=60t

18t=8

t=8/18=0.444 s

- The distance travelled by each particle in the period from t= 0 to t= 4 seconds

For particle A

∆S=S4-S1

∆S={3(4)  + 10(4) – 8}-{3(0)2 + 10(0) – 8}= 88 m

For particle B

∆S=S4-S1

∆S={7(4)3 + 4(4)2 + 4}-{7(0)3 + 4(0)2 + 4}= 512 m

For particle C

∆S=S4-S1

∆S={10(4)3 – 30(4) + 2}-{10(0)3 – 30(0) + 2}= 520 m

- The time required for practical B to reach the velocity of 10 ms?

Vb=dSbdt=21t2 + 8t

10=21t2 + 8t

21t2 + 8t-10=0

By using factorization we can find that

t=-0.86 ignore,

or t=0.5 s      Answer

Queestion Four (20 marks)

For the given system block diagram:

G3

G4

G55

H2

G2

G1

H1

Y(s)

R(s)

+

+

+

_

+

+

_

+

H3

G3

G4

G55

H2

G2

G1

H1

Y(s)

R(s)

+

+

+

_

+

+

_

+

H3

- Produce a Signal Flow Graph of the system from the block diagram showing all nodes, branches, and gains.

- Apply Masons Gain Formula to determine the transfer function Y(s)R(s) of the system.

Good luck

End of the exam