Assignment 1 of 1

Structural Design and Analysis

Rhys Ratner (1803424)

- Design report cover page

- The sketch (es) of the structural scheming identifying the position of columns and beam to support the structure (you can draw this with any CAD tools or manually).

- Complete loading schedules for the structure considering the intended use as annotated on the architectural drawing (ACBA 6007_CW_Drawings).

- A durability design for the structure to be constructed beside the IQ building in Swansea

- A complete analysis of the loading on the most critical slab and the design of the slab using reinforced concrete (Note: provide justification for the identification of the most critical slab).

- A complete analysis and design of a two-span continuous beam within the structure using a material of your choice.

- A complete analysis and design of a column within the structure using a material of your choice.

- A complete design of a reinforced concrete pad footing for the chosen column in step 7.

Table of Contents

- - Structural scheming

- - Loading Schedule

- - Durability Study

- - Beam Loading

- - Column Loading

- - Conclusion

1.0 – Structural Scheming

See figure 1 showing a structural scheming plan. The beams and columns have been put in the locations where they will not affect the ergonomic of the downstairs living area by not having any beams or columns on show.

P.1

P.2

There are 7no panels where the load will need to be estimated in the loading schedule.

Panel 1 includes Bedroom 2.

P.4

Panel 2 includes Bedroom 4 and a toilet room.

P.3

Panel3 Includes Toilet, Stairs and a hallway.

Panel 4 includes Family Lounge and Toilet.

Panel 5 Includes Bedroom 3, hallway and an en suite.

Panel 6 includes the Master Bedroom.

Panel 7 includes the balcony.

P.7

P.6

P.5

Figure 1 - Structural Scheming Plan

2.0 – Loading Schedule

The table below shows the Schedule of loading for each room in the proposed family dwelling.

1.5

1.5

1.5

1.5

1.5

1.5

1.5

1.5

1.5

1.5

1.5

1.5

0.3

0.3

0.3

0.3

0.3

0.3

1.2

1.2

1.2

1.2

1.2

1.2

Figure 2 - Loading Schedule

Figure 3 - Construction Materials-concrete and mortar

The 2 below tables have been taken from the Eurocodes. Table 6.2 shows the imposed loads on different areas in the building and Table A.1 shows density of different strengths of concrete.

Figure 4 - Imposed Loads

To estimate the thickness of the slab, I have taken the longest beam length of 5780mm and divided it by 23 to give me a thickness of 250mm.

In order to complete the loading on the slab, it is required to carry out a calculation for dead load (gk) and live load (qk).

- Dead Load

This includes the self-weight of the slab, weight of finishes and services (Assumed 1.5km/m2).

Self Weight =Density ×Volume

Self Weight.

=24 × 0.25

=6 kn/m²

Total gk

=6+1.5

=7.5 kn/m²

Total Live Load

=1.5kn/m²

Ultimate Load

Qk =7.5 ×1.4

=10.5kn/m²

Gk=1.5 ×1.6

=2.4 kn/m²

=10.5+2.4

=12.9 kn/m²

Balcony Loading

Gk=2.5 ×1.6

=4

=10.5+4

14.5 kn/m²

Figure 5 - Loading Plan

To determine whether a slab is 2-way load spread or 1-way. It is required to carry out the following equation.

LyLx , If the outcome of the equation is equal to or greater than then the slab is 1 way, if its less than 2 it is 2 way.

P1 - LyLx = 4.134.09 = 1.01   P5 - LyLx = 5.455.42 = 1.01

P2 - LyLx = 5.184.13 = 1.25   P6 - LyLx = 3.973.82 = 1.04

P3 - LyLx = 5.454.33 = 1.26   P7 - LyLx = 3.121.73 = 1.8

P4 - LyLx = 5.783.82 = 1.51

It is clear from the above calculations from each panel, that they are all 2-way spreads as all the results of the equations are less than 2.

See Figure 5 which shows the estimated loading for each beam in the design.

3.0 – Durability Design

DESIGN CALCULATION FOR DURABILITY

| Reference | Description |  |
| --- | --- | --- |
|  | FLOOR SLAB |  |
| Eurocode 3.1 | Bar Size Assumed Bar Size = 12mm Nominal Cover C ≥ 12mm |  |
|  | Aggregate Size Nominal maximum size of aggregate Hagg = 21mm Hagg ≤ 0.25 x Slab Thickness = 0.25 x 150 = 37.5mm Nominal cover ≥ 20mm | Hagg is Ok |
|  | Exposure Condition Environment is considered to be Mild Concrete Grade C25 is intended to be used. Nominal cover ≥ 25mm |  |
|  | Fire Resistance 1hr fire resistance is intended Nominal cover ≥ 20mm Minimum depth of slab panel = 95mm Depth of slab panel =150mm. | Slab depth ok |
|  | Nominal cover to all floor slab = 25mm | C = 25mm |
|  | FLOOR BEAMS |  |
|  | Bar Size Assumed Bar Size = 16mm Nominal Cover C ≥ 16mm |  |
|  | Aggregate Size Nominal maximum size of aggregate Hagg = 20mm Hagg ≤ 0.25 x Beam width = 0.25 x 200 = 50mm Nominal cover ≥ 20mm | Hagg is Ok |
| Reference | Description |  |
|  | Exposure Condition Environment is considered to be Mild Concrete Grade C25 is intended to be used. Nominal cover ≥ 25mm |  |
|  | Fire Resistance 1hr fire resistance is intended Nominal cover ≥ 20mm Minimum width of beam = 200mm Width of beam = 230mm | Beam width ok |
|  | Nominal cover to all floor beams = 25mm | C = 25mm |
|  | COLUMNS |  |
|  | Bar Size Assumed Bar Size = 16mm Nominal Cover C ≥ 16mm |  |
|  | Aggregate Size Nominal maximum size of aggregate Hagg = 20mm Hagg ≤ 0.25 x col. width = 0.25 x 150 = 37.5mm Nominal cover ≥ 20mm | Hagg is Ok |
|  | Exposure Condition Environment is considered to be Mild Concrete Grade C25 is intended to be used. Nominal cover ≥ 25mm |  |
|  | Fire Resistance 1hr fire resistance is intended Nominal cover ≥ 20mm Minimum width of fully exposed = 150mm Minimum width of 50% exposed = 125mm Minimum width of one face exposed =100mm |  |
|  | Nominal cover to all columns = 25mm | C = 25mm |
|  | FOUNDATION BEAMS |  |
|  | Bar Size Assumed Bar Size = 16mm Nominal Cover C ≥ 16mm |  |
| Reference | Description |  |
|  | Aggregate Size Nominal maximum size of aggregate Hagg = 20mm Hagg ≤ 0.25 x Beam width = 0.25 x 230 = 57.5mm Nominal cover ≥ 20mm | Hagg is Ok |
|  | Concrete is cast against an adequate 50mm concrete blinding. Nominal cover ≥ 40mm |  |
|  | Exposure Condition Environment is considered to be Moderate Concrete Grade C25 is intended to be used. Nominal cover ≥ 40mm As Grade C25 is to be used, nominal concrete cover is increased to 50mm to provide adequate minimum cover against corrosion. |  |
|  | Fire Resistance 1hr fire resistance is intended Nominal cover ≥ 20mm Minimum Beam width = 200mm |  |
|  | Nominal cover to all foundation beams = 50mm | C = 50mm |
|  | FOUNDATION SLABS |  |
|  | Bar Size Assumed Bar Size = 12mm Nominal Cover C ≥ 12mm |  |
|  | Aggregate Size Nominal maximum size of aggregate Hagg = 20mm Hagg ≤ 0.25 x Assumed Slab depth = 0.25 x 150 = 37.5mm Nominal cover ≥ 20mm | Hagg is Ok |
|  | Concrete is cast on top of an adequate 50mm concrete blinding. Nominal cover ≥ 40mm |  |
| Reference | Description | Output |
|  | Exposure Condition Environment is Moderate C25 Concrete Grade is intended to be used. Nominal cover ≥ 40mm As Grade C25 is to be used, nominal concrete cover is increased to 50mm to provide adequate minimum cover against corrosion. |  |
|  | Fire Resistance 1hr fire resistance is intended Nominal cover ≥ 20mm Assumed minimum Slab depth = 95mm |  |
|  | Nominal cover to all foundation slabs = 50mm | C = 50mm |

4.0 – Slab Design

Figure 6 - Slab Design Process

The first part to completing the slab design is to determine the maximum bending moment for the beam.

In the design the critical slab is P.4 as it contains the largest span.

The first formula to carry out the maximum bending moment is

=WL²8

= 12.9×5.78²8

=53.87 Kn/m

As class C25 concrete is being it is less than C50/60.

The next Formula is

-K=M/Fcubd²

Where;

M = 53.87Knm

Fcu = 25Nmm²

B = 3820mm

H = 200mm

D = 200 – (16/2) – 20 = 172

-K=5387025 ×3820 x 172²

- K =0.043

The next equation is

z= d21+ 1-3.53k

z=172 0.5+ (0.25-0.43÷0.9)

z=163.35

The final equation is Area of steel reinforcement

AS=MFydZ

Where;

M = 53.87knM

Fyd = 600Nmm²

Z = 163.35

AS= 53870600 ×163.35

AS=55mm