## 1. Offsite Construction – Definition and Examples (5%)

Offsite construction refers to the process where building components are manufactured and pre-assembled in a controlled factory setting before being transported to and installed on the construction site. This modern method of construction (MMC) offers improved quality control, faster build times, reduced on-site waste, and less disruption to surrounding areas.

### The four main categories of offsite construction include:

- Volumetric (Modular) Construction: Complete 3D modules—such as entire rooms (e.g., bathroom or kitchen pods)—are manufactured off-site.

- Panelised Systems: Flat panel components (e.g., walls, floors) are assembled off-site and include systems such as Timber Twin Frame and SIPS (Structural Insulated Panels).

- Sub-assemblies and Components: Prefabricated parts like roof trusses, floor cassettes, or MEP (mechanical, electrical, and plumbing) systems.

- Hybrid Systems: Combines modular and panelized elements with traditional on-site construction methods for maximum flexibility.

## 2.i. Ground Floor Construction and External Wall Junctions (30%)

All three-construction methods interface with a raft foundation that includes a damp-proof membrane (DPM), floor insulation, and underfloor heating embedded within a screed layer. All comply with Building Regulations Part L concerning thermal performance.

Method Details:

### Timber Twin Frame

  - Construction: Twin timber stud wall with a central insulation-filled cavity.

  - Layers: Breathable membrane, sheathing board, internal vapour control layer.

  - External Finish: Brick cladding or render on battens.

  - Junction Detail: Timber wall anchored to sole plate on raft, airtight seal maintained at junction.

### SIPS (Structural Insulated Panels)

  - Construction: Factory-made panels with high-performance insulation core between two OSB (oriented strand board) faces.

  - Fixing: Panels secured to a treated timber sole plate.

  - External Finish: Typically render or rainscreen cladding with breathable membrane.

  - Junction Detail: Precise interlocking joint with minimal cold bridging at wall-floor interface.

### Brick and Block

  - Construction: Traditional cavity wall with inner blockwork leaf, full-fill cavity insulation, and outer brick/rendered masonry leaf.

  - DPC (damp-proof course) integrates with wall and floor DPM.

  - Junction Detail: Inner block wall built off raft slab; insulation continuity ensured across wall/floor junction.

Illustration (Recommended):
 A labelled diagram should accompany this section, showing the ground floor and external wall junctions for each method. This should highlight:

- Continuity of insulation and damp-proofing,

- Sole plate/foundation interface,

- Material layers in wall build-up.

## 2.ii. Window Opening Detailing (30%)

All construction methods incorporate triple-glazed aluminium windows designed for high thermal efficiency. Each installation includes an insulated lintel, damp-proof course (DPC), insulation, and air-tight sealing to meet modern building standards.

Method-Specific Window Detailing:

### Timber Twin Frame:

  - The window is installed within a pre-formed opening in the timber frame.

  - Sheathing board surrounds the frame, and a cavity closer ensures thermal and moisture protection around the reveal.

### SIPS:

  - Openings are factory-cut with high precision, with reinforced panel edges.

  - An air-tight cavity closer and sealant provide a robust, insulated finish around the window.

### Brick & Block:

  - Traditional masonry construction uses a cavity closer and insulated steel lintel.

  - The outer reveal is typically rendered for weather protection, with internal airtight detailing.

## 2.iii. Eaves Detailing and Roof Structure (30%)

The roof structure across all methods includes trussed rafters, OSB roof decking, and a metal profile roof covering. Soffits and fascia boards are used to finish the eaves, with attention to ventilation and moisture control.

Method-Specific Eaves Detail:

### Timber Twin Frame:

  - The twin-stud wall continues to the underside of the eaves.

  - A breathable membrane wraps over the wall to maintain ventilation.

  - An eaves ventilator strip is included for roofspace airflow.

### SIPS:

  - Panels extend to the eaves, providing continuous integrated insulation.

  - A weatherproof membrane is applied over the panels, with detailing to maintain air-tightness and water resistance.

### Brick & Block:

  - The full cavity wall construction continues to the wall plate.

  - Insulation within the cavity is carried out to avoid thermal bridging.

  - A ventilated soffit ensures airflow to the roof void.

## 3. Recommendation and Justification (5%)

### Recommended Construction Method: Structural Insulated Panels (SIPS)

### Why SIPS is the Best Choice:

- Delivers excellent thermal performance and airtightness, meeting and exceeding energy efficiency standards.

- Enables rapid on-site assembly thanks to precise factory manufacturing, reducing build time and labour costs.

- The lightweight design is ideal for raft foundations, reducing groundworks and structural load.

- Compatible with various external finishes, including brick cladding or render, providing flexibility in appearance.

- Allows for an internal service void, simplifying installation of electrical and plumbing systems without compromising insulation.

- A sustainable choice, generating minimal site waste and supporting low-carbon construction goals.

Ground_Floor_Wall_Junction

Window_Opening_Detail