University of Wales Trinity Saint David

Wales Institute of Science and Art

Kinetic Energy Recovery

Alternative Propulsion Systems (ACHA5010)

Chloe Rees

2218480

October 2024

Table of Contents

1.Introduction	3

2.Vehicle Dynamics and Characteristics	3

2.1 Test Mass	3

2.2 Frontal Area	3

2.3 Coast down Test	4

2.4 Rolling Resistance Coefficient	5

2.5 Coefficient of Aerodynamic Drag	6

3.WLTP Cycle	7

3.1 WLTP Kinetic Energy Recovery	7

4.Forces Graphs	8

4.1 Acceleration Force	8

4.2 Aerodynamic Force	9

4.3 Total Forces	10

4.4 Power	11

5.Suitable Kinetic Energy Recovery Systems	12

5.1 Electric KERS	12

5. System Operations	13

5.1 How the System Operates	13

5.2 KERS Driving feel vs ICE Driving Feel	13

References	14

# 1.Introduction

In this document it explores the effect that kinetic energy recovery systems (KERS) have on vehicle fuel consumption. By selecting a vehicle, it can then be analysis to look at its different dynamic characteristics, such as rolling resistance and frontal aera. Acceleration, aerodynamics, power and total forces calculations and graphs will be produced and analysed using the Worldwide harmonised vehicle Test Procedure data for what category the selected vehicle falls into. Graphs will show where energy is lost during braking, then the identifications of different KERS and its energy storage and components will be considered to see how effective they are.

# 2.Vehicle Dynamics and Characteristics

The chosen vehicle is the 2022 Hyundai i30n line 1.5L GDI.

## 2.1 Test Mass

Using a website that gives specific data on vehicles. It shows that the test mass for the vehicle is 1250kg (Anon., n.d.).

## 2.2 Frontal Area

The frontal area is a great value to know as it can help to calculate other important data for a vehicle.

1.4m

1.7m

Figure 1- Hyundai i30n line

The Vehicle has a width of 1.7m and a height of 1.4m

Frontal Area=1.7 × 1.4=2.4m2

## 2.3 Coast down Test

The coast down test gives an indication of how long it takes for a vehicle to slow down. By gaining speed to 40km/h (11.1m/s) on a road, then putting the vehicle into neutral it will come to a steady stop. It will show the time taken to come back down to 10km/h (2.8m/s) and the velocity it goes down every second.

Figure 2- A graph showing a coast down test.

The whole time of the coast down test was 41 seconds and the time taken to get from 11.1m/s to 2.8m/s is 21 seconds.

## 2.4 Rolling Resistance Coefficient

In order to calculate the rolling resistance other calculations must be done first. By doing this it will show how efficient the vehicle is.

Air Density (ρ)= 1.223kg/m3

Coefficient of Drag (Cd)= 0.3

Frontal Area (A)= 2.4m2

Velocity (v)= 11.1m/s

Mass= 1250kg

Step 1:

Aerodynamic Drag= 12×ρ×Cd×A×v2

Equation 1-Aerodynamic Drag.

Aerodynamic Drag= 12×1.223×0.3×2.4×11.12

Aerodynamic Drag Losses=54.25N

Step 2:

Resistive Power=54.25×11.1

Resistive Power=602.18

Step 3:

Rolling Resistance Force(Frr)=Total Force-Aero Force

Equation 2- Rolling Resistance Force.

Frr=602.18-54.25

Frr=547.93N

Step 4:

Rolling Resistance CoefficientCrr= FrrMass × Gravity

Equation 3- Rolling Resistance Coefficient.

Crr= 547.931250 × 9.81

Crr=0.04

## 2.5 Coefficient of Aerodynamic Drag

Using the same data from the rolling resistance coefficient, it can be used to calculate the coefficient of aerodynamic drag by using the aerodynamic drag value from equation 1. By doing this, it will show how aerodynamic and streamlined the vehicle is.

Coefficient of Drag(Cd)=2×Aerodynamic Dragρ×A×v2

Equation 4- Coefficient of Drag.

Cd=2×54.251.223×2.4×11.12

Cd=0.3

Potential Energy(PE)=m×g×h

PE=1250×9.81×20

PE=245kJ

Work Done(WD)=54.25×500

WD=27kJ

# 3.WLTP Cycle

The Worldwide Harmonised Light Vehicle Test Procedure or WLTP is the new testing measurement for cars fuel economy, electric driving range and emissions produced after 2017. Before this the New European Driving Cycle or NEDC was used for testing cars since the 1980s. The WLTP is an accurate testing method showing fuel efficiency and emissions and matches it to how cars would be driven in the real world. It also takes into consideration such as rolling resistance and aerodynamic forces, when accelerating and braking (Dale, 2021).

## 3.1 WLTP Kinetic Energy Recovery

The graph below shows the WLTP acceleration, on the x-axis it shows the total time taken for the acceleration and on the y-axis is the speed in km/h. The graph shows many areas where a kinetic energy system is lost and could be stored and used for extra acceleration. This can be seen mainly in the highlighted braking zones of the graph; this is where kinetic energy can be regenerated into the storage system.

Figure 3- A graph showing WLTP acceleration.

# 4.Forces Graphs

## 4.1 Acceleration Force

The graph below shows the acceleration forces against the total time. The data shows that as the time increase the acceleration forces are high during acceleration and high velocity, then decreases when in the braking zones.

Figure 4- A graph showing the acceleration force.

## 4.2 Aerodynamic Force

figure 5 shows a graph with the aerodynamic forces against the total time. during the first part of the graph between 1-568 seconds these is very little to no aerodynamic forces on the vehicle. Between 568-1135 seconds there is a little more aerodynamic forces which could be affecting the grip and velocity of the vehicle. Finally, between 1135-1765 seconds there are high aerodynamic forces on the vehicle.

Figure 5- A graph showing aerodynamic forces.

## 4.3 Total Forces

The graph below shows the combined forces added up. During the slowest velocities the forces are low as the vehicle isn’t at a high speed. The forces can be seen to increase

Figure 6- A graph showing the total forces.

## 4.4 Power

The power in the graph below shows that there is more power being used at a higher velocity. The power in the negative is lost power and with a suitable KERS that power can be saved and stored to use as extra acceleration.

Figure 7- A graph showing the power in J/s.

# 5.Suitable Kinetic Energy Recovery Systems

In order to store the lost energy a KERS must be used in the vehicle.

## 5.1 Electric KERS

An electric KERS is used widely by many car manufactures. Its main use is to capture the braking rotational force by the wheels and stores the energy into the battery while also charging the battery. In a motorsport environment this energy would be deployed through a overtake button which will give the car an extra 160hp through the lap.

Lithium batteries are used in vehicles to store the kinetic energy. While these batteries are able to store large amounts of energy it does require an expensive and heavier battery. These batteries may also not perform to its best ability as they can overheat very quickly, cooling ducts and thermal managing systems will be required for the battery to maintain optimum performance. Lithium batteries are also not meant to have a full storage of energy at one time as this can decrease its performance but also lifetime expectancy (Batalha, no date).

A car manufacture that uses a regenerative braking electric KERS is Tesla. It is estimated that the electric KERS on tesla’s can save up to 70% of its lost kinetic energy from braking. It also states that the regenerative braking system prolongs the brake system’s lifetime (Armstrong, 2023).

# 5. System Operations

## 5.1 How the System Operates

The system operates by where kinetic energy is stored in motion, and then that energy can be used to stop the vehicle gradually (Engineering, 2009).

Figure 8- Electric KERS (Armenta-Dé & Cortés, 2023).

## 5.2 KERS Driving feel vs ICE Driving Feel

When driving a vehicle with a KERS drivers may feel more instant torque and acceleration compared to a combustion engine with a lower top speed. Most top speed KERS vehicles also use a single speed gear which would help with that instant acceleration. Compared to a combustion engine vehicle a KERS in a vehicle with large batteries for storage can feel considerably heavier at the wheel. KERS vehicles have a positive environmental impact as it doesn’t create any pollution unlike ICE vehicles, but also reduces the noise pollution as it doesn’t make a loud noise compared to a v8 engine. When braking with a KERS it may feel weightier, as it needs to regenerate a large amount of power the braking may be less sudden but feel heavier (Etengoff, 2024).

# References

Anon., n.d. Parkers. [Online] 
Available at: https://www.parkers.co.uk/hyundai/i30/hatchback-2017/15t-gdi-n-line-5d/specs/

Armenta-Dé, C. & Cortés, H., 2023. Analysis of Kinetic Energy Recovery Systems in Electric Vehicles. [Online] 
Available at: https://www.mdpi.com/2624-8921/5/2/22

Armstrong, K., 2023. How Tesla's Regenerative Braking Works. [Online] 
Available at: https://www.notateslaapp.com/tesla-reference/1051/how-tesla-s-regenerative-braking-works#:~:text=It's%20estimated%20that%20regenerative%20braking,range%20between%20needing%20to%20charge.

Batalha, J., no date. Kinetic Energy Recovery Systems. [Online] 
Available at: https://cecas.clemson.edu/cvel/auto/AuE835_Projects_2009/batalha_project.html

Carlin, S., 2015. An Analysis of Kinetic Energy Recovery Systems and their potential for contemporary Internal Combustion Engine powered vehicles. [Online] 
Available at: https://sear.unisq.edu.au/29183/1/Carlin_S_Malpress.pdf

Dale, N., 2021. What is WLTP and what does it mean? WLTP Explained. [Online] 
Available at: https://www.evanshalshaw.com/blog/what-is-wltp/?srsltid=AfmBOorAgSysV0LBPhQTPGMdvhqBvnh_a04T2ypsfzVVHi9PcFSttfVr&_its=eF5Vj01ugzAQhe8y6xKwDf7hBl30DGjA42DJwZFxyiIKZ-80XXUzi3mfnr73hO_oYQRFWiMO2HjlRdOb2TeOOtd0dvBKukCCFHzAXrES4yXVUxglbWeFdmfCzZ9ykv0g7bRTnkKwp

Engineering, R., 2009. The basics of F1 KERS. [Online] 
Available at: https://www.racecar-engineering.com/articles/the-basics-of-f1-kers/

Etengoff, A., 2024. How does EV driving experience differ from an ICE vehicle?. [Online] 
Available at: https://www.evengineeringonline.com/how-does-the-ev-driving-experience-differ-from-an-ice-vehicle/