#include <main.h>

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#include <lcd.c>

unsigned int value1;

unsigned int light_l[] = {128,64,32,16,8,4,2,1,1,2,4,8,16,32,64,128,}; // choose which lights light up in what sequence flip values

unsigned int button_display;

unsigned int light_display;

void analogs(void)                                                       //Subroutine for any function

{

set_adc_channel(0);                                               //select channel 0

delay_us(30);                                                     //wait 30us

value1=read_adc();                                              //call conversion function

}

void task1(void)

{

if (input(pin_A2))

{

lcd_gotoxy(1,1);                                         //go to line number y position x on the LCD

printf(lcd_putc,"value1 = %3u", value1);     //print the number of the channel and the conversion value on the LCD

analogs();

output_b(value1);			// display the value on the virtual terminal

printf("value1 = %3u \r \n", value1);	// display the value on the virtual terminal

if (value1 > 175 )		//if the pot1 value is over 175

{

output_high(pin_C0);		//spin the motor clockwise

output_low(pin_C1);		//no output from this pin

lcd_gotoxy(5,2);

printf(lcd_putc,"clockwise");	//display on the screen the direction of the spinning motor

}

else if (value1 < 75)		//if the pot1 value is less than 75

{

output_high(pin_C1);		//spin the motor anticlockwise

output_low(pin_C0);

lcd_gotoxy(2,2);

printf(lcd_putc,"anti  clockwise");	//display on LCD the anticlockwise direction the motor is spinning

}

else				// if neither above 175 or below 75

{

output_low(pin_C1);

output_low(pin_C0);		//motor is stopped

lcd_gotoxy(4,2);

printf(lcd_putc,"stopped  ");	//display stopped on LCD

}

}

}

void task2(void)			//start of task 2 function

{

if (input(pin_a3))		//if there is an input on pin a3 execute what is next

{

output_b(value1 & 0xFF);	//show the RV1 output on the virtual terminal

lcd_gotoxy(1,1);

printf("value1 = %3u \r \n", value1);    //go to line number y position x on the LCD

printf(lcd_putc,"value1 = %3u", value1);	//display the value on the LCD as well

output_low(pin_c1);		//motor is not spinning

output_low(pin_c0);

delay_ms(3000);			//delay to avoid bouncing

output_b(0x00);			//clear virtual terminal at the end

}

}

void task3(void)		//start of task 3

{

if (input(pin_A4))	//if pin a4 has an input execute task3

{

if (input(pin_a1))	//if the pin a1 has an input then continue with task 3

{

printf("\f");		//clear the virtual terminal

printf("on");		//print the buttons status on the virtual terminal

lcd_gotoxy(1,2);

printf(lcd_putc,"on ");	//print the buttons status on the LCD

output_high(pin_C0);	//spin the motor

output_low(pin_C1);

}

else			 //if any other pin has an input or a1 and a4 do not

{

printf("\f");		//clear the LCD

printf("off");		//print off on the virtual termianl

lcd_gotoxy(1,2);

printf(lcd_putc,"off ");//print off on the LCD

output_low(pin_c1);	//motor is not spinning

output_low(pin_c0);

}

}

}

void task4(void) 		//task4 start

{

if (input(pin_A5))	//if a5 has an input execute the task

{

lcd_gotoxy(27,2);		//goto 27,2 on the LCD

printf(lcd_putc,"kitt mode");	//display kitmode on the LCD

delay_ms(300);			//delay to prevent bouncing

if (button_display == 0)	//if the button has not been pressed

{

button_display = 1;		//if the button is pressed

printf("kitt mode \r \n");	//print kitt mode on the virtual termianl

}

for(int i=0; i < 16; i++)	// this means the function will go up in 1 increments from 0-11

{

output_b(light_l[i]);		//going through all of the a pins in increments of 1

delay_ms(100);			//delay to prevent bouncing

}

output_b(0x00);

}

if (button_display == 1)

{

button_display = 0;		//if no button input

printf("\f");			//clear virtual teminal

}

}

void clear_lcd(void)

{

delay_ms(200);

printf(lcd_putc,"\f");	//clear LCD function

}

void main()

{

setup_adc_ports(sAN0);                                 // initialise adc channel

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);          // setup adc operation

lcd_init();                                          // initialise the LCD

while(TRUE)

{

analogs();

task1();

task4();

task3();

task2();

clear_lcd();

}

}

Comments

2.. 20   3.. 30    4.. 40   5.. 10

****************************************

2.. Your code has a number of your own functions and all work, you should always put your subroutines after the main function to follow convention. Your statements indicate what each line does 70%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented.75%