| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 6 | LEFEL 6 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| May 2017 | 2017 |

MODULE CODE: M6x

Plant and Asset Engineering

TIME:  2 HOURS

answer Three quESTION out of four

Equipment or materials required –

Calculators may be used and machine capability paper and average range control chart paper to be provided.

Adnoddau neu ddefnyddiau sydd eu hangen - Dim

Question 1

One of the main reasons for often carrying out maintenance is to improve the capability of a process or a machine.

- Using the data below from a cutting process perform a machine capability study to ascertain whether the machine is capable or not.  Comment on your findings.

(18 marks)

| Sample | Reading | Sample | Reading | Sample | Reading |
| --- | --- | --- | --- | --- | --- |
| 1 | 24.70 | 18 | 24.74 | 35 | 24.68 |
| 2 | 24.68 | 19 | 24.70 | 36 | 24.70 |
| 3 | 24.70 | 20 | 24.76 | 37 | 24.64 |
| 4 | 24.66 | 21 | 24.68 | 38 | 24.70 |
| 5 | 24.72 | 22 | 24.76 | 39 | 24.74 |
| 6 | 24.78 | 23 | 24.68 | 40 | 24.70 |
| 7 | 24.72 | 24 | 24.74 | 41 | 24.68 |
| 8 | 24.68 | 25 | 24.80 | 42 | 24.72 |
| 9 | 24.74 | 26 | 24.78 | 43 | 24.78 |
| 10 | 24.70 | 27 | 24.72 | 44 | 24.66 |
| 11 | 24.66 | 28 | 24.62 | 45 | 24.76 |
| 12 | 24.76 | 29 | 24.74 | 46 | 24.72 |
| 13 | 24.72 | 30 | 24.66 | 47 | 24.70 |
| 14 | 24.68 | 31 | 24.66 | 48 | 24.68 |
| 15 | 24.74 | 32 | 24.74 | 49 | 24.72 |
| 16 | 24.72 | 33 | 24.64 | 50 | 24.70 |
| 17 | 24.70 | 34 | 24.72 |  |  |

Specification 24.69 ± 0.05

- Evaluate what factors should be addressed prior to undertaking a machine capability study.

(7 marks)

Question 2

Reliability centred maintenance (RCM) analysis provides a structured framework for analyzing the functions and potential failures for a physical asset with a focus on preserving system functions, rather than preserving equipment.

- Explain the seven basic questions that the RCM process must address using an example to support your answer.

(14 marks)

- In the context of RCM discuss why it is important to evaluate the effects of failure and to identify the causes of failure.

(11 marks)

Question 3

Lighter Engineering manufacture discs for use in central locking systems.  Every hour a sample is taken and the following results have been obtained on the thickness of the discs in mm.

| Reading | 14.00 | 15.00 | 16.00 | 17.00 | 18.00 | 19.00 | 20.00 | 21.00 | 22.00 | 23.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8.55 | 8.50 | 8.52 | 8.52 | 8.50 | 8.50 | 8.51 | 8.50 | 8.51 | 8.52 |
| 2 | 8.51 | 8.49 | 8.48 | 8.51 | 8.49 | 8.38 | 8.47 | 8.52 | 8.40 | 8.51 |
| 3 | 8.50 | 8.46 | 8.47 | 8.56 | 8.48 | 8.49 | 8.49 | 8.49 | 8.55 | 8.49 |
| 4 | 8.47 | 8.50 | 8.53 | 8.50 | 8.49 | 8.37 | 8.49 | 8.47 | 8.36 | 8.49 |

| Reading | 00.00 | 01.00 |
| --- | --- | --- |
| 1 | 8.48 | 8.52 |
| 2 | 8.46 | 8.49 |
| 3 | 8.51 | 8.51 |
| 4 | 8.52 | 8.52 |

Table 1 Constants for calculating control limits

| Sample size | A2 | D4 | d2 |
| --- | --- | --- | --- |
| 2 | 1.88 | 3.27 | 1.13 |
| 3 | 1.02 | 2.57 | 1.69 |
| 4 | 0.73 | 2.28 | 2.06 |
| 5 | 0.58 | 2.11 | 2.33 |

- Using the data above construct an average/range control chart to demonstrate if the process is in control or not.

(15 marks)

- Comment on your findings from part a)

(4 marks)

- Explain what is meant by common cause and special causes of variation and the significance of each, in relation to overall performance of the processes.

(6 marks)

Question 4

A vast number of processes rely on the efficient combustion processes to be able to operate cost effectively and legally.

- In terms of combustion processes explain what is meant by the following terms: -

- Activation energy								(2 marks)

- Exothermic reaction							(2 marks)

- Enthalpy change								(2 marks)

- An experimental burner is being developed that burns pure carbon in the presence of pure oxygen, in which the combustion conditions can be changed.  The input of carbon is at a rate of 24g/minute using the following data below determine the following:

∆H Carbon dioxide = -394 kJ/mol

∆H Carbon monoxide = -112 kJ/mol

∆H Hydrogen = -286 kJ/mol

- The amount of heat produced when 20% of the carbon burns to carbon monoxide (CO) and remaining 80% burns to carbon dioxide (CO2).

(6 marks)

- After further changes in the experimental system it is found that 70% of the carbon burns to give carbon dioxide (CO2), 20% burns to give carbon monoxide (CO) and 10% of the carbon is unburnt.  What is the current rate of heat production?

(6 marks)

- Calculate the percentage loss of potential heat in (i) and (ii).  Maximum heat release from carbon is 788 KJ/min

(4 marks)

- Comment on your results from (i), (ii) and (iii)

(3 marks)