# Name: Morgan jones

# Student number: 1706863

# Module : (ACDA4004)

# Course: Electric and Electronic Engineering

Contents

Name: Morgan jones	1

Student number: 1706863	1

Module : (ACDA4004)	1

Course: Electric and Electronic Engineering	1

Introduction	2

Section 1:	7

Section 2:	11

Bibliography	14

# Introduction

What is a PIC ?

PIC microcontrollers are an electrical circuit that can be programmed to carry out a variety of tasks. They can be programmed to be clocks or to control a construction line and much more. They are in most electronic devices such as alarm systems, computer control systems, phones, in essence they can be in any electronic device. [1]

[1]

There are many types of pic board, here are some:

Curiosity pic microcontroller Development board

The curiosity program development board is designed by microchip for 8-bit pic microcontrollers. It is cost effective and fully integrated with 8-bit microcontrollers. It is figure is shown below:

Features of Curiosity pic Development board:

- It supports the 40 pin 8-bit microcontroller with low voltage.

- It has USB interface with integrated programmer or debugger.

- It has RN 4020 Bluetooth foot print.

- It has physical switches and analogue potentiometer.

- It has wide interface options such as m touch button.

Ready for PIC microcontroller development Board

Reay of pic is the development board which is used for development of 28 and 40 pins pic microcontrollers such as PIC 18F45k22 by connecting the 8MHz crystal oscillator. It is shown in figure below:

Features of Reay for PIC development Board:

- It is fully supported by mikro c mikro basic and mikro pascal complier for pic compiler.

- It has 10 IDC headers for available microcontroller ports.

- It has USB-UART module, prototyping area and power supply circuit.

- It has fast UART bootloader for on board pic programming.

It is designed on white plastic casing so each component could be sold separately

XLP 8-Bit pic microcontroller Development Board

XLP 8-Bit Development Board is designed by microchip for a very low power and have a very low sleep current such as 20 nA.It is very sustainable for prototype very low power applications such as energy harvesting, temperature sensors ,electronics door locks, smart cards , security sensors. For extensive low power applications, the extensive line of daughter cared is interfaced with this microchip development board. It is very low cost board and is very ideal for providing the development environment to the pic 16F and 18F microcontrollers. This XLP 8-Bit Development Board is shown below.

Microchip XLP 8-Bit Development Board

Features of XLP 8-Bit pic microcontroller Development Board:

- It supports the Pic 18F87k22, which have 80 pins and 128KB flash memory.

- It also supported the microcontroller such as 6LF194PIC17 which 64 pins and 28KB flash memory.

- It also has current and voltage measurements facility.

- It has USB interface for power and PC communication.

- It has LEDs for identifications.

- It has LCD interface facility.

- It has analogue temperature sensor and CTMU based diode temperature sensor.

- It has power options such as energy harvesting, AAA, USB, CR2032, external or 9 V power supply.

- It has very easy connection with MPLAB PICKIT3, ICD3 or RESL ICE for input debugging and programming

XLP 16-Bit pic microcontroller Development Board

XLP 16-Bit Development Board is also designed by microchip and is used for very low power applications. It is very ideal for low power motor control, digital power conversion, advanced integration and low power security systems. This XLP 16-Bit Development Boards shown below.

Features of XLP 16-Bit pic Development Board

- It has 20 and 28 pins shared footprints sockets with associated headers for PIC 24F microcontroller.

- It has 8 MHz and 32.768KHz oscillator circuit for PIC 24F microcontroller.

- It has power LED.

- It has power control switch IC (IC PWR).

- It has adjustable regulator (LDO).

- It has temperature sensor, potentiometer and serial EEPROM.

- It also has capacitive touch pads and capacitive measurements points

XLP 32-Bit pic microcontroller Development Board

XLP 32-Bit Development Board is also designed by microchip, which provides the very lowest and easiest method to experience the 32 bit pic microcontrollers. It was developed by microchip first time. It has all the features such write, debug and execute the program for high performance 32bit microcontroller

Features of XLP 32-Bit pic microcontroller Development Board

- It has free MPLAB IDEX and MPLABXC32 compiler.

- It has 512K flash,32K ram and 4 channel DMA.USB powered board.

- It has 3 users switched and 3 LEDs.

- It has full source based debugging in MPLAB XIDE.

- It run and verify the program in 9 seconds.

Easy PIC V7 8-Bit pic microcontroller Development Board

Easy pic v7 8-bit development board is designed by mikroelektronia development tools company and is designed for experienced the 8-bit microcontroller such as PIC 18F78K22 microcontroller. It is shown in figure below:

Features of Easy PIC V7 8-Bit pic Development Board

- It could be used for variety of applications such as Ethernet, USB, temperature measurements and including graphics.

- It also supports the PIC18F, PIC18FJ, and PIC18FLK.

- It has the special function registers (SFR).

- It has CODE, RAM, and EEPRM along with code execution hardware.

- It also 80 Pins MCU card that could be sold separately.

PIC KICK Development Board with PIC16F877A

PIC kick development board is designed for pic 16F and 18F microcontrollers. It is low coat and kick start development board for pic 16F877A and is used for low power applications. It is shown in figure below

EASY PIC Development Board for PIC18f4520

Easy pic development board for pic 18F4520 is the starter board for 40 and 28 pin microcontrollers. It is low and is only designed for pic it figure is shown below:

Features of EASY PIC Development Board for PIC18f4520:

- It has serial a bootloader with on board microchip 18F4520 microcontroller.

- It is powered up externally or USB port.

- It has on board USB to serial communication converter

- It 12 MHZ replaceable crystal to clock the microcontroller.

- It also has reset push button.

PIC-P26J50  Porto pic Development Board

PIC-P26J56 Porto development board is designed for pic 18F26J56 microcontroller and it has developed by the using the high speed low power CMOS flash memory technology. Its figure is shown below:

Features of PIC-P26J50 Porto Development Board:

- It has 64 K flash with pic microcontroller.

- It has UBS connector with 8 LEDs.

- It has user button and reset button.

- It has UEXT connector.

- It has dimensions 61*45mm.

[2]

what the purpose of MPLAB is, and what are its general features.

MPLAB is produced by Arizona Microchip. MPLAB IDE is a integrated toolset for the development of embedded applications on PIC Microchips and microcontrollers. It is called an Integrated Development Environment because it provides a single integrated environment to develop code for implanted microcontrollers.

Features

MPLAB IDE is a Windows OS based IDE for the PIC MCU families and dsPIC Digital Signal Controllers. The MPLAB IDE provides the ability to create and edit source code via the built-in editor. Assemble, compile and link source code. Debug the executable logic by watching program flow with the built-in simulator or in real time with in-circuit emulators or in-circuit debuggers. Make timing measurements with the simulator or emulator. View variables in Watch windows and Program firmware into devices with device programmers.

[3]

Circuit Designing

Figure 1 diagram of the physical device

The PIC microcontroller transmits as well as receives data with electrical pulses, the PIC microcontroller can operate with a 4MHz crystal. Two capacitors are connected to the crystal oscillator which is used to stabilize the electrical signals. Sometimes the PIC microcontroller goes to a block state or has a missing time calculation, at that time we need to reset the microcontroller. If a microcontroller is reset for with a time delay, the resistor and the capacitor are connected to the respective pins.

Circuit Components

Reset device, A power-on reset generator is a microcontroller that generates a reset signal when power is applied to the device. [4]

Capacitors are used in power supplies where they smooth the output

[5]

Resistors are used to reduce current flow, adjust signal levels, to divide voltages, bias active elements, and terminate transmission lines, among other uses.

[6]

As you can see the diagram shows the physical components on the PIC board there are two button one black  and one green these can be used to control the actions of the sounder or seven segment display.

Buttons [7]

There is a switch which helps control whether the board receives power in essence turn on or off there is also a led connected to the switch which helps verify that there is power going to the components

There is a sounder this is used to create a sound for example if the pic is programmed so that the sounder creates a tune or plays a piece of music

Piezo Buzzer [8]

There is a seven segment led display this is used to show any particular letters or numbers that the pic has been program to show for example “H” “E” “L” “P” or “S” “T” “O” “P”

Seven segment display [9]

There is also the PIC microcontroller which can be connected and disconnected from the board mean it is easier program with a set of instructions as if it was stuck on you would not be able to program the PIC as there will be feasible way or uploading the program code.

[10]

There is crystal oscillator which is used for the resonance of a vibrating crystal of piezoelectric material. It will create an electrical signal with a given frequency.

crystal oscillator

[11] [12]

# Section 1:

Error 1

Error[113]   C:\USERS\ERGO\DOWNLOADS\COUNT.ASM 30 : Symbol not previously defined (status)
Error[113]   C:\USERS\ERGO\DOWNLOADS\COUNT.ASM 31 : Symbol not previously defined (status)
Error[113]   C:\USERS\ERGO\DOWNLOADS\COUNT.ASM 33 : Symbol not previously defined (status)
Error[113]   C:\USERS\ERGO\DOWNLOADS\COUNT.ASM 36 : Symbol not previously defined (status)
As you can see from the above lines there are multiple line with the same error these lines however are not the issue line 17 is as there is a mistype as the word Status should be lowercase not uppercase as it is in the count file an easy way to fix this is go to line 17 and change the word to lowercase letters.

Error 2

Error[113]   C:\USERS\ERGO\DOWNLOADS\COUNT.ASM 57 : Symbol not previously defined (get_count)

The above error is not with line 57 specifically but line 41 as get_counnt should match line 57 however due to a mistype there is a discrepancy of get_count having an extra n in the word count to rectify this just remove the extra n.

Error 3

Warning[202] C:\USERS\ERGO\DOWNLOADS\COUNT.ASM 74 : Argument out of range.  Least significant bits used.

The error for this line is another mistype as the line shows movlw	0xfff

When it should show movlw 0xff as this indicates what is show on the segment board as 0xfff will either make the program not work or cause it to act different to the programmer’s intentions so to fix this issue remove an f making it 0xff

Error 4

Error[113]   C:\USERS\ERGO\DOWNLOADS\COUNT.ASM 75 : Symbol not previously defined (mCont)

The error for this line is that there is a mistype as the word mcont this is not the correct instruction and should really be mcount as it works as the outer loop counter and to correct it you should add a u in between the o and n making the correct instruction

Error 5

Warning[207] C:\USERS\ERGO\DOWNLOADS\COUNT.ASM 89 : Found label after column 1. (en)

The error that the full word should be end instead of en but there was a mistype as there was a gap between the en and d so this can be corrected by removing the gap making the word end instead of en d

Error 6

Error[122]   C:\USERS\ERGO\DOWNLOADS\COUNT.ASM 89 : Illegal opcode (d)
The error that the full word should be end instead of d but there was a mistype as there was a gap between the en and d so this can be corrected by removing the gap making the word end instead of en d

Error 7

Error[129]   C:\USERS\ERGO\DOWNLOADS\COUNT.ASM 98 : Expected (END)

This error is a mistyped word as there should be the word end instead of en d but there was a mistype as there was a gap between the en and d so this can be corrected by removing the gap making the word end instead of en d

Figure 2 proteus schematic of the PIC board

Description of the purpose and general operation of each component in MAPLAB.

The MPLAB IDE possesses both built-in and plug-in components to set up the system for an assortment of software and hardware tools.

Project Manager, the project manager offers integration and communication between the IDE and the language tools.

Editor, the editor is a full-featured programmer's text editor that also serves as a window into the debugger.

Assembler and Language Tools, the assembler can be used stand-alone to assemble a single file or can be used with the linker to build a project from separate source files, libraries and recompiled objects. The linker is responsible for positioning the compiled code into memory areas of the target microcontroller.

Debugger: The Microchip debugger allows breakpoints, single stepping, watch windows and all the features of a modern debugger for the MPLAB IDE. It works in conjunction with the editor to reference information from the target being debugged back to the source code.

Execution Engines: There are software simulators in MPLAB IDE for all PIC devices. These simulators use the PC to simulate the instructions and some peripheral functions of the PIC devices. Optional in-circuit emulators and in-circuit debuggers are also available to test code as it runs in the applications hardware.

[3]

You should perform additional research of your own to in this area and include data/diagrams from your research sources to aid in your explanations.

Note - using internet-based resources is perfectly acceptable for this task, but make sure you reference any sources using the correct numeric format as covered in the Academic and Employability Skills module.

# Section 2:

- Modify “Count.asm” so the seven segment display shows “H” “E” “L” “P” at a rate that is easy to read.

Figure 3 flowchart design for count.asm

The flowchart shows that the when the power switch is turned on the PIC will light the led with the first letter H then it will wait 1 second then proceed to the next letter E then it will wait 1 second then proceed to the next letter P then it will wait 1 second then proceed to the next letter P then it will wait 1 second then proceed to clear the led display and then restart the program

Modify “Count.asm” so that the piezo sounder produces a 440Hz tone while the Green button is pressed.

Figure 4

The flowchart shows that the when the power switch is turned on you can press the green button as the sounder produces a 440 hz tone then you release the green button then the program will reset so that you can repress the green button and the activate the sounder    .

Modify “Count.asm” so that the piezo sounder produces a low frequency tone while the Green button is pressed and a high frequency tone while the Black button is pressed.
figure 5

The flowchart shows that the when the power switch is turned you will have to decide which button to press if you press the green button the sounder will produce a low frequency tone once the button is released the program will restart and if you press the black button the sounder will produce a high frequency tone once the button is released the program will restart.

# Bibliography

| [1] | V. Ryan, " WHAT IS A PIC MICROCONTROLLER? WHAT CAN IT DO?," [Online]. Available: http://www.technologystudent.com/pics/picgen1.html. [Accessed 1 january 2019]. |
| --- | --- |
| [2] | "Pic Microcontroller Development Boards with Their Features," [Online]. Available: http://microcontrollerslab.com/pic-microcontroller-development-boards-features/. [Accessed 21 january 2019]. |
| [3] | A. Tomar, "Microchip: MPLAB Integrated Development Environment (IDE) Overview," [Online]. Available: https://www.element14.com/community/docs/DOC-39318/l/microchip-mplab-integrated-development-environment-ide-overview#anchor1. [Accessed 1 January 2019]. |
| [4] | "Power-on reset," [Online]. Available: https://en.wikipedia.org/wiki/Power-on_reset. [Accessed 11 January 2019]. |
| [5] | "Applications of capacitors," [Online]. Available: https://en.m.wikipedia.org/wiki/Applications_of_capacitors. [Accessed 12 january 2019]. |
| [6] | "Resistors," [Online]. Available: https://en.m.wikipedia.org/wiki/Resistor. [Accessed 12 january 2019]. |
| [7] | "Aliexpress buttons," [Online]. Available: http://www.aliexpress.com/i/32851421040.html. [Accessed 11 january 2019]. |
| [8] | "robomart.com piezo-buzzer-b-10n," [Online]. Available: https://www.robomart.com/piezo-buzzer-b-10n. [Accessed 11 january 2019]. |
| [9] | "Seven-segment display," [Online]. Available: https://en.wikipedia.org/wiki/Seven-segment_display. [Accessed 1 January 2019]. |
| [10] | "PIC microcontrollers," [Online]. Available: https://en.m.wikipedia.org/wiki/PIC_microcontrollers. [Accessed 13 january 2019]. |
| [11] | "Crystal Oscillator Circuit and Working," [Online]. Available: https://www.elprocus.com/crystal-oscillator-circuit-and-working/. [Accessed 1 january 2019]. |
| [12] | "How to Program a PIC Microcontroller to Build a Project," [Online]. Available: https://www.elprocus.com/pic-microcontroller-programming-using-c-language/. [Accessed 1 january 2019]. |

Appendices

Count.asm corrections

MPASM  5.51             COUNT CORRECTIONS.ASM   1-31-2019  9:53:36         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; FILE:                 count.asm

00002 ; AUTHOR:               JMY

00003 ; DATE:                 24th March 2005

00004 ; DESCRIPTION:  Count to 255 at 1 second intervals and display on a seven

00005 ;                               segment display in binary.

00006 ;                               The pause subroutine takes apx. 0.2 sec.

00007 ;                               Pause is called 5 times

00008

Warning[215]: Processor superseded by command line.  Verify processor symbol.

00009         list    p=16F84         ; select processor type

00010         radix   hex                     ; define radix as hexadecimal

00011 ;       #include "p16f84.inc"

00012 ;----------------------------------------------------------------------

00013 ;       cpu equates (memory map)

00000006            00014 portB   equ     0x06            ;port B address

00000006            00015 trisb   equ 0x06                ;trisB address in register bank 1

00016

00000003            00017 status  equ 0x03                ;status register lower case s no captital

00000005            00018 RP0             equ 5                   ;bit 5 of status register

00000006            00019 RP1     equ     6                       ;bit 6 of status register

00020

0000000C            00021 count   equ     0x0c            ; counter 0 to 255

0000000D            00022 nCount  equ     0x0d            ; pause routine inner loop counter

0000000E            00023 mCount  equ     0x0e            ; pause routine outer loop counter

00024

00025 ;----------------------------------------------------------------------

00026

000000                00027         org     0x000

00028

000000                00029 start

000000 9A03           00030         bcf             status,RP0      ; clear bank selection bits in the status reg

000002 9C03           00031         bcf             status,RP1

00032

000004 8A03           00033         bsf             status,RP0      ; select bank 1

000006 0E00           00034         movlw   0x00            ; load w with 0x00

000008 6E06           00035         movwf   trisb           ; load it into trisb to make portb all outputs

00000A 9A03           00036         bcf             status,RP0      ; reselect bank 0

00037

00000C 6A06           00038         clrf    portB           ; clear all lines low

00000E 6A0C           00039         clrf    count           ; initialize counter to 0

00040

000010                00041 get_count

000010 500C           00042         movf    count, w        ; move count to W

00043

00044

000012 6E06           00045         movwf   portB           ; move W to port B to update the display

00046

000014 0F01           00047         addlw   1                       ; add +1 to W

00048

000016 6E0C           00049         movwf   count           ; save W in count

00050

000018 EC18 F000      00051         call    pause           ; delay 200ms by subroutine

00001C EC18 F000      00052         call    pause

MPASM  5.51             COUNT CORRECTIONS.ASM   1-31-2019  9:53:36         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

000020 EC18 F000      00053         call    pause

000024 EC18 F000      00054         call    pause

000028 EC18 F000      00055         call    pause           ; five pause executions equals ~ 1 second

00056

00002C EF08 F000      00057         goto    get_count               ; repeat forever

00058

00059 ;--------------------------------------------------------------------

00060 ;subroutine pause: lets waste some time....

00061 ;

00062 ;inputs: nothing

00063 ;outputs: nothing

00064 ;

00065 ;does: nothing but waste some time

00066 ;corrupts: register W

00067 ;

00068 ;description: two loops using "mCount" and "nCount" register files

00069 ;                         "mCount" is the outer loop counter (255 down to zero)

00070 ;                         "nCount" is the inner loop counter (255 down to zero)

00071 ;

00072 ;--------------------------------------------------------------------

000030                00073 pause

000030 0EFF           00074         movlw   0xff            ; set w = 255 decimal to f no three

000032 6E0E           00075         movwf   mCount          ; mCount = w: outer loop counter count not cont

000034                00076 loadN

000034 0EFF           00077         movlw   0xff            ; set w = 255 decimal

000036 6E0D           00078         movwf   nCount          ; nCount = w: inner loop counter

000038                00079 decN

000038 2E0D           00080         decfsz  nCount, f       ; nCount--

00003A EF1C F000      00081         goto    decN            ; if nCount != 0 then repeat nCount--

00082

00003E 2E0E           00083         decfsz  mCount, f       ; else decrement Count

000040 EF1A F000      00084         goto    loadN           ; if mCount != 0 then

00085                                                 ; reload nCount to 255 and decrement

000044 0012           00086         return                          ; else exit subroutine

00087

00088

00089         end; no space between en and d

MPASM  5.51             COUNT CORRECTIONS.ASM   1-31-2019  9:53:36         PAGE  3

SYMBOL TABLE

LABEL                             VALUE

RP0                               00000005

RP1                               00000006

__18F452                          00000001

count                             0000000C

decN                              00000038

get_count                         00000010

loadN                             00000034

mCount                            0000000E

nCount                            0000000D

pause                             00000030

portB                             00000006

start                             00000000

status                            00000003

trisb                             00000006

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX

0040 : XXXXXX---------- ---------------- ---------------- ----------------

All other memory blocks unused.

Program Memory Bytes Used:    70

Program Memory Bytes Free: 32698

Errors   :     0

Warnings :     1 reported,     0 suppressed

Messages :     0 reported,     0 suppressed

Modify “Count.asm” so the seven segment display shows “H” “E” “L” “P” at a rate that is easy to read.

MPASM  5.51                          HELP.ASM   1-8-2019  17:10:20         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; FILE:                 count.asm

00002 ; AUTHOR:               JMY

00003 ; DATE:                 24th March 2005

00004 ; DESCRIPTION:  Count to 255 at 1 second intervals and display on a seven

00005 ;                               segment display in binary.

00006 ;                               The pause subroutine takes apx. 0.2 sec.

00007 ;                               Pause is called 5 times

00008

00009         list    p=16F84         ; select processor type

00010         radix   hex                     ; define radix as hexadecimal

00011 ;       #include "p16f84.inc"

00012 ;----------------------------------------------------------------------

00013 ;       cpu equates (memory map)

00000006            00014 portB   equ     0x06            ;port B address

00000006            00015 trisb   equ 0x06                ;trisB address in register bank 1

00016

00000003            00017 Status  equ 0x03                ;status register

00000005            00018 RP0             equ 5                   ;bit 5 of status register

00000006            00019 RP1     equ     6                       ;bit 6 of status register

00020

00021

0000000D            00022 nCount  equ     0x0d            ; pause routine inner loop counter

0000000E            00023 mCount  equ     0x0e            ; pause routine outer loop counter

00024

00025 ;----------------------------------------------------------------------

00026

0000                  00027         org     0x000

00028

0000                  00029 start

0000   1283           00030         bcf             Status,RP0      ; clear bank selection bits in the status reg

0001   1303           00031         bcf             Status,RP1

00032

0002   1683           00033         bsf             Status,RP0      ; select bank 1

0003   3000           00034         movlw   0x00            ; load w with 0x00

0004   0086           00035         movwf   trisb           ; load it into trisb to make portb all outputs

0005   1283           00036         bcf             Status,RP0      ; reselect bank 0

00037

0006   0186           00038         clrf    portB           ; clear all lines low

00039

0007                  00040 write_stop

00041

0007   3009           00042     movlw 0x09  ;load LED states for letter 'H' into W reg

0008   0086           00043     movwf portB ; put W reg onto Port B

0009   2020           00044     call delay1s

00045

000A   3006           00046     movlw 0x06  ;load LED states for letter 'e' into W reg

000B   0086           00047     movwf portB

000C   2020           00048     call delay1s

00049

000D   3047           00050     movlw 0x47 ;load LED states for letter 'l' into W reg

000E   0086           00051     movwf portB

000F   2020           00052     call delay1s

00053

MPASM  5.51                          HELP.ASM   1-8-2019  17:10:20         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

0010   300C           00054     movlw 0x0c  ;load LED states for letter 'P' into W reg

0011   0086           00055     movwf portB

0012   2020           00056     call delay1s

00057

0013   307F           00058     movlw 0x7f  ;load LED states for letter ' ' into W reg

0014   0086           00059     movwf portB

0015   2020           00060     call delay1s

00061

00062

0016   2807           00063         goto    write_stop              ; repeat forever

00064

00065 ;--------------------------------------------------------------------

00066 ;subroutine pause: lets waste some time....

00067 ;

00068 ;inputs: nothing

00069 ;outputs: nothing

00070 ;

00071 ;does: nothing but waste some time

00072 ;corrupts: register W

00073 ;

00074 ;description: two loops using "mCount" and "nCount" register files

00075 ;                         "mCount" is the outer loop counter (255 down to zero)

00076 ;                         "nCount" is the inner loop counter (255 down to zero)

00077 ;

00078 ;--------------------------------------------------------------------

0017                  00079 pause

0017   30FF           00080     movlw       0xff ; set w = 255 decimal

0018   008E           00081     movwf       mCount ;mcount = w outer loop counter

0019                  00082 loadN

0019   30FF           00083     movlw       0xff ; set w  = inner loop counter

001A   008D           00084     movwf       nCount ; nCount != 0 then

001B                  00085 decN

00086

001B   0B8D           00087     decfsz      nCount,f

001C   281B           00088     goto        decN

00089

001D   0B8E           00090     decfsz      mCount,f

001E   2819           00091     goto        loadN

00092

001F   0008           00093         return

00094

00095

0020                  00096 delay1s

00097

0020   2017           00098     call  pause ; delay 200ms by subroutine

0021   2017           00099     call  pause

0022   2017           00100     call  pause

0023   2017           00101     call  pause

0024   2017           00102     call  pause; five pause executions equals ~ 1 second

00103

0025   0008           00104     return

00105

00106

MPASM  5.51                          HELP.ASM   1-8-2019  17:10:20         PAGE  3

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00107         end

MPASM  5.51                          HELP.ASM   1-8-2019  17:10:20         PAGE  4

SYMBOL TABLE

LABEL                             VALUE

RP0                               00000005

RP1                               00000006

Status                            00000003

__16F84                           00000001

decN                              0000001B

delay1s                           00000020

loadN                             00000019

mCount                            0000000E

nCount                            0000000D

pause                             00000017

portB                             00000006

start                             00000000

trisb                             00000006

write_stop                        00000007

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXX---------- ----------------

All other memory blocks unused.

Program Memory Words Used:    38

Program Memory Words Free:   986

Errors   :     0

Warnings :     0 reported,     0 suppressed

Messages :     0 reported,     0 suppressed

Modify “Count.asm” so that the piezo sounder produces a 440Hz tone while the Green button is pressed.

MPASM  5.51                        440 HZ.ASM   1-30-2019  15:50:53         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; ; FILE:               count.asm

00002 ; AUTHOR:               JMY

00003 ; DATE:                 24th March 2005

00004 ; DESCRIPTION:  Count to 255 at 1 second intervals and display on a seven

00005 ;                               segment display in binary.

00006 ;                               The pause subroutine takes apx. 0.2 sec.

00007 ;                               Pause is called 5 times

00008 ; AUTHOR:               MJGJ

00009 ; DATE:                 November 2018

Warning[215]: Processor superseded by command line.  Verify processor symbol.

00010         list    p=16C84         ; select processor type

00011         radix   hex                     ; define radix as hexadecimal

00012 ;       #include "p16f84.inc"

00013

00014 ;----------------------------------------------------------------------

00015 ;       cpu equates (memory map)

00016

00017

00000005            00018 portA   equ 0x05             ; port A address

00000005            00019 trisa   equ 0x05            ; trisA address in register bank 1

00020

00000006            00021 portB   equ     0x06    ;port B address

00000006            00022 trisb   equ 0x06                         ;trisB address in register bank 1

00023

00000003            00024 Status  equ 0x03                ;status register

00000005            00025 RP0             equ 5           ;bit 5 of status register

00000006            00026 RP1     equ     6               ;bit 6 of status register

00027

00000000            00028 black   equ 0             ; black button conncets to bit 0 port A (count up)

00000001            00029 green  equ 1             ; green button connects to bit 0 port A (count down)

00030

00031

0000000D            00032 nCount  equ     0x0d            ; pause routine inner loop counter

0000000E            00033 mCount  equ     0x0e            ; pause routine outer loop counter

00034

00035

00036 ;----------------------------------------------------------------------

00037

000000                00038         org     0x000

00039

000000                00040 start

000000 9A03           00041         bcf             Status,RP0      ; clear bank selection bits in the status reg

000002 9C03           00042         bcf             Status,RP1

00043

000004 8A03           00044         bsf             Status,RP0      ; select bank 1

00045

000006 0E00           00046         movlw   0x00            ; load w with 0x00

000008 6E06           00047         movwf   trisb           ; load it into trisb to make portb all outputs

00048

00000A 0E03           00049         movlw 0x03

00000C 6E05           00050         movwf     trisa     ; setting RA0 and RA1 as inputs

00000E 9A03           00051         bcf             Status,RP0      ; reselect bank 0

00052

MPASM  5.51                        440 HZ.ASM   1-30-2019  15:50:53         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

000010 6A06           00053         clrf    portB           ; clear all lines low

00054

00055

00056

000012                00057 loop

00058

000012 A205           00059      btfss portA,green   ; is green button pressed

000014 EC0E F000      00060      call sounder

00061

00062

000018 EF09 F000      00063         goto    loop            ; repeat forever

Warning[207]: Found label after column 1. (sounder)

00001C                00064  sounder

00001C 8E06           00065     bsf   portB,7       ; turn on sounder

Warning[203]: Found opcode in column 1. (call)

00001E EC15 F000      00066 call pause

000022 9E06           00067     bcf   portB,7       ; turn off sounder

Warning[203]: Found opcode in column 1. (call)

000024 EC15 F000      00068 call pause

Warning[203]: Found opcode in column 1. (return)

000028 0012           00069 return

00070

00071 ;--------------------------------------------------------------------

00072 ;subroutine pause: lets waste some time....

00073 ;

00074 ;inputs: nothing

00075 ;outputs: nothing

00076 ;

00077 ;does: nothing but waste some time

00078 ;corrupts: register W

00079 ;

00080 ;description: two loops using "mCount" and "nCount" register files

00081 ;                         "mCount" is the outer loop counter (255 down to zero)

00082 ;                         "nCount" is the inner loop counter (255 down to zero)

00083 ;

00084 ;--------------------------------------------------------------------

00002A                00085 pause

00002A 0E13           00086     movlw       0x13 ; set w = 19 decimal

00002C 6E0E           00087     movwf       mCount ;mcount = w outer loop counter

00002E                00088 loadN

00002E 0EFF           00089     movlw       0xff ; set w  = inner loop counter

000030 6E0D           00090     movwf       nCount ; nCount != 0 then

000032                00091 decN

00092

000032 2E0D           00093     decfsz      nCount,f

000034 EF19 F000      00094     goto        decN

00095

000038 2E0E           00096     decfsz      mCount,f

00003A EF17 F000      00097     goto        loadN

00098

00003E 0012           00099         return

00100

00101

MPASM  5.51                        440 HZ.ASM   1-30-2019  15:50:53         PAGE  3

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

000040                00102 delay1s

00103

000040 EC15 F000      00104     call  pause  ; delay 200ms by subroutine

000044 EC15 F000      00105     call  pause

000048 EC15 F000      00106     call  pause

00004C EC15 F000      00107     call  pause

000050 EC15 F000      00108     call  pause  ; five pause executions equals ~ 1 second

00109

000054 0012           00110     return

00111

00112

00113         end

MPASM  5.51                        440 HZ.ASM   1-30-2019  15:50:53         PAGE  4

SYMBOL TABLE

LABEL                             VALUE

RP0                               00000005

RP1                               00000006

Status                            00000003

__18F452                          00000001

black                             00000000

decN                              00000032

delay1s                           00000040

green                             00000001

loadN                             0000002E

loop                              00000012

mCount                            0000000E

nCount                            0000000D

pause                             0000002A

portA                             00000005

portB                             00000006

sounder                           0000001C

start                             00000000

trisa                             00000005

trisb                             00000006

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX

0040 : XXXXXXXXXXXXXXXX XXXXXX---------- ---------------- ----------------

All other memory blocks unused.

Program Memory Bytes Used:    86

Program Memory Bytes Free: 32682

Errors   :     0

Warnings :     5 reported,     0 suppressed

Messages :     0 reported,     0 suppressed

Modify “Count.asm” so that the piezo sounder produces a low frequency tone while the Green button is pressed and a high frequency tone while the Black button is pressed.

MPASM  5.51            LOW HIGH FREQUENCY.ASM   1-8-2019  17:40:54         PAGE  1

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00001 ; ; FILE:               count.asm

00002 ; AUTHOR:               JMY

00003 ; DATE:                 24th March 2005

00004 ; DESCRIPTION:  Count to 255 at 1 second intervals and display on a seven

00005 ;                               segment display in binary.

00006 ;                               The pause subroutine takes apx. 0.2 sec.

00007 ;                               Pause is called 5 times

00008 ; AUTHOR:               MJGJ

00009 ; DATE:                 November 2018

Warning[215]: Processor superseded by command line.  Verify processor symbol.

00010         list    p=16C84         ; select processor type

00011         radix   hex                     ; define radix as hexadecimal

00012 ;       #include "p16f84.inc"

00013

00014 ;----------------------------------------------------------------------

00015 ;       cpu equates (memory map)

00016

00017

00000005            00018 portA   equ 0x05             ; port A address

00000005            00019 trisa   equ 0x05            ; trisA address in register bank 1

00020

00000006            00021 portB   equ     0x06    ;port B address

00000006            00022 trisb   equ 0x06                         ;trisB address in register bank 1

00023

00000003            00024 Status  equ 0x03                ;status register

00000005            00025 RP0             equ 5           ;bit 5 of status register

00000006            00026 RP1     equ     6               ;bit 6 of status register

00027

00000000            00028 black   equ 0             ; black button conncets to bit 0 port A (count up)

00000001            00029 green  equ 1             ; green button connects to bit 0 port A (count down)

00030

00031

0000000D            00032 nCount  equ     0x0d            ; pause routine inner loop counter

0000000E            00033 mCount  equ     0x0e            ; pause routine outer loop counter

00034

00035

00036 ;----------------------------------------------------------------------

00037

0000                  00038         org     0x000

00039

0000                  00040 start

0000   1283           00041         bcf             Status,RP0      ; clear bank selection bits in the status reg

0001   1303           00042         bcf             Status,RP1

00043

0002   1683           00044         bsf             Status,RP0      ; select bank 1

00045

0003   3000           00046         movlw   0x00            ; load w with 0x00

0004   0086           00047         movwf   trisb           ; load it into trisb to make portb all outputs

00048

0005   3003           00049         movlw 0x03

0006   0085           00050         movwf     trisa     ; setting RA0 and RA1 as inputs

0007   1283           00051         bcf             Status,RP0      ; reselect bank 0

00052

MPASM  5.51            LOW HIGH FREQUENCY.ASM   1-8-2019  17:40:54         PAGE  2

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

0008   0186           00053         clrf    portB           ; clear all lines low

00054

00055

00056

0009                  00057 loop

0009   1C05           00058      btfss portA,black   ; is black button pressed

000A   200E           00059      call sounder

00060

000B   1C85           00061      btfss portA,green   ; is green button pressed

000C   200E           00062      call sounder

00063

00064

000D   2809           00065         goto    loop            ; repeat forever

Warning[207]: Found label after column 1. (sounder)

000E                  00066  sounder

000E   1786           00067     bsf   portB,7       ; turn on sounder

Warning[203]: Found opcode in column 1. (call)

000F   2013           00068 call pause

0010   1386           00069     bcf   portB,7       ; turn off sounder

Warning[203]: Found opcode in column 1. (call)

0011   2013           00070 call pause

Warning[203]: Found opcode in column 1. (return)

0012   0008           00071 return

00072

00073 ;--------------------------------------------------------------------

00074 ;subroutine pause: lets waste some time....

00075 ;

00076 ;inputs: nothing

00077 ;outputs: nothing

00078 ;

00079 ;does: nothing but waste some time

00080 ;corrupts: register W

00081 ;

00082 ;description: two loops using "mCount" and "nCount" register files

00083 ;                         "mCount" is the outer loop counter (255 down to zero)

00084 ;                         "nCount" is the inner loop counter (255 down to zero)

00085 ;

00086 ;--------------------------------------------------------------------

0013                  00087 pause

0013   3013           00088     movlw       0x13 ; set w = 19 decimal

0014   008E           00089     movwf       mCount ;mcount = w outer loop counter

0015                  00090 loadN

0015   30FF           00091     movlw       0xff ; set w  = inner loop counter

0016   008D           00092     movwf       nCount ; nCount != 0 then

0017                  00093 decN

00094

0017   0B8D           00095     decfsz      nCount,f

0018   2817           00096     goto        decN

00097

0019   0B8E           00098     decfsz      mCount,f

001A   2815           00099     goto        loadN

00100

001B   0008           00101         return

MPASM  5.51            LOW HIGH FREQUENCY.ASM   1-8-2019  17:40:54         PAGE  3

LOC  OBJECT CODE     LINE SOURCE TEXT

VALUE

00102

00103

001C                  00104 delay1s

00105

001C   2013           00106     call  pause  ; delay 200ms by subroutine

001D   2013           00107     call  pause

001E   2013           00108     call  pause

001F   2013           00109     call  pause

0020   2013           00110     call  pause  ; five pause executions equals ~ 1 second

00111

0021   0008           00112     return

00113

00114

00115         end

MPASM  5.51            LOW HIGH FREQUENCY.ASM   1-8-2019  17:40:54         PAGE  4

SYMBOL TABLE

LABEL                             VALUE

RP0                               00000005

RP1                               00000006

Status                            00000003

__16F84                           00000001

black                             00000000

decN                              00000017

delay1s                           0000001C

green                             00000001

loadN                             00000015

loop                              00000009

mCount                            0000000E

nCount                            0000000D

pause                             00000013

portA                             00000005

portB                             00000006

sounder                           0000000E

start                             00000000

trisa                             00000005

trisb                             00000006

MEMORY USAGE MAP ('X' = Used,  '-' = Unused)

0000 : XXXXXXXXXXXXXXXX XXXXXXXXXXXXXXXX XX-------------- ----------------

All other memory blocks unused.

Program Memory Words Used:    34

Program Memory Words Free:   990

Errors   :     0

Warnings :     5 reported,     0 suppressed

Messages :     0 reported,     0 suppressed