# Vehicle Dynamics Analysis

Tamzin Cooper

2300764

BEng Motorsport Engineering

07/12/2025

ACHA6011 Advanced Motorsport Dynamics

Tim Tudor

Word Count: 3284

Contents Page

Chapter 1 - Introduction	4

1.1 Aims and Objectives	4

Chapter 2 – Methodology	5

2.1 Simulink Model	5

2.2 Four degree of freedom model	5

2.2.1 Equation Derivations	5

2.2.2 Equations of Motion (4 DOF)	7

2.2.3 Aerodynamic Incorporation	7

2.2.4 Variable Damping Input	9

2.3 VI Grade Model	11

Chapter 3 – Damper Changes	13

3.1 Bump Input	13

3.2 Straight-Line Acceleration	13

3.2.1 Preload Impact	14

3.2 Pembrey Analysis	15

Chapter 4 – Spring Changes	17

4.1 Spring change Simulink model comparison	18

Chapter 6 – 7 DOF Model	19

6.1 Validation against 4-DOF	19

6.2 Roll movement	20

6.3 Lateral Load Transfer	20

Chapter 7 – Conclusion	22

7.1 Model to model comparison and limitations	22

7.2 Improvements	22

Reference List	23

Appendix	24

Figure Table

Figure 1 - Comparing heave displacement between one and two DOF Simulink models.	5

Figure 2 - A diagram depicting a 4 degree of freedom sprung mass damper system. (Tudor, 2020)	6

Figure 3 - Rear Tyre Forces comparison in straight line acceleration with and without aerodynamic loads.	8

Figure 4 - A comparison in heave with and without aerodynamic load.	8

Figure 5 - Comparing damper forces with variable and constant damping.	9

Figure 6 - Front damper velocity comparisons between variable and constant dampers in Simulink.	10

Figure 7 - Comparing front tyre load with constant vs variable damping against VI grade.	11

Figure 8 - Heave displacement using a bump input in Simulink to analyse differing damping set ups.	13

Figure 9 - Comparing pitch velocity between differing damper set ups.	14

Figure 10 - Pitch comparison with and without preload applied to springs.	15

Figure 11 - Heave velocity comparison around Pembrey changing damper stiffness.	15

Figure 12 - Comparing front tyre loads around Brooklands Hairpin.	16

Figure 13 - Section view of front tyre loads during Hatches Hairpin.	17

Figure 14 - Pembrey comparison of pitch angle between Simulink and VI grade (baseline vs soft spring set up).	18

Figure 15 - Comparing rear tyre loads between 4DOF, 7DOF and VI grade model.	19

Figure 16 - Comparing roll angle between 7-DOF model and VI grade at differing spring set-ups.	20

# Chapter 1 - Introduction

This report provides a comparison of analytical and simulated vehicle models to investigate dynamic behaviour. VI Grade and Simulink are demonstrated to analyse suspension and ride performance in response to set up changes. The report homes in on how spring stiffness and damping variations influence the vehicle’s longitudinal weight transfer and overall performance.

## 1.1 Aims and Objectives

1.1.1 To create a full car Simulink model, validated by one, two and four degrees of freedom (DOF) models to compare to the VI grade simulations.

1.1.2 Investigate spring and damper changes on the dynamic behaviour of the vehicle simulated around Pembrey circuit.

1.1.3 Assess the performance and handling of the vehicle in response to dynamic manoeuvres including straight line acceleration, steady-state cornering, and slalom tests.

# Chapter 2 – Methodology

## 2.1 Simulink Model

To begin, simplified one and two degrees of freedom sprung mass damper system models were created in Simulink to validate the initial equation derivations as shown in figure A1 and A2 in the appendix. The one degree of freedom model only incorporates the sprung mass of the system, whereas the two degrees of freedom develops this with the addition of unsprung mass and its subsequent vertical movement as demonstrated by Albinsson (2013).

Figure 1 - Comparing heave displacement between one and two DOF Simulink models.

Simple bump inputs were incorporated (as shown in the appendix) to verify the vertical displacement and derived heave velocity and acceleration. Figure 1 displays the heave motion of both the one and two degrees of freedom systems to validate both systems derive heave correctly.

## 2.2 Four degree of freedom model

### 2.2.1 Equation Derivations

Moving further, in the four degrees of freedom sprung mass damper system, the sprung mass moves in the vertical direction (Z), to represent heave, as well as the ability to create a pitch angle (θ). The front and rear unsprung masses also move vertically (Zf and Zr).  The upwards movement has been defined as the positive direction as shown in figure 2.

Figure 2 - A diagram depicting a 4 degree of freedom sprung mass damper system. (Tudor, 2020)

Table 1 - Table of extensions.

|  | Extension (x) |
| --- | --- |
| Ksf | Z - Zf - a θ |
| Ksr | Z - Zr + b θ |
| Kuf | Zf - yf |
| Kur | Zr - yr |
| Cf | Z -Zf - a θ |
| Cr | Z - Zr + b θ |

Table 1 displays the calculation of springs and dampers extending in relation to the sprung mass, unsprung mass and the ground (yf and yr) to represent bump. The extension is required to determine the force of all spring’s using equation 1, to then later use Newton’s second law of motion (equation 2). The addition of b θ in the rear spring and damper extension is due to the rotation of the mass by the angle of ϑ over the lever arm length of ‘b’. This is added due to moving the rear sprung mass in its positive direction (upwards). The opposite applies to the front, the lever arm length of ‘a’ is multiplied by the angle of pitch rotation but is subtracted, due to the front compressing and moving downwards which is the opposite of the defined direction of movement.

F=Kx

Equation 1

F=ma

Equation 2

### 2.2.2 Equations of Motion (4 DOF)

To find the 4 DOF motion of the vehicle, Newton’s second law of motion can be applied using equation 2 and rearranged to find the acceleration of the movement and therefore can be integrated to find the velocity and integrated once further to find the displacement.

- MsZ=Ksf(Z-Zf-aθ)+Ksr(Z-Zr+bθ)+Cf Z- Zf-aθ+CrZ- Zr+bθ

Equation 3 – Heave derivation (sprung mass vertical motion)

- Iθ=bKrZ-Zr+bθ-aKfZ-Zf-aθ+bCr Z- Zr+bθ-aCf(Z- Zf-aθ)

Equation 4 – Pitch derivation

- MfZf=KtfZf-yf-KfZ-Zf-aθ+CtfZf-yf-Cf(Z-Zf-aθ)

Equation 5- Front unsprung mass vertical motion derivation

- MrZr=KtrZr-yr-KrZ-Zr+bθ+CtrZr-yr-CrZ-Zr+bθ

Equation 6 – Rear unsprung mass vertical motion derivation

### 2.2.3 Aerodynamic Incorporation

To further improve the Simulink model, the incorporation of aerodynamic forces must be applied. Initially, a constant aerodynamic force is applied using equation 7. This is applied to the sprung mass of the model and no assumed front and rear distribution.

Faero=0.5×ρ ×A ×Cl ×v2

Equation 7 – Aerodynamic force

Further development was completed by incorporating variable aerodynamic loads to the model using a signal editor. Front and rear downforce is added and divided by two to represent the half car model. The addition of aerodynamic loads was validated using a straight-line acceleration test.

Figure 3 - Rear Tyre Forces comparison in straight line acceleration with and without aerodynamic loads.

Figure 3 shows the impact of applying the aerodynamic load extracted from the VI Grade sports car model. The validation of working variable aero can be identified between 3-10s whereby as the velocity increases after the gear change, the aerodynamic load is being applied downward and increasing with speed. (Sergers, 2014) This explains the divergence seen between no aerodynamic load and VI grade.

Figure 4 - A comparison in heave with and without aerodynamic load.

Figure 4 further validates working variable aerodynamic load using heave. As speed increase and downforce increases, the car runs closer to the ground due to increased load being applied in the negative direction as defined in Simulink. Therefore, explaining the trend continuing below the zeroed offset heave. The test with no aerodynamic load decreases at a considerably lower gradient due to no additional load being applied, showing the inaccuracies when comparing to VI grade.

It must be noted that the aerodynamic load is being applied directly on the centre of gravity in the sprung mass and not split front to rear. In the VI grade model, the downforce is distributed front to rear with a centre of pressure of 40% frontward and so effectively will create a small aerodynamic-induced negative pitch on the car which is not accounted for in Simulink. This is demonstrated in figure 3, whereby the offset in front tyre loads is due to the reduced unloading in Simulink as there is no pitch offset and so there is no initial loading on the rear tyres.

### 2.2.4 Variable Damping Input

#### Bump Validation

VI Grade uses variable damping, whereby, the damping coefficient is selected using the differing damping velocities represented in the curve. To develop the Simulink model, a 1D look up table (MathWorks, 2025) was incorporated to create the damping curve and produce variable damping for bump and rebound.

Figure 5 - Comparing damper forces with variable and constant damping.

From figure 5, the differing damper forces between variable damping and constant damping within the Simulink model are displayed. The graph shows during rebound, the variable damper force is higher than the constant, whereas during bump, the constant damper is overshooting the damper force. This is expected and validates the use of the variable damper as the constant damping coefficient was assumed 10,000. From figure 6 below, the damper only reaches approximately 0.02 m/s, which will show an interpolated damper coefficient between the 0 m/s and 0.052 m/s (13,919 damping coefficient) for the variable damper (according to A10 in the appendix). Therefore, the constant damper underpredicts the damper force in rebound compared to variable damping. However, in bump, the constant 10,000 coefficient sits around -0.131 m/s mark, where figure 4 shows the damper is reaching approximately -0.021s (approximately 8574 damping coefficient) and so overpredicting the damper force compared to the variable damping.

Figure 6 - Front damper velocity comparisons between variable and constant dampers in Simulink.

When correlating figure 6 above and figure A10 in the appendix, it is seen that the damper is only ever in low-speed velocity and so only using the interpolation between 0-0.052s for bump and rebound. This is due to the small bump input being analysed. This explains the lack of deviance in figure 5, due to the dampers not reaching the high-speed velocities, and therefore, not demonstrating the variable damping’s full potential.

#### Track Validation

Figure 7 - Comparing front tyre load with constant vs variable damping against VI grade.

Further validation of variable damping can be seen when simulating around Pembrey. As seen in turn 1 from figure 7, there is a clear delta between the constant and variable damping which validates the working variable damping model. During the braking zone where the front tyres are loaded, the variable damper is closer to representing VI grade. However, when the front tyre starts to unload as the vehicle accelerates, the constant damping goes on to prove favourable in rebound.

This can be explained through comparing damping curves in figure A8 in the appendix, there is a clear difference in the rebound interpretation. VI grade interpolates the values as a curvature, whereas in Simulink they are linearly interpolated. The error is more prominent in rebound due to the increased curvature seen on the positive section of the graph in VI grade. During bump, the data points follow a more linear trend, therefore, the interpolation error is smaller. As a result, the variable method gives a more accurate high-speed bump, whilst the constant coefficient happens to approximate the rebound forces better by effectively averaging the non-linear region that the lookup table does not represent as precisely.

## 2.3 VI Grade Model

The sports car predefined model in VI grade was chosen to simulate differing tests which are replicated and compared in Simulink. Vehicle parameters from VI grade were collated and inputted into the Simulink model.

Straight line acceleration, sine steer (slalom), steady-state cornering and flying laps around Pembrey were conducted in VI grade changing spring and damper set ups. The acceleration and aerodynamic profiles were inputted into the 4-DOF and 7-DOF model to replicate the differing tests and validate the model. The differing spring and damper set up tests can be found in the appendix.

# Chapter 3 – Damper Changes

## 3.1 Bump Input

To analyse the critical damping, underdamping and overdamping phases in the Simulink model, a small bump was inputted into the model.

Figure 8 - Heave displacement using a bump input in Simulink to analyse differing damping set ups.

Figure 8 displays differing damper set ups to highlight overdamping, underdamping and critical damping. According to Milliken et al (1995), as expected, the soft set up (50% reduction in baseline coefficient) shows large oscillations in heave, demonstrating the vehicle taking longer to settle after the bump and is underdamped. When comparing the baseline and stiff set up on the baseline springs, there is less deviation, and both steer towards being critically damped. This suggests the initial damping configuration is stiff enough that any further damping has a diminishing return when paired with the stiff spring set up. Therefore, a baseline damper run was completed at a 50% softer spring rate, the graph shows that the damper then becomes overdamped at the same coefficient, suggesting that the baseline spring set up hides the damping effect.

## 3.2 Straight-Line Acceleration

To analyse the impact of damper changes on body motions of the VI grade sports car, a straight-line acceleration test from 0kph was conducted. During acceleration, the car squats and therefore front rebound to control the front extension, and rear bump to control rear compression, is prominent. (Vehicle Physics Pro, 2011) Combining both stiffened front rebound and rear bump results in an anti-squat set up. Dampers control the rate at which the motion occurs rather than the magnitude, therefore, pitch velocity can be compared between the damper set ups to understand how they resist velocity. (Ansys, 2020)

Figure 9 - Comparing pitch velocity between differing damper set ups.

Figure 9 displays the rate in which the vehicle pitches under acceleration. As expected, the anti-squat configuration produces the lowest peak velocity due to the stiffer setup restricting movement. (Azman et al, 2004) It can be inferred that softening the front rebound proves the highest delta change in velocity compared to the baseline rather than changing rear bump, displaying front suspension controlling pitch velocity. This is due to the stiffer rear springs dominating rear suspension movement and preventing the compression of the rear even when softening bump. Furthermore, from figure A10 in the appendix, it is seen that the baseline front rebound is stiffer than the rear bump initially and so softening an already softer rear bump will show less of an effect than softening a stiff rebound and allowing the front suspension to move more freely.

### 3.2.1 Preload Impact

When conducting the straight-line acceleration test, a large offset in pitch and heave were identified. A portion of this was attributed to preload as displayed in figure 10, whereby the Simulink model predicts pitch much closer to the VI grade with no preload.

Figure 10 - Pitch comparison with and without preload applied to springs.

The Simulink model doesn’t account for the pre-load on the springs. Therefore, the model’s axle loads will not start from the same baseline and so there will be an initial offset in tyre loads and heave.

## 3.2 Pembrey Analysis

Pembrey was simulated in VI grade to assess the effect of damper changes on the vehicle’s response and handling.

Figure 11 - Heave velocity comparison around Pembrey changing damper stiffness.

Figure 12 - Comparing front tyre loads around Brooklands Hairpin.

Figure 11 shows that softening the dampers results in a significant increase in heave velocity, as the suspension provides less resistance to vertical motion. This reduced damping leads to greater transient body movement, making the vehicle’s response less predictable. Figure 12, focusing on the Brooklands hairpin due to the large braking zone, demonstrates that the soft damper setting generates the largest instantaneous peak in front tyre force. As described by Albinsson (2013), this occurs because the increased heave and pitch velocity (figure A9 in the appendix) causes the sprung mass to compress the suspension more rapidly, producing an overshoot in tyre load and larger oscillations, showing an underdamped system. These short-duration load spikes can alter available grip, contributing to the more unpredictable handling behaviour associated with an underdamped system. The simulink trace was included to show the working model capturing soft damping and therefore increased front tyre load and subsequent oscillations under braking.

# Chapter 4 – Spring Changes

A design of experiments matrix was simulated in VI Grade around Pembrey to test differing spring set up configurations and their resultant behaviours.

Figure 13 - Section view of front tyre loads during Hatches Hairpin.

Figure 13 demonstrates the clear relationship between spring stiffness and front-loading during braking. Set ups configured with stiffer front springs (e.g., HB and HH) show higher initial tyre load peaks. This is due to the stiffer front springs resisting pitch and so suspension compresses less, resulting in more loads being directed to the tyres quicker. Contrastingly, the softer set ups show a reduced initial load spike due to the suspension being less restricted to pitch and allowing absorption of load under braking, resulting in less being transferred to the tyres. This demonstrates that whilst the overall magnitude of longitudinal load transfer is unchanged, spring stiffness influences the distribution of tyre load front to rear during the transient phase. It is seen that the front baseline and rear soft set up also proved to have a higher initial spike in load transfer, this suggests the baseline set up is comparatively stiff for the vehicle, resulting in limiting pitch similarly to the hard set-ups. The continued high front tyre load of this set up can be explained through insufficient pitch resistance. Once the front axle becomes loaded, the reduced pitch stiffness at the rear means it takes longer to return and so the front tyre remains loaded for longer, as supported by Koylu (2021).

## 4.1 Spring change Simulink model comparison

To validate the model can withstand set up-changes and maintain correlation with the VI grade simulation, soft springs were inputted.

Figure 14 - Pembrey comparison of pitch angle between Simulink and VI grade (baseline vs soft spring set up).

From figure 14, there is a clear delta between pitch angle in the baseline and soft spring set up in both the simulink and VI grade models. As expected, the softer springs allow more compression of the suspension, increasing the pitch angle in both braking and acceleration events around Pembrey. This proves the Simulink’s validity even with adjusted spring rates. However, despite the trends remaining similar, the simulink model clearly underpredicts the pitch angle throughout.

# Chapter 6 – 7 DOF Model

The 7-DOF model goes further in replicating the sprung mass damper system by incorporating lateral forces and lateral rotation. The 7-DOF model derives roll motion using α and β as the distance from the centreline to the respective left and right wheel centre. These distances are multiplied by the roll and added/subtracted within the extension equations to calculate the spring forces.

-Ixx∅= αKf2 Zs-Zf2-aθ+α∅+ α Kr2 Zs-Zr2+bθ+ α∅- βKf1 Zs-Zf1-aθ-β∅- βKr1Zs-Zr1+bθ-β∅-mayh

Equation 8 – Roll equation derivation

## 6.1 Validation against 4-DOF

Figure 15 - Comparing rear tyre loads between 4DOF, 7DOF and VI grade model.

Figure 15 displays the validation between all models. It is evident that during loading of the rear tyres under acceleration, the 7-DOF model proves more accurate to the VI grade simulation compared to the 4-DOF model. This can be explained through the capture of roll and individual wheel motion.  The 7-DOF model allows each wheel to move independently in response to pitch and vertical movements, whereas the 4-DOF averages this and so flattens the peak loads. Furthermore, despite the large improvement seen during acceleration (when rear tyres are fully loaded), there are still small lateral forces acting on the tyre that is not accounted for in the 4-DOF model due to not deriving roll. These effects accumulate to show the underprediction in rear tyre load for the 4-DOF model and is improved in the 7-DOF model, as supported by Sun’s (2002) simplified 4-DOF model.

## 6.2 Roll movement

To understand the effect of spring stiffness on roll, a slalom test can be analysed to capture large roll movements in the system.

Figure 16 - Comparing roll angle between 7-DOF model and VI grade at differing spring set-ups.

Figure 16 demonstrates the expected increase in roll angle as spring stiffness reduces. This is due to the suspension becoming less resistant to compression and allowing movement laterally, causing roll. Despite Simulink not incorporating anti-roll bars which VI grade does, highlighting limitations to the model, the 7-DOF still largely underpredicts the magnitude of roll seen in all spring sets. This can be explained by the lack of motion ratio in the Simulink model and therefore assumed to be 1. Whereas, in the VI grade model, the front motion ratio is 0.6094 and the rear is 0.7641. Therefore, where the simulink model is assuming the spring acts directly at the wheel, it increases the suspension stiffness and roll stiffness, causing an underprediction in roll.

## 6.3 Lateral Load Transfer

A steady state cornering test was conducted to compare the lateral load transfers and resultant distribution between differing spring set up configurations.

Table 2 - Lateral load transfer distribution comparison between spring set configurations in steady state cornering.

| Spring Set | LLTD (%) | Handling |
| --- | --- | --- |
| Baseline | 34.99 | Oversteer |
| HS | 53.09 | Understeer |
| SH | 21.33 | Oversteer |
| BS | 46.31 | Oversteer |
| SB | 24.89 | Oversteer |
| BH | 48.55 | Oversteer |
| HB | 41.12 | Oversteer |

From table 2, there is a clear correlation between increasing spring stiffness and increased lateral load transfer in steady state cornering due to greater resistance to roll. (Gillespie, 1996) As expected, the soft front and stiff rear set up proves the lowest lateral load transfer distribution and therefore increasing oversteer tendencies due to the outside rear tyre loading earlier than the front outside tyre. This results in an increase in the vehicle’s yaw sensitivity, therefore making the car more reactive but harder for the driver to control. Unlike the hard front and soft rear springs setup, the hard front and baseline rear springs do not steer toward understeer tendencies, despite the increased stiffness on the front axle. This suggests that the initial spring set is stiff enough whereby stiffening any further has little effect on the lateral load transfer distribution.

# Chapter 7 – Conclusion

To conclude, the report has demonstrated the response of springs and damper changes on the vehicles handling though a variety of track and dynamic tests. Agreeing with the fundamental dynamics, stiffer damping and spring set ups found to improve pitch and heave control of the vehicle, whilst softer set ups allow for greater and faster body movement with a less controlled handling response.

## 7.1 Model to model comparison and limitations

Throughout the report, it was found that the Simulink models tend to underpredict the body motions and tyre load changes compared to VI grade. This demonstrates the limitations to the model, which firstly can be attributed to assuming a motion ratio of 1 when in VI grade they are below. Therefore, making the Simulink model stiffer and reducing predicted suspension travel, so Simulink is more resistant to heave, roll and pitch. It also must be noted that the Simulink model doesn’t include anti-roll bars in its geometry and so can affect the overall stiffness and load distribution in the model. Additionally, the simulink model assumes a perfectly rigid suspension, with no flexibility in components. However, VI grade includes compliance effects due to its more complex geometry modelling and so resulting in more realistic tyre load and body movements, where the rigid assumptions in Simulink suppress these behaviours, contributing to the overall under prediction. Finally, the Simulink simplifies tyre modelling to a constant spring and damping coefficient, whereas VI Grade incorporates a tyre property file including slip angles and scaling coefficients. This will result in an under prediction in tyre force especially during transient manoeuvres where non-linear tyre dynamics are prominent.

## 7.2 Improvements

To improve the model further, ride frequency can be used to offer a more structured approach to spring stiffness selection. As explained by Rajeev (2019), ride frequency provides a link between mass, spring rate and expected dynamic response and so is a baseline tuning method for suspension stiffness. The current baseline sports car in VI grade produces a ride frequency of 1.55 Hz front and 1.94 Hz rear, which according to Rajeev (2019), is below the typical sports car target of 2.0-2.5 Hz. So, this shows a required increase in spring stiffness to create a more representative model, with sharper responses and improved pitch and roll control. Once spring rates are established using ride frequency, the dampers can be tuned to achieve the appropriate damping coefficient, ensuring the suspension is not over or under damped. This ensures the damping effects are not hidden by spring stiffness and so allows the suspension to settle quickly without excessive oscillations, improving stability and overall handling.

# Reference List

Albinsson, A. (2013) The damper levels influence on vehicle roll, pitch, bounce and cornering behaviour of passenger vehicles, Chalmers. Available at: https://odr.chalmers.se/server/api/core/bitstreams/da15e2f9-a56b-4a24-8901-edacbcdea924/content.

Ansys (2020) Fundamentals of damping effects: Lesson 2. Available at: https://innovationspace.ansys.com/courses/wp-content/uploads/sites/5/2020/12/2.6.2-Fundamentals-of-Damping-New-Template.pdf

Azman, M. et al (2004) Influence of anti-dive and anti-squat geometry in Combined Vehicle Bounce and pitch dynamics, Loughborough University. Available at: https://journals.sagepub.com/doi/10.1243/1464419043541464

Gillespie, T.D. (1996) Fundamentals of Vehicle Dynamics. Warrendale, PA: SAE International.

Koylu, H. (2021) Improvement of dynamic wheel load oscillations by coupling pitch motion into bounce motion during pitching motion of Passenger Vehicle, DergiPark. Available at: https://dergipark.org.tr/en/pub/politeknik/issue/67878/641156

MathWorks (2025) 1-D lookup table, Approximate one-dimensional function - Simulink. Available at: https://uk.mathworks.com/help/simulink/slref/1dlookuptable.html

Milliken, W.F. and Milliken, D.L. (1995) Race Car Vehicle Dynamics William F. Milliken; Douglas L. Milliken. Warrendale, Pa: Society of Automotive Engineers.

Rajeev, N. (2019) Natural frequency, ride frequency and their influence.

Sergers, J. (2014) Analysis techniques for race car data acquisition. SAE International: SAE International.

Sun, T. (2002) SAE International: Advancing mobility knowledge and solutions, SAE International, Available at: https://www.sae.org/publications/technical-papers/content/2002-01-1580/

Tudor (2020) SMD Systems Lectures: Spring Mass Damper Systems. Accessed: 13 October 2025.

Vehicle Physics Pro (2011) How simple suspensions work, Vehicle physics pro. Available at: https://vehiclephysics.com/advanced/how-suspensions-work.

# Appendix

Figure A1 -One degree of freedom Simulink model.

Figure A2 - Two DOF (unsprung mass) Simulink model.

Figure A3 - Four DOF (pitch derivation) Simulink model.

Figure A4 - Seven DOF equations of motion and inputs required in Simulink.

Figure A5 - Sevon DOF (roll derivation) Simulink model.

Figure A6 - Rear tyre loads comparison between variable and constant damping around Pembrey.

Figure A7 - Simulink bump input.

Figure A8 - Comparison between simulink (left) and VI grade (right) damping curve interpolations.

Figure A9 - Pitch velocity comparison around Pembrey changing damper stiffness.

Figure A10 - Damper set ups.

Figure A11 - Spring set ups.