| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| MAY 2024 | 2024 |

MODULE CODE: ACDA5001

Electrical Engineering

TIME:  2 HOURS

Maximum Marks: 100

answer ALL the questions

Equipment or materials required – Calculator, formula sheet

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

ANSWER ALL THE QUESTIONS

Question 1 [25 marks]

- A supply voltage of V1=15V is given to the circuit shown in fig:1. Determine the following

- The load current,								[8 Marks]

- Value of load resistor, when maximum power is transferred across the load,

[3 Marks]

- Simplified circuit diagram for the circuit shown in fig.1			[3 Marks]

- The maximum power transferred across the load (PL).			[3 Marks]

Fig:1

- Define Norton’s Theorem, with the aid of its equivalent Circuit.		[5 Marks]

- Calculate the voltage VDC across the open circuited terminal of the circuit shown in fig:2.											[3 Marks]

Fig:2

Question 2 [25 Marks]

- A 415V, three phase AC motor has a power output of 12.75kW and operates at a power factor of 0.77 lagging and with an efficiency of 85%. If the motor is delta-connected, determine the following,

- The Power input	[5 Marks]

- The Line current 	[4 Marks]

- The Phase current	[4 Marks]

- An Alternating voltage is given by the equation V=100Sin (200πt-90). Calculate the following.

- The amplitude								[2 Marks]

- The peak to peak voltage							[2 Marks]

- The RMS voltage								[3 Marks]

- The periodic time								[3 Marks]

- The Frequency								[2 Marks]

Question 3 [25 marks]

- A Coil of inductance 318.3mH is connected in series with a 200 Ω resistor to a 240V, 50Hz supply. Calculate the following.

- Inductive reactance of the coil						[3 Marks]

- Impedance of the circuit 							[3 Marks]

- Current in the circuit							[2 Marks]

- Potential difference across each components				[4 Marks]

- Circuit phase angle								[3 Marks]

- Find the RMS value and Phase of the current, I=25+j40 A			[4 Marks]

- Differentiate Active power, Reactive power and Apparent power [ minimum of two differences] 									[6 Marks]

Question 4 [25 Marks]

- List any five types of DC Generators.						[5 Marks]

- A Transformer has a rated output of 200KVA at a power factor of 0.8. Determine the real power and the reactive power.							[5 Marks]

- Describe the functions of the following components used in DC machines

- Field System								[3 Marks]

- Yoke									[3 Marks]

- Commutator								[3 Marks]

- Carbon brush								[3 Marks]

- Armature									[3 Marks]

Diwedd/End