#include <main.h>

#define LCD_ENABLE_PIN PIN_D2

#define LCD_RS_PIN PIN_D0

#define LCD_RW_PIN PIN_D1

#define LCD_DATA4 PIN_D4

#define LCD_DATA5 PIN_D5

#define LCD_DATA6 PIN_D6

#define LCD_DATA7 PIN_D7

#include <lcd.c>

unsigned int RV1;									//integer designation RV1 for the adc value

unsigned int prev_kitt_input;						//integer designation prev_kitt_input

unsigned int prev_but_input;						//integer designation preb_but_input

unsigned int refresh_lcd;							//variable designation for refresh lcd

unsigned int anti_c,  clo, stop;					//integer designation for anti_c, clo, stop

//---------------------------------------------------------------------------------------------------------------------

void analog(void) {									//analog function start

set_adc_channel(0);								//sets the adc channel we are using

delay_us(30);									//time delay

RV1 = read_adc();								//assigns a value to RV1 - the value that is on the adc

lcd_gotoxy(6,1);								//assigns which position the text will go to on the lcd display

printf(lcd_putc,"RV1 = %3u", RV1);				//prints text onto the lcd display

}													//end of the analogs function

//---------------------------------------------------------------------------------------------------------------------

void motor_direction(void) {						//start of the motor_direction function

analog();										//calls the subroutine analog, in order to get a value from the potentiometer

output_b(RV1);									//outputs the value of the potentiometer onto port b to show the number in binary form

if(RV1 <= 75) {									//comparing the value of RV1 to 75

clo = 0;									//sets the variable to 0 to clear any previous declerations

stop = 0;									//^^

output_high(pin_C1);						//if the value of RV1 < 75 the pin C1 is set high

output_low(pin_C0);							//pin_C0 is set low

lcd_gotoxy(4, 2);							//assigns a position for the text anti-clockwise

printf(lcd_putc,"Anti-Clockwise");			//prints anti-clockwise on the lcd display

if(anti_c == 0) {							//testing if this variable is equal to zero

anti_c = 1;								//setting the variable to 1 to perform a latching function, so it only display once

printf("\fAnti-clockwise");				//displays Anti-clockwise onto the virtual terminal, "\f" clears the words previously on the terminal

}

}												//end of if statement

if(RV1 >= 175) {								//comparing RV1 to 175

anti_c = 0;									//clears any value attached to that variable

stop = 0;									//^^

output_high(pin_C0);						//if RV1 is greater than 175 pin C0 is set high

output_low(pin_C1);							//pin C1 is set low

lcd_gotoxy(4, 2);							//assigns a position on the lcd display

printf(lcd_putc," Clockwise   ");			//prints clockwise on the lcd display

if(clo == 0) {								//checks to see if this variable is zero

clo = 1;

printf("\fclockwise");

}

}												//end of if statement

if(76 <= RV1 && RV1 <= 174) {					//comparing RV1 to 76 & 174

anti_c = 0;

clo = 0;

output_low(pin_C1);							//if RV1 is between 76 and 174 output pins C1 and C0 are set low

output_low(pin_C0);

lcd_gotoxy(4, 2);							//assigns a position on the lcd display

printf(lcd_putc,"   Stopped    ");			//prints stopped onto the lcd display into the assigned position

if(stop == 0) {

stop = 1;

printf("\fstopped");

}

}												//end of if statement

}													//end of motor_direction function

//---------------------------------------------------------------------------------------------------------------------

void adc_led(void) {								//start of adc_led

analog();										//runs the function analog to get a value from the potentiometer

output_b(RV1);									//outputs the value from the potentiometer onto port b

printf("RV1 = %3u \r", RV1);					//displays this value in the virtual terminal

}													//end of adc_led

//---------------------------------------------------------------------------------------------------------------------

void kitt_mode(void) {																		//function to turn the leds into the lights from the famous car Kitt from knight rider

unsigned int ledl_r[] = {1, 2, 4, 8, 16, 32, 64, 128, 128, 64, 32, 16, 8, 4, 2, 1};		//this array contains the order in which the leds will turn on to produce the kitt effect

output_low(pin_C0);																		//clears the pins associated with the motor

output_low(pin_C1);

output_b(0);																			//clears the leds in preparation for the kitt mode

for(int i = 0; i < 16; i++) {															//this for loop will iterate variable 'i' up to 16

output_b(ledl_r[i]);																//outputs the value on the array according to the index i

delay_ms(200);																		//time delay inbetween leds switching on and off

}

}																							//end of kitt mode function

//---------------------------------------------------------------------------------------------------------------------

void motor_hold(void) {								//start of the motor function for task 3

if(input(pin_A1)) {								//testing if the button is pressed

output_high(pin_C0);						//setting pin C0 high

output_low(pin_C1);							//sets pin C1 low to make the motor spin clockwise

lcd_gotoxy(27, 1);							//assigns a position on the lcd display for the message

printf(lcd_putc,"on ");						//displays the string "on " onto the lcd display

if(prev_but_input == 0) {					//testing if the variable prev_but_input is equal to 0 (latching function)

prev_but_input = 1;						//attaches the value 1 to the variable prev_but_input

printf("\fon ");						//displays the word 'on' in the virtual terminal

}

}

else {

if(prev_but_input == 1) {					//testing if the variable prev_but_input is equal to 1, which it will be due to line 84

prev_but_input = 0;						//equates prev_but_input to 0

printf("\foff");						//displays off in the virtual terminal

}

output_low(pin_C0);							//if the button is not pressed, both pins are switched low to stop thye motor from spinning

output_low(pin_C1);

lcd_gotoxy(27, 1);							// assigns a position on the lcd display for the next message

printf(lcd_putc,"off");						//prints 'off' onto the lcd display

}

}													//end of motor function

//---------------------------------------------------------------------------------------------------------------------

void main() {										//main function

setup_adc_ports(sAN0);							//initialisation code for the adc

setup_adc(ADC_CLOCK_INTERNAL|ADC_TAD_MUL_0);	//initialisation code for the adc

lcd_init();										//initialisation code for the lcd display

while(TRUE) {									//this will loop eternally

if(input(pin_A2)) {							//testing if switch a has been switched on

motor_direction();						//calls the function motor_direction

}

if(input(pin_A3)) {							//testing if switch b has been switched on

output_low(pin_C0);						//clears the motor pins

output_low(pin_C1);

adc_led();								//calls the function adc_led

}

if(input(pin_A4)) {							//testing if switch c has been pressed

output_b(0);							//clears the leds

motor_hold();							//calls the function motor_hold

}

if(input(pin_A5)) {							//testing if switch d has been pressed

if(prev_kitt_input == 0) {				//testing if the value associated with prev_kitt_input is zero

prev_kitt_input = 1;				//equates the variable to 1

lcd_gotoxy(27, 2);					//assigns a position on the lcd display for the next message

printf(lcd_putc,"Kitt Mode");		//displays kitt mode on the lcd display

printf("\fKitt Mode\r");			//prints kitt mode on to the virtual terminal, once

}

Kitt_mode();							//calls the subrountine kitt_mode

}

else {

if(prev_kitt_input == 1) {				//testing if the variable prev_kitt_mode is equal to one

prev_kitt_input = 0;				//equates prev_kitt_input to 0

printf(lcd_putc,"\f");				//clears the lcd display

printf("\f");						//clears the virtual terminal

}

}

}

}

Comments

2.. 20%   3.. 30%    4.. 40%   5.. 10%

****************************************

2.. Your code has a number of structures and all work. Your statements indicate what each line does 75%

3.. Your program flows nicely 75%

4.. Your program achieves the desired results. 75%

5.. Your program is nicely laid out and well indented. 75%

Total 75%