Question 1

Our products must not only work correctly when first manufactured, they must continue to do so, for as long as our customers reasonably expect them to.

- A component of a system has a known constant failure rate of 0.03/failures per 1,000 hours under its current operating conditions.

Calculate its reliability for continuous operating periods of 2,500 hours, 5,000 hours and 7,500 hours.  Comment on the results.

(10 marks)

- Assuming a component has a known constant failure rate of 0.025/failures per 1,000 hours; predict the combined reliability over 3,000 hours of two such components connected together in series and parallel.

(7 marks)

- Analyse the advantages and disadvantages associated with performing reliability testing.

(8 marks)

Question 2

Critical equipment is expected to be reliable continuously and run for long periods of time without failing.

- There are many definitions of reliability, but four factors keep coming to mind (1) Numerical value; (2) Intended function; (3) Life and; (4) Environmental conditions.  Evaluate each of these four factors in relation to the reliability of products.

(8 marks)

- Discuss two ways in which failure can be classified.

(4 marks)

- Twenty components were put on test and the following failures occurred in hours:

15, 85, 105, 145, 225, 295, 335, 465, 684, 878, 1000, 1250, 1305, 1425.  The test failed after 1500 hours with 6 components still surviving.

Calculate the mean time between failure (MTBF) and the failure rate.

(6 marks)

- Analyse the significance of mean time between failure and mean time to repair in modern manufacturing operations.

(7 marks)

Question 2

The study of reliability of equipment and failure patterns is an essential part of process and equipment improvement plans.

- A leading manufacturer of centrifugal pumps has gathered the following data related to repairs required by model MFG over the past 12 years

| Year | Failure rate |
| --- | --- |
| 1 | 18,000 |
| 2 | 2,600 |
| 3 | 950 |
| 4 | 875 |
| 5 | 800 |
| 6 | 825 |
| 7 | 890 |
| 8 | 970 |
| 9 | 960 |
| 10 | 1750 |
| 11 | 28,000 |
| 12 | 60,000 |

Using the data above, construct a bathtub curve and clearly explain the significance of the main sections of the curve in relation to reliability engineering.

(12 marks)

- SHD Logistics is considering installing a new picking line, but requires data on the overall reliability of the system.  Using the data below calculate the overall reliability of the picking line.

R1 = 0.89

R2 =0.82

(13 marks)

R3 = 0.95

R4 = 0.95

R8 = 0.90

R6 = 0.98

R5 = 0.90

R7 = 0.98