A REPORT FOCUSING ON:

A CLIENT LETTER DEMONSTRATING AN UNDERSTANDING OF THE BIM MATRUITY REQUIREMENTS APPENDED WITH A BIM EXECUTION PLAN FOR THE YSGOL BAE ABERTAWE PRIMARY SCHOOL PROJECT.

FELIX HOARE

STUDENT NUMBER: 1908089

SUBMISSION DATE: 29/03/22

LECTURER: GAVIN TRAYLOR

MODULE: BUILDING INFORMATION MODELING

Table of Contents                                                                                            Page Number

0.0	BIM Execution Plan                                                                                4

1.0	Purpose of Document                                                                              9

2.0	Project Background                                                                                 9

2.2	Project Information                                                                                  9

3.0       Management                                                                                           10

3.1       Roles, responsibilities, and authorities                                                   10

3.2       Major Project Milestones                                                                        11

3.3      Approval of Information 	 11

3.4      Authorisation Process                                                                              12

4.0     Planning and Documentation                                                                    12

4.1     Project Implementation Plan                                                                     12

4.2     Agreed processes for Collaboration and information modelling              13

Table of Contents                                                                                              Page Number

4.3 Clash Avoidance, Detection and Resolution                                               14

4.4 Responsibility Assignment Matrix                                                              14

4.5 Task Information Delivery Plan                                                                  14

4.6 Master Information Delivery Plan                                                               14

5.0 Standards, Methods and Procedures                                                            15

5.1 Origin and Orientation                                                                                 15

5.2 File Naming Codes                                                                                      15

6.0 Digital Software                                                                                          18

6.1 Software Versions                                                                                       18

6.2 Exchange Formats                                                                                       18

6.3 Federation Strategy                                                                                     19

City and County of Swansea

Civic Centre

Oystermouth Road

Swansea

SA1 3SN

The Post House

Adelaide St

Swansea

SA1 1SB

29th March 2022

RE: Ysgol Bae Abertawe

Dear Mr Jones,

I am writing to you regarding the upcoming project for a state-of-the-art infant school in Swansea. We are delighted to have the opportunity to deliver Ysgol Bae Abertawe primary school in the heart of Swansea Bay Marina which we believe will be a great addition to the beach front.

The project which is aiming to begin construction in July 2023 will be aiming to successfully achieve BIM maturity level 2 through collaborative working throughout all stages of the project. The main contractor will be Westbrook Contractors due to their experienced background in public sector commercial buildings. We (Greys Architects) will be managing the design, whilst Whitestone Consultants will be handling the consultancy work. This letter will detail how the knowledge exchange partnership framework will be established and implemented to enable information to be shared, coordinated, and managed effectively.

Traditionally, the goals and objectives set out in the initial stages of the project to comply with BIM level 2 would be in response to the employers Information Requirements (EIR), as this document is not available, we have anticipated what we believe will be the BIM requirements for the Ysgol Bae Abertawe project using the information we have access too. This has been established in the BIM Execution Plan (BEP) which is appended to this letter. Within this plan, it sets out the plan of works, project deliverables, roles and responsibilities, information management, project delivery and the key methods and standards that will be adhered to in accordance with BSN ISO 19650. The BEP will be supported by the task information delivery plan (TIDP) consisting of the information deliverables presented by the task teams which will form the master information delivery plan (MIDP) which will set out the timeframes, roles, and correct standards that will be adhered to.

It is recognized that to achieve a fully collaborative method of working, the use of information technology is key to support the new workflow (Alazmeh, Underwood and Coates, 2018). A common data environment (CDE) will therefore be implemented which will consist of information from the Project Information Model (PIM) and Asset Information Model (AIM) which will allow all documentation, graphical models, and non-graphical assets to be shared to support the collaborative production of information in accordance with BS ISO 19650 – 2:2018, 5.6 and 5.7 to establish a federated information model. This model will contain information which will be delivered by the task teams from the respective parties (Winfield, M. 2022). Task information management functions will be implemented for each individual task team to coordinate the information associated with that team across multiple tasks (BSN ISO 19650 -1 2018).

A platform called ‘Revizto’ will be used to share the information between project team members. The software will allow the information to be broken down into several key areas to facilitate collaboration through the project team and help to avoid duplication or mistakes, whilst improving the efficiency of the creation and sharing of information (Common data environment CDE, 2022).

The information within the CDE will be structured to ensure there is a clear workflow process in place. The four main areas will be:

Work In Progress (WIP) – In this section it will allow the individual task teams from Greys, Westbrook, and Whitestone to hold unapproved information whilst in the process of being developed until it is ready to publish.

Shared – Here information has been reviewed and approved for sharing with task and delivery teams from the other project parties. This stage enables the approvals process where information is reviewed and accepted.

Published – This is information that has been authorized for use in more detailed design.

Archive – This area will show a journal of all information transactions and an audit trail of information throughout the lifecycle of the project.

(BSN ISO 19650 -1, 2018)

All documentation within the CDE will be quality assured and delivered according to the accepted criteria and level of information required. There will be quality assurance processes in place to allow a document to change status as specified within EN ISO 19650-2:2018, 5.6.4.

Through using Revizto as a single storage point, it avoids confusion regarding what information should be being referenced and will give the project team members confidence that they are using the latest assets. Ownership for the information within each of the sections remains with the project team that produced it, and only they are allowed to change the contents improving spatial coordination of data which will ultimately reduce both time and cost on the project (McPartland, McPartland and Sharman, 2022).

To ensure that BS ISO 19650 – 2:2018 5.4 is satisfied, an information management process (IMP) will be established to maintain order of the AIM. A BIM manager will be appointed to oversee the information in the shared and published sections and ensure the integrity of the work.

Data security will be a key focus for all information within the CDE to comply with PAS 1192-5:2015. Careful consideration will be taken to ensure that sharing information is done in a secure way. Through using Revizto, it can maintain security at folder level in addition to access/deletion rights assigned to project roles. This will be overseen by the information manager and a full audit trail will be available to ensure no hostile activity takes place and information is safe from being copied.

To ensure that all parties on the project are BIM compliant and can locate and manage data easily, a unified classification system will be implemented in accordance with BS ISO 19650 – 2:2018 5.1.7 b. Naming containers will be used to ensure the information that is being shared is structured, consistent, and understandable (File Naming Convention - BIM Level 2 Guidance, 2022). Through implementing a project information standard format, it will improve navigation between documents for all parties and improve coordination of information management.

Finally, information in open data formats will be structured in COBie format in accordance with the UK National Annex within BS EN ISO 19650-2. Although there is no EIR present, the data will be collected via a single spreadsheet and updated throughout the lifecycle of the project as we believe this single point of information will be a vital built legacy dataset for the project.

I trust this letter provides you with the reassurance that the project will confidently achieve the BIM level 2 maturity requirements and will demonstrate a clear and effective collaborative approach to building the Ysgol Bae Abertawe project.

Yours Sincerely,

Felix Hoare

References

Alazmeh, N., Underwood, J. and Coates, P., 2018. Implementing a BIM collaborative workflow in the UK construction market. International Journal of Sustainable Development and Planning, 13(01), pp.24-35.

Bimportal.scottishfuturestrust.org.uk. 2022. Task: File Naming Convention - BIM Level 2 Guidance. [online] Available at: <https://bimportal.scottishfuturestrust.org.uk/level1/stage/8/task/47#:~:text=The%20numbering%20for%20standard%20coding,given%20in%20Table%205%20below.> [Accessed 19 March 2022].

Designingbuildings.co.uk. 2022. Common data environment CDE. [online] Available at: <https://www.designingbuildings.co.uk/wiki/Common_data_environment_CDE> [Accessed 15 March 2022].

EN ISO 19650-1, Organization of information about construction works — Information management using building information modelling — Part 1: Concepts and Principles

EN ISO 19650-2, Organization of information about construction works — Information management using building information modelling — Part 2: Delivery phase of the assets

McPartland, R., McPartland, R. and Sharman, J., 2022. What is the Common Data Environment (CDE)?. [online] NBS. Available at: <https://www.thenbs.com/knowledge/what-is-the-common-data-environment-cde> [Accessed 16 March 2022]

Winfield, M. (2022). Legal Implications of Digitization in the Construction Industry. In: Bolpagni, M., Gavina, R., Ribeiro, D. (eds) Industry 4.0 for the Built Environment. Structural Integrity, vol 20. Springer, Cham

Ysgol Bae Abertawe Infant School

Project No: 5010

Document Title: BIM Execution Plan

Document Status: Work in Progress

Revision: P01

Date: 29/03/2022

Client Name: Welsh Government

Author: Felix Hoare

File Name: 5010-GRE-XX-ZZ-D-A-001

The Post House

Adelaide Street

Swansea, SA1 1SB

United Kingdom

T +44 7393865626

Document History and Status

|  | Revision |  |  | Date |  |  | Description |  |  | Author |  |  | Checked |  | Reviewed |  |  |  | Approved |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | P01 |  | 29 March 2022 |  |  | First Issue |  | FH |  |  | GT |  |  | FH |  |  |  | FH |  |

Executive Summary

This BIM Execution document and its contents has been prepared and structured in accordance with BS EN ISO 19650-2:2018 and is intended solely for Greys, the Welsh Government or their Employer’s Agents information and use in relation to the Ysgol Bae Abertawe Infant School project.

1. Purpose of Document

  - The purpose of this document is to establish the BIM (Building Information Modelling) Execution Plan (BEP) and shall list the agreed targets for responsibility, timely delivery, exchange, and quality of information and data to the client for the Ysgol Bae Abertawe Infant School. This BEP is produced in response to BS EN ISO 19650-2:2018 section 5.4, “Information management process — Appointment”.

  - No Pre-Appointment BEP was prepared or submitted as part of the Tender response. This BEP shall therefore also confirm and detail the prospective requirements normally contained within a Pre- Appointment BEP.

  - It is understood that the Employers Information Requirements (EIR) is not available. Therefore, this BEP supersedes the EIR, and shall be delivered in accordance with the requirements placed on the Lead Appointed Party (Westbrook Contractors Ltd) and Appointed Party (Greys Architects) detailed in BS EN ISO 19650.

2. Project Background

  - The project objective is to deliver a ‘state of the art’ infant school in the Maritime Quarter, Swansea. The project will provide an infant school for 90 pupils with 48 part time nursery provision. The location of the land is a brown field site which has a small statue, boathouse and retaining wall which will need to be demolished as part of the build. The project will be built in accordance with BIM Maturity level 2 and aim to achieve a collaborative working approach in accordance with BS ISO 19650.

2.2 Project Information

2.2.1 Table 1 -2 includes key project information and milestones.

Table 1. Project Information

|  |  |  |  | Project Specific Details |
| --- | --- | --- | --- | --- |
|  | Client / Employer |  | Welsh Government |  |
|  | Project Name |  | Ysgol Bae Abertawe Infant School |  |
|  | Project Address |  | Maritime Quarter, Swansea |  |
|  | Project Description |  | Development of 1 form entry infant school for 90 pupils, with 48 Nursery provision |  |
|  | Project Number |  | 5010 |  |
|  | Contract Form |  | NEC Professional Services Contract Option C Target Cost |  |

|  |  |  |  | Project Specific Details |
| --- | --- | --- | --- | --- |
|  | Project Design Start Date |  | February 2022 |  |
|  | Project Construction Start Date |  | July 2023 |  |
|  | Project Completion and Handover Date |  | July 2024 |  |
|  | Project Description |  | See 2.1 Project Background |  |

Table 2. Project Information & Milestones

3.0 Management

3.1    Roles, responsibilities, and authorities.

This section aims to fulfil BS EN ISO 19650-2:2018 section 5.4.1(a). This BEP will nominate the capable persons to fulfil the required roles as to who will manage the information management functions of the Lead Appointed Party and Appointed Party for the project delivery.

    - Table 3. defines key roles.

Table 3 Named Key roles

| Name | Company | Role | Contact Information |
| --- | --- | --- | --- |
| Felix Hoare | Grey’s Architects | Lead Designer | Felix.Hoare@greys.com |
| Steve Jenkins | Westbrook Construction | Construction Manager | Steve.Jenkins@westbrook.com |
| James Best | Grey’s Architects | Task Team Manager | James.Best@greys.com |
| Andrew Davies | Westbrook Construction | Task Team Manager | Andrew.Davies@westbrook.com |
| Kevin Morris | Whitestone Consultants | Task Team Manager | Kevin.Morris@whitestone.com |
| Chris Davies | Grey’s Architects | Interface Manager | Chris.Davies@greys.com |
| Carl Jones | Grey’s Architects | Project Information Manager | Carl.Jones@greys.com |
| Ian Parker | Grey’s Architects | Project Delivery Manager | Ian.Parker@greys.com |
| Cameron John | Grey’s Architects | BIM Manager | Cameron.John@greys.com |

|  | BS EB ISO 19650 Term |  | Description |  |  |  | Contract Definitions |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Appointing party |  | The organisation leading the project. |  |  | The Employer or the person/organisation authorised to act on behalf of the Employer (Welsh Government) Client (Welsh Government) |  |  |  |
|  | Lead appointed party |  | The party that is responsible for managing and coordinating information between task teams. |  |  |  |  | Contractor (Westbrook Contractors Ltd) |  |
|  | Appointed party |  | Anyone generating information about the project |  |  |  |  | Designer (Greys Architects) |  |
|  | Project team |  | Everyone involved in the project, regardless of appointment/contract arrangement. |  |  |  |  | The Employer, Engineer, Contractor, Designer, Checker, and Supply Chain Members working on the project. |  |

    - Table 4 shows the new terminology of roles within an BS EN ISO 19650 project.

    - Table 5 shows the role responsibilities.

Table 5. Role Responsibilities

| Responsibility | Resource |
| --- | --- |
| Overseeing the processes and procedures for the information exchange on projects. | Project Information Manager |
| Enforce Spatial coordination | Lead Designer |
| Ensure documents standards are adhered to | Task Team Manager |
| Manage and coordinate the space | Interface Manager |
| Manage drawings, models, and documents to ensure they are complaint | Task Team Manager |
| Oversee the CAD related BIM Standards | CAD Coordinator |

3.2 Major Project Milestones

Project Milestones have been identified in section 2.2.1 and shown within table 2.

3.3 Approval of Information

Table 8 is used to identify the approval process for information when distributed amongst parties. It is imperative that any information shared adheres to the approval process to ensure it is checked and agreed by the relevant personnel.

Table 8 – Schedule of information approval responsibilities.

| Role | Models | Drawings | Peer Review | Lead Designer/ Lead Contractor | Client Review Team |
| --- | --- | --- | --- | --- | --- |
| Lead Designer | X | X | X |  |  |
| Project Delivery Manager | X | X | X |  |  |
| Construction Manager |  | X |  | X |  |
| Project Information Manager | X |  |  | X |  |
| Task Team Manager | X | X | X |  |  |
| Project Manager |  |  |  |  | X |

3.4 Authorisation Process

The approval process for all information should be confirmed at the initial stages of the project. The persons responsible for reviewing the information will take the necessary steps to confirm and approve the information within 5 working days to avoid delays.

4.0 Planning and Documentation

This section of the BEP covers the requirements of BS EN ISO 19650-2:2018 section 5.4.1.

4.1 Project Implementation Plan

Due to the fact the Appointing Party’s Exchange Information Requirements (EIR) were not available, the level of detail required for data exchanges is unknown. As a result, the models, documents, and data will be produced in accordance with the requirements set out in BS EN ISO 19650-2:2018.

The modelling shall be developed for all design disciplines, supported by the Task information Delivery plans which will contain the deliverables from each task team (See section 4.5 - 4.6). The models will be to an agreed level of detail and accuracy in accordance with Clause 11.2 of BS EN ISO 19650-2:2018.

The role of the information manager will be to ensure project model information is exchanged in an agreed format, to the specified level of detail, and verified before sharing and issued for approval. The Integrated Delivery Team (IDT) consisting of Greys, Westbrook and Whitestone will establish a federated 3D BIM model which will consist of architectural and structural models linked via Revit, see section 6.1.

The design teams will be responsible for modelling and checking their own designs before sharing them in the Common Data Environment. They will also be expected to take a proactive approach to resolving any clashes through collaboration with other teams where necessary. To ensure there is an audit trail, clashes will be tracked until resolved, see section 4.3 Avoidance.

4.2 Agreed Processes for Collaboration & Information Modelling

This section is used to identify how information will be managed and exchanged amongst parties. As there is no EIR present, the below formats have been assumed as the most effective platform to allow parties to collaborate effectively.

The exchange of information between the identified task teams for Greys, Westbrook and Whitestone will be carried out through Revizto which will be used as the Common Data Environment (CDE). All deliverables will be managed via this platform which will allow parties to upload documents to enter a review process before approval. The process in which information will be shared through the four stages of the CDE is shown in Table 9.

Table 9 – Common Data Environment Stages

| Stage |  |  | Description |  |  |
| --- | --- | --- | --- | --- | --- |
| Work – In – Progress | Task teams from Greys, Westbrook and Whitestone can hold unapproved information whilst it is still being worked on until it is ready to publish. |  |  |  |  |
| Shared | This is information that has been reviewed and approved for sharing with task and delivery teams. Informaton at this stage will clearly show if it has been approved or rejected. |  |  |  |  |
| Published | This is information that has been authorized for use in more detailed design. |  |  |  |  |
| Archive | This area will show a journal of all information transactions and an audit trail of information throughout the lifecycle of the project. |  |  |  |  |

(BSN ISO 19650 -1, 2018)

Clash avoidance process can be seen under section 4.3 and the file naming suitability can be seen in section 5.3.

Table 10 – Processes for collaboration and information modelling

| Company | Solution | Network | Database | File Based | Comments |
| --- | --- | --- | --- | --- | --- |
| Greys Architects | Revit | Revizto | Model | RVT |  |
| Whitestone | Revit | Revizto | Model | RVT |  |
| Westbrook | Revit | Revizto | Model | RVT |  |
| Greys Architects | Revizto | Revizto | Model | RVT | Clash Detection |

4.3 Clash Avoidance, Detection and Resolution

4.3.1      Through using Revizto, it enables information to be shared between project team members which has been broken down into key areas. This process will act as a clash avoidance mechanism helping to avoid duplication and mistakes through the CDE.

4.3.2      The project information manager will assist in managing Interdisciplinary Design Coordination meetings (IDC). These meetings will enable the teams to identify any issues and confirm actions. This process for closing out an IDC will be part of the design verification.

4.3.4 	Agreed Clash Rendition Viewers are listed in the Table 10 below.

Table 11 Model Rendition Viewer

| Agreed Clash Rendition Viewer | Version |
| --- | --- |
| Revizto | 4.14 |

4.4 Responsibility Assignment Matrix (RACI)

Considering the client has not provided an EIR, the level of detail is yet to be confirmed. The RACI will be produced using the LOD template in due course.

4.5 Task Information Delivery Plan (TIDP)

In accordance with BS EN ISO 19650-2:2018 section 5.4.4, task information delivery plans will be produced by the task team managers from Greys, Whitestone and Westbrook which will establish the responsibilities for the information deliverables and how the information will be manged. Although there is no EIR available, the task teams will produce a schedule of the list of information containers which will identify for each one its name and title, predecessor or dependencies, level of information, estimated time required for production, author and milestones (UKbimframework,2022).

4.6 Master Information delivery plan (MIDP)

In accordance with BS EN ISO 19650-2:2018 section 5.4.5, the master information delivery plan will be developed through collaboration with the task teams to combine the TIDPs to form the MIDP.  Westbrook Contractors will be responsible for forming the MIDP. They must baseline deliverables and dates within the MIDP in addition to notifying task teams of any changes and ensure all risks are properly managed (UKbimframework,2022). Reviews with the client will also be conducted to ensure the MIDP is accurate and up to date.

5.0 Standards, Methods and Procedures

This section fulfils BS EN ISO 19650-2:2018 section 5.4.1 (e) of the Information Standards.

5.1 Origin and Orientation

The origin and orientation for the project are based on the project location and its reference to other global or local grids.

| Point | Grid Reference | X (Easting) | Y (Northing) | Latitude / Longitude |
| --- | --- | --- | --- | --- |
| Site local grid origin | SS659923 | 265976.9 | 192364.7 | 51°36′51″N , 003°56′14″W |

Table 12 – Origin and Orientation schedule

5.2 File Naming Convention

Greys will generate filenames that conform to the conventions defined in BS EN ISO 19650-2:2018 National Annex (NA). All files will be named using Revizto to avoid disputes against the file naming convention defined for the scheme.

Table 13 – Example of File Naming

| Project |  | Originator |  | Functional |  | Location Location 2 Type Role Number Spatial Breakdown |  | Form |  | Discipline |  | Number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Breakdown |  |  |  |  |  |  |  |  |
| 5010 | GRE |  | XX |  | ZZ |  | D |  | A |  | 0001 |  |

For example, the above would give a filename of: 5010-GRE-XX-ZZ-D-A-001

5.2.1 Project Code

The project number could consist of 4no. digits  
Table 14. Project Code example

| Code |  | Project |  |
| --- | --- | --- | --- |
| 5010 | Ysgol Bae Abertawe |  |  |

5.2.3 Originator Codes

The originator code which is usual between 3 and 6 letters defines the company or organisation which initially created the file.

Table 15. Originator Codes

|  | Code |  | Originator Company Name |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | GRE | Grey’s Architects |  |  |  |

5.2.4 Document Form

The document form relates to the type of file that the document is. It allows files to be located easily and improves segregation of project information. As per BS ISO  19650-2:2018 National Annex NA. 3.6.

|  | File Type |  |  | Type |
| --- | --- | --- | --- | --- |
|  | D |  | Drawing |  |

Table 16. Document Form

5.2.5 Discipline

Discipline codes identify the role the originator organisation play in the creation of the specific file. As per BS EN ISO 19650 this field cannot extend beyond 2 characters in length.

The role defined below is in accordance with BS EN ISO 19650-2:2018.   
Table 17. Document Discipline

|  | Code | Role |
| --- | --- | --- |
|  | A | Architect |

5.2.6 Functional Breakdown

The function breakdown is a fixed text used as a unique identifier for the functional aspects of the information container breakdown structure in compliance BS EN ISO19650:2-2018 (NA.4.4).

Table 18. Functional Breakdown

|  | Code | Location Description |
| --- | --- | --- |
|  | XX | General |

5.2.7 Spatial Breakdown

The spatial breakdown shall be in accordance BS EN ISO19650:2-2018 (NA.4.4) and as stated in GG 184 CAD/BIM Standards.

Table 19 – Spatial Breakdown codes

|  | Code | Level |
| --- | --- | --- |
|  | 00 | Ground Floor |
|  | 01 | First Floor |
|  | 02 | Second Floor |
|  | ZZ | Mulitlevel |

5.2.8 Number

The ‘number’ is a 4-character code at the end of the filename The numbers are sequentially generated in order, i.e. 0001, 0002, etc.

Table 20 – Number naming codes

| Code | Level |
| --- | --- |
| 0001 | Ground Floor |

5.2.9 Revisions

Through using the Common Data Environment, the naming of revisions of documents and drawings should be automatically resolved. Revizto, which will generate a specific revision letters and name for any revision of any file as outlined in BS EN ISO19650-2:2018and the originator will not be required to input revision numbers. Revisions will begin with P01.

5.2.10 Metadata

The Metadata provides a description or information with respect to the data file, its purpose, and is essential for efficient file control.

5.2.11 Status Codes

Each file created on the scheme must have the relevant status code defined by the originator and each team member with responsibility for checking, reviewing, and approving as the process progresses.

Table 21. Metadata Status Codes from BS EN ISO 19650-2:2018

|  | Code | Description | Typical uses on this scheme for this stage |
| --- | --- | --- | --- |
|  | Work in Progress |  |  |
|  | S0 | Initial status or WIP | Document creation and development |
|  | Shared/Issued |  |  |
|  | S1 | Suitable for Coordination | For use by others |
|  | S2 | Suitable for Information | interim If requested by Appointing Party in the |
|  | S3 | Suitable for Review & Comment | A draft for comment |
|  | S4 | Suitable for Stage Approval | Approved by Greys and ready to be issued for acceptance and authorisation |

6. Digital Software

This section fulfils BS EN ISO 19650-2:2018 section 5.4.1 (f) of the Lead Appointed Party’s Digital Software.

6.1 Software Versions

The software and versions that will be used by the design teams shall be agreed before starting the project.

Table 22. Agreed software versions

| Company | Database | CAD Software | Version | Format | Comments |
| --- | --- | --- | --- | --- | --- |
| Greys Architects | Model | Revit | 2021 | RVT |  |
| Greys Architects | Model | Revizto | 2021 | NWF | Check file format |
| Westbrook | Model | Revit | 2021 | RVT |  |
| Whitestone | Model | Revit | 2021 | RVT |  |

6.2 Exchange formats

The agreed formats for the model and drawing file exchange are shown in table 23.

Table 23. Agreed Exchange formats

|  | RVT | NWF | PDF | DWG | Excel |
| --- | --- | --- | --- | --- | --- |
| Models | X | X |  |  |  |
| Drawings |  |  | X | X |  |
| Final Drawing format |  |  | X | X |  |
| Schedules or spreadsheets |  |  |  |  | X |

6.3 Federation Strategy

This section shall fulfil BS EN ISO 19650-2:2018 section 5.3.2(c).

The project information model will be produced as detailed in table 24 of this BEP and will consist of a 3D Geometric model and associated content and information. The proposed structure for the Ysgol Bae Abertawe Primary School is summarised in table 24.

A federated concept model will be developed which will shall consist of architectural and structural models that are linked via Revit using shared coordinates.

Table 24. Project Information Model

| Discipline | Model Content | Phase | Responsibility |
| --- | --- | --- | --- |
| Architectural | Proposed detail design of the school | Detailed design & as-builts | Greys Architects |
| Structural | Structural frame of the building including all beams and columns | Detailed design & as-builts | Greys Architects |

7. References

EN ISO 19650-1, Organization of information about construction works — Information management using building information modelling — Part 1: Concepts and Principles

EN ISO 19650-2, Organization of information about construction works — Information management using building information modelling — Part 2: Delivery phase of the assets

Ukbimframework.org. 2022. [online] Available at: <https://ukbimframework.org/wp-content/uploads/2020/09/Guidance-Part-F_About-information-delivery-planning_Edition-1.pdf> [Accessed 26 March 2022].