Using and programming the 7 segment display prototype circuit

Micros, Peripherals & Interfaces for Electronics (ACDA4004) (EES Version)

UWTSD

Louis S (2322688)

Contents

Introduction	4

Section 1 – Circuit Operation	4

Circuit Diagram	4

Devices & Functions	4

PIC	4

7 Segment Display	6

Crystal	7

Resistors	7

Capacitors	8

Buttons	8

Switch	8

Buzzer/Sounder	9

Transistor	9

Connector	10

Voltage regulator	11

Circuit Modules	12

Button Inputs	12

Clock Inputs	12

Sounder	13

Power Input	13

PIC16F84A & 7 Segment Display	14

Section 2 – 7 Segment Display Operation	15

Section 3 – Coding Task	18

Coding Task 1 Flow Chart	19

Coding Task 2 Flow Chart	20

Coding Task 3 Flow Chart	21

Coding Task 4 Flow Chart	22

Coding Task 5 Flow Chart	23

Bibliography	23

Appendix	24

Task 1 Code	24

Task 2 Code	27

Task 3 Code	28

Task 4 Code	29

Task 5 Code	31

# Introduction

This report contains the device and circuit operation for a PIC16F84A, programming of a 7 Segment-Display, as well as coding tasks as outlined by the report specification.

# Section 1 – Circuit Operation

## Circuit Diagram

## Devices & Functions

### PIC

The PIC is a type of microcontroller, often used in embedded systems for its relative ease of use and versatile applications. The specific PIC16F84A has 35 instructions and is able to be programmed with languages such as C or Assembly.

Below shows the block diagram for the PIC16F84A.

[1]

Port A (RA0 – 4) and Port B (RB0 - 7) are the PIC16F84A’s 13 I/O pins, meaning they can be configured as either inputs or outputs through the PIC’s loaded program. They utilise TTL (Transistor Transistor Logic) shown below in the block diagrams for RA/B0 - 3

[1]

The PIC also contains a watchdog timer which is enabled/disabled via the program loaded onto the pic, a watchdog timer is a counter that will restart the device if the countdown is not reset, to ensure that program is not stuck in an infinite loop.

The MCLR Pin, when grounded and set low, will reset the PIC, thus needs to be held high during operation.

### 7 Segment Display

The seven-segment display is made up from 7 LEDs, with the device having 8 pins; 1 for supply, and 7 for the return path for each of the LEDs for this case by being a common anode, common cathode is essentially the same but inverted, with 7 supply pins for each of the LEDs and a single return pin.

Below is an example of the internals of a Common Anode 7 Segment-Display, similar to the one used in the simulation circuit, with the addition of a Decimal Point Pin.

[2]

### Crystal

Crystals, typically quartz, are used due to their piezoelectric effect which generate a charge when mechanically pressured or vibrates when an electrical signal is applied to it. With the frequency dependant on shape and dimension of the crystal, the latter effect can be used to produce precise waveforms that can be used for timing of microcontrollers, among other things.

### Resistors

The resistors are used to limit current, and protect components from being damaged, by adding resistance to the path with the same voltage, the current passing through is lower.

I=VR

V = Voltage (V), I = Current (A), R = Resistance (Ω)

### Capacitors

Capacitors retain charge with the use of two internal conductive plates separated with a dielectric in the form of an electric potential.

They have numerous uses such as filters for DC signals; use in RC circuits to create time delays; Voltage stabilisation to smooth out voltage spikes.

### Buttons

A simple component that, in this case, creates an electrical connection when the button is depressed, returning to an open state when released with use of a spring. There are various types of buttons for differing applications, however, are similar in their operation.

### Switch

The switch used in the circuit is a SPCO (Single Pole Change Over), meaning the switch has two states with a common contact on the left and two contacts on the left that the switch can alternate between which is near identical to a SPDT (Single Pole Double Throw) switch with the only main distinction being their typical use case. Those being SPCO is typically used for more automated applications such as relays, where SPDT switches are used for manual operation like user operated switches.

### Buzzer/Sounder

The buzzer creates a tone based on the voltage and signal of the input; a physical buzzer is made up from a small electromagnet placed behind a flexible diaphragm with a ferromagnetic disk connected to it. As the electromagnet has a current passed through its coil, this attracts the ferromagnetic disk, deforming the diaphragm producing sound waves

### Transistor

The transistor also operates as both a switch and as and an amplifier, in this case the former. When there is current supplied to the base pin, current can flow from collector to emitter. The BC107 is a common NPN BJT (Bipolar Junction Transistor).

The maximum ratings for the BC107/108/109 can found below.

[3]

### Connector

Serves as a connection point for the circuit to receive power input, this specific one used is a SIL-100-02, SIL referring to Single in Line arrangement of the terminal block. Terminal blocks are used to securely connect wires often to a PCB. Typical constructions had a conductive screw above a slot, that the wire is fed into and tightened to hold it in place, pins on the underside then connect the terminal to the circuit.

### Voltage regulator

This Specific Voltage regulator (LM7805) Supplies the circuit with a stable and regulated +5V, denoted by the ‘05’ from the LM78xx series of linear voltage regulators. The regulator takes in a higher voltage than its output, typically at least +2V above, to output a smoother 5V.

This specific regulator also uses a combination of internal current limiting, thermal shutdown, and safe-area compensation to prevent any potential damage to the component, which essentially mean the IC’s circuit has a ‘sensor’ to limit power dissipation

Below shows the block diagram for the LM7800 series of voltage regulators

[4]

For more information regarding operation, see the power input module

## Circuit Modules

### Button Inputs

The above module operates as the main input for the overall circuit, when the either of the buttons are pressed, they bring their respective PIC pins low, acting as an input for the PIC to detect. The pull up resistors are used to give definitive logic high (+5V) when the buttons aren’t being pressed.

### Clock Inputs

The External clock module dictates and synchronises the PIC’s operations and the time it takes to execute instructions based on the clock speed. The capacitors are used in conjunction with the crystal to allow it to operate at its desired frequency.

### Sounder

The sounder module is driven by the power input module but is operated based on the state of the transistor switching, which itself is driven via an output from the PIC. When current is supplied to the base of the transistor, current can flow from the collector to the emitter, allowing the sounder to produce output in the form of noise. Both resistors in this module are acting as current limiters for both the sounder and transistor.

### Power Input

The circuit is powered by from the input terminal block leading to a relay which allows switching of the ‘power input’ by an external lower power circuit. The relay then feeds into a voltage regulator which provides a stable 5v for the rest of the circuit. Decoupling capacitors are used either side of the voltage regulator to reduce any noise or fluctuations from the supply.

Below shows the basic implementation of the regulator as per the datasheet

[4]

### PIC16F84A & 7 Segment Display

The PIC has its MCLR pin (Master Clear) held high to avoid accidental resets during circuit operation.

The PIC is also connected to the common anode 7 Segment-Display via the RB0 – 6 pins with current limiting resistors to protect the LEDs of the display from potential over current.

In this circuit, the RA0 and RA1 pins are configured in software to function as inputs, to detect change in the buttons.

OSC1/CLKIN and OSC2/CLKOUT are connected to the Clock module that are acting as the PIC’s external oscillator

RB7 is configured to function as an output that leads to the base of the transistor detailed in the sounder module

# Section 2 – 7 Segment Display Operation

To have the seven-segment display light correctly, each the pins require a connection to ground due the seven-segment display being a common anode.

Each pin has a ‘value’ that the total dictates the LED’s lit, and due to the 7 Segment-Display being common anode, the total value equates to those component value pins being unlit. For example, to display a zero, the middle LED needs to be off, thus the value would equal 64. Running this through the PIC results in all but the centre LED being on.

| 7 Segment | Example | Decimal Value | Code |
| --- | --- | --- | --- |
| 0 |  | 64 | 0x40 |
| 1 |  | 121 | 0x79 |
| 2 |  | 36 | 0x24 |
| 3 |  | 48 | 0x30 |
| 4 |  | 25 | 0x19 |
| 5 |  | 18 | 0x12 |
| 6 |  | 2 | 0x02 |
| 7 |  | 120 | 0x78 |
| 8 |  | 0 | 0x00 |
| 9 |  | 16 | 0x10 |
| A |  | 8 | 0x08 |
| B |  | 3 | 0x03 |
| C |  | 70 | 0x46 |
| D |  | 33 | 0x21 |
| E |  | 6 | 0x06 |
| F |  | 14 | 0x0e |

# Section 3 – Coding Task

The table below outlines the coding tasks and flowcharts created, for the code in its entirety, see Appendix

| Task | Flowchart | Code |
| --- | --- | --- |
| 1 | Completed | Completed |
| 2 | Completed | Completed |
| 3 | Completed | Completed |
| 4 | Completed | Non-Functional |
| 5 | Completed | Completed |
| 6 | Not Attempted | Not Attempted |

## Coding Task 1 Flow Chart

## Coding Task 2 Flow Chart

## Coding Task 3 Flow Chart

## Coding Task 4 Flow Chart

## Coding Task 5 Flow Chart

# Bibliography

| [1] | Microchip Technology, “PIC16F84A Data Sheet,” Microchip Technology Inc, 2001. [Online]. Available: https://ww1.microchip.com/downloads/en/devicedoc/35007b.pdf. [Accessed 13 January 2025]. |
| --- | --- |
| [2] | Cypress Semiconductor Corporation, “7-Segment LED Controller Datasheet,” Cypress Perform, 2 July 2014. [Online]. Available: https://www.infineon.com/dgdl/Infineon-LED7SEG_User_Module-Software%20Module%20Datasheets-v01_02-EN.pdf?fileId=8ac78c8c7d0d8da4017d0fa9f7d51aaf. [Accessed 13 January 2025]. |
| [3] | Farnell, “BC107/ BC108/ BC109 Low Power Bipolar Transistors,” Multicomp, 30 May 2005. [Online]. Available: https://www.farnell.com/datasheets/314715.pdf. [Accessed 13 January 2025]. |
| [4] | Texas Instruments, “LM340, LM340A and LM7805 Family Wide VIN 1.5-A Fixed Voltage Regulators,” Texas Instruments, 2016. [Online]. Available: https://www.ti.com/lit/ds/symlink/lm7800.pdf. [Accessed 13 January 2025]. |

# Appendix

## Task 1 Code

list    p=16F84     ; select processor type

radix   hex         ; define radix as hexadecimal

;----------------------------------------------------------------------

portB   equ 0x06        ;port B address

trisb   equ     0x06        ;trisB address in register bank 1

status  equ     0x03        ;status register

RP0 equ     5           ;bit 5 of status register

RP1     equ 6           ;bit 6 of status register

count   equ 0x0c        ; counter 0 to 255

nCount  equ 0x0d        ; pause routine inner loop counter

mCount  equ 0x0e        ; pause routine outer loop counter

;----------------------------------------------------------------------

org     0x000

start

bcf status,RP0  ; clear bank selection bits in the status reg

bcf status,RP1

bsf status,RP0  ; select bank 1

movlw   0x00        ; load w with 0x00

movwf   trisb       ; load it into trisb to make portb all outputs

bcf status,RP0  ; reselect bank 0

clrf    portB       ; clear all lines low

clrf    count       ; initialize counter to 0

get_count

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x40        ; load w with 0x40 (0)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x79        ; load w with 0x79 (1)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x24        ; load w with 0x24 (2)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x30        ; load w with 0x30 (3)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x19        ; load w with 0x19 (4)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x12        ; load w with 0x12 (5)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x02        ; load w with 0x02 (6)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x78        ; load w with 0x78 (7)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x00        ; load w with 0x00 (8)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x10        ; load w with 0x10 (9)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x08        ; load w with 0x08 (A)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x03        ; load w with 0x03 (B)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x46        ; load w with 0x46 (C)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x21        ; load w with 0x21 (D)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x06        ; load w with 0x06 (E)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

movf    count, w    ; move count to W

movwf   portB       ; move W to port B to update the display

movlw   0x0e        ; load w with 0x0e (F)

movwf   count       ; save W in count

call    pause       ; delay 200ms by subroutine

call    pause       ; five pause executions equals ~ 1 second

goto    get_count   ; repeat forever

pause

movlw   0xff        ; set w = 255 decimal

movwf   mCount      ; mCount = w: outer loop counter

loadN

movlw   0xff        ; set w = 255 decimal

movwf   nCount      ; nCount = w: inner loop counter

decN

decfsz  nCount, f   ; nCount--

goto    decN        ; if nCount != 0 then repeat nCount--

decfsz  mCount, f   ; else decrement Count

goto    loadN       ; if mCount != 0 then

; reload nCount to 255 and decrement

return              ; else exit subroutine

end

## Task 2 Code

list    p=16F84     ; select processor type

radix   hex     ; define radix as hexadecimal

;----------------------------------------------------------------------

portA   equ 0x05

portB   equ 0x06        ;port B address

trisb   equ     0x06        ;trisB address in register bank 1

status  equ     0x03        ;status register

RP0 equ     5       ;bit 5 of status register

RP1     equ 6       ;bit 6 of status register

count   equ 0x0c        ; counter 0 to 255

nCount  equ 0x0d        ; pause routine inner loop counter

mCount  equ 0x0e        ; pause routine outer loop counter

;----------------------------------------------------------------------

org     0x000

start

bcf status,RP0  ; clear bank selection bits in the status reg

bcf status,RP1

bsf status,RP0  ; select bank 1

movlw   0x00        ; load w with 0x00

movwf   trisb       ; load it into trisb to make portb all outputs

bcf status,RP0  ; reselect bank 0

clrf    portB       ; clear all lines low

clrf    count       ; initialize counter to 0

buttons

btfsc   portA, 0

goto    segL        ; goto the segL Subroutine

movf    count, w

movlw   0x4e        ; Displays R on the 7 Segment display

movwf   portB

movwf   count

call    pause       ; Calls the Pause Subroutine

segL

btfsc   portA, 1

goto    buttons     ; loop back to buttons subroutine

movf    count, w

movlw   0x47        ; Displays L on the 7 Segment display

movwf   portB

movwf   count

call    pause       ; Calls the Pause Subroutine

goto    buttons     ; loop back to buttons subroutine

pause

movlw   0xff        ; set w = 255 decimal

movwf   mCount      ; mCount = w: outer loop counter

loadN

movlw   0xff        ; set w = 255 decimal

movwf   nCount      ; nCount = w: inner loop counter

decN

decfsz  nCount, f   ; nCount--

goto    decN        ; if nCount != 0 then repeat nCount--

decfsz  mCount, f   ; else decrement Count

goto    loadN       ; if mCount != 0 then

; reload nCount to 255 and decrement

return          ; else exit subroutine

end

## Task 3 Code

list    p=16F84     ; select processor type

radix   hex         ; define radix as hexadecimal

;----------------------------------------------------------------------

portA   equ 0x05

portB   equ 0x06        ;port B address

trisb   equ     0x06        ;trisB address in register bank 1

status  equ     0x03        ;status register

RP0 equ     5           ;bit 5 of status register

RP1     equ 6           ;bit 6 of status register

count   equ 0x0c        ; counter 0 to 255

nCount  equ 0x0d        ; pause routine inner loop counter

mCount  equ 0x0e        ; pause routine outer loop counter

;----------------------------------------------------------------------

org     0x000

start

bcf status,RP0  ; clear bank selection bits in the status reg

bcf status,RP1

bsf status,RP0  ; select bank 1

movlw   0x00        ; load w with 0x00

movwf   trisb       ; load it into trisb to make portb all outputs

bcf status,RP0  ; reselect bank 0

clrf    portB       ; clear all lines low

clrf    count       ; initialize counter to 0

all

btfsc   portA, 0

goto    none        ; goto the none Subroutine

movf    count, w

movlw   0x00        ; Displays all LEDs on the 7 Segment display

movwf   portB

movwf   count

call    pause       ; Calls the Pause Subroutine

none

btfsc   portA, 1

goto    all     ; loop back to all subroutine

movf    count, w

movlw   0xff        ; Displays no LEDs on the 7 Segment display

movwf   portB

movwf   count

call    pause       ; Calls the Pause Subroutine

goto    all     ; loop back to all subroutine

pause

movlw   0xff        ; set w = 255 decimal

movwf   mCount      ; mCount = w: outer loop counter

loadN

movlw   0xff        ; set w = 255 decimal

movwf   nCount      ; nCount = w: inner loop counter

decN

decfsz  nCount, f   ; nCount--

goto    decN        ; if nCount != 0 then repeat nCount--

decfsz  mCount, f   ; else decrement Count

goto    loadN       ; if mCount != 0 then

; reload nCount to 255 and decrement

return              ; else exit subroutine

end

## Task 4 Code

list    p=16F84     ; select processor type

radix   hex         ; define radix as hexadecimal

;----------------------------------------------------------------------

portA   equ 0x05

portB   equ 0x06        ;port B address

trisb   equ     0x06        ;trisB address in register bank 1

status  equ     0x03        ;status register

RP0 equ     5           ;bit 5 of status register

RP1     equ 6           ;bit 6 of status register

count   equ 0x0c        ; counter 0 to 255

nCount  equ 0x0d        ; pause routine inner loop counter

mCount  equ 0x0e        ; pause routine outer loop counter

;----------------------------------------------------------------------

org    0x000

start

bcf status,RP0  ; clear bank selection bits in the status reg

bcf status,RP1

bsf status,RP0  ; select bank 1

movlw   0x12        ; load w with 0x00

movwf   trisb       ; load it into trisb to make portb all outputs

bcf status,RP0  ; reselect bank 0

clrf    portB       ; clear all lines low

movlw   count       ; initialize counter to 0

Table

retlw   0x40    ; 0 on 7 Segement Display

retlw   0x79    ; 1 on 7 Segement Display

retlw   0x24    ; 2 on 7 Segement Display

retlw   0x30    ; 3 on 7 Segement Display

retlw   0x19    ; 4 on 7 Segement Display

retlw   0x12    ; 5 on 7 Segement Display

retlw   0x02    ; 6 on 7 Segement Display

retlw   0x78    ; 7 on 7 Segement Display

retlw   0x00    ; 8 on 7 Segement Display

retlw   0x10    ; 9 on 7 Segement Display

IncrementR

btfsc   portA, 0

goto    DecrementL      ; goto the segL Subroutine

movf    count, w

; Displays R on the 7 Segment display

movwf   portB

movwf   count

call    pause       ; Calls the Pause Subroutine

movf    count, w

; Displays R on the 7 Segment display

movwf   portB

movwf   count

call    pause

DecrementL

btfsc   portA, 1

goto    IncrementR      ; loop back to buttons subroutine

movf    count, w

; Displays L on the 7 Segment display

movwf   portB

movwf   count

call    pause

movf    count, w

; Displays L on the 7 Segment display

movwf   portB

movwf   count

call    pause       ; Calls the Pause Subroutine

goto    IncrementR      ; loop back to buttons subroutine

pause

movlw   0x07        ; set w = 255 decimal

movwf   mCount      ; mCount = w: outer loop counter

loadN

movlw   0xc8        ; set w = 255 decimal

movwf   nCount      ; nCount = w: inner loop counter

decN

decfsz  nCount, f   ; nCount--

goto    decN        ; if nCount != 0 then repeat nCount--

decfsz  mCount, f   ; else decrement Count

goto    loadN       ; if mCount != 0 then

; reload nCount to 255 and decrement

return          ; else exit subroutine

end

## Task 5 Code

list    p=16F84     ; select processor type

radix   hex         ; define radix as hexadecimal

;----------------------------------------------------------------------

portA   equ 0x05

portB   equ 0x06        ;port B address

trisb   equ     0x06        ;trisB address in register bank 1

status  equ     0x03        ;status register

RP0 equ     5           ;bit 5 of status register

RP1     equ 6           ;bit 6 of status register

count   equ 0x0c        ; counter 0 to 255

nCount  equ 0x0d        ; pause routine inner loop counter

mCount  equ 0x0e        ; pause routine outer loop counter

;----------------------------------------------------------------------

org    0x000

start

bcf status,RP0  ; clear bank selection bits in the status reg

bcf status,RP1

bsf status,RP0  ; select bank 1

movlw   0x00        ; load w with 0x00

movwf   trisb       ; load it into trisb to make portb all outputs

bcf status,RP0  ; reselect bank 0

clrf    portB       ; clear all lines low

clrf    count       ; initialize counter to 0

ToneR

btfsc   portA, 0

goto    ToneL       ; goto the ToneL Subroutine

movf    count, w

movlw   0xff        ; Switches Transistor High

movwf   portB

movwf   count

call    pause       ; Calls the Pause Subroutine

movf    count, w

movlw   0x7f        ; Switches Transistor low

movwf   portB

movwf   count

call    pause

ToneL

btfsc   portA, 1

goto    ToneR       ; loop back to buttons subroutine

movf    count, w

movlw   0xff        ; Switches Transistor High

movwf   portB

movwf   count

call    pause

movf    count, w

movlw   0x7f        ; Switches Transistor low

movwf   portB

movwf   count

call    pause       ; Calls the Pause Subroutine

goto    ToneR       ; loop back to buttons subroutine

pause

movlw   0x07        ; set w = 255 decimal

movwf   mCount      ; mCount = w: outer loop counter

loadN

movlw   0xc8        ; set w = 255 decimal

movwf   nCount      ; nCount = w: inner loop counter

decN

decfsz  nCount, f   ; nCount--

goto    decN        ; if nCount != 0 then repeat nCount--

decfsz  mCount, f   ; else decrement Count

goto    loadN       ; if mCount != 0 then

; reload nCount to 255 and decrement

return          ; else exit subroutine

end