Lab Report – Calorimetry

Tegan Ingram

University of Wales Trinity Saint David

Ordinance, Munitions and Explosives Engineering

Supervisor:  Dr Ewa Kazimierska

April 2024

Table of Contents

List of Tables	2

Introduction	2

Results	2

Discussion	5

Conclusion	6

Bibliography	6

Appendix	6

# List of Tables

Table 1	4

# Introduction

Calorimetry is used to determine whether a reaction is exothermic or endothermic by measuring enthalpy change, to do this the heat absorbed or released during a reaction is recorded. According to the first law of thermodynamics heat energy cannot be created nor destroyed, but it can be transferred from one location to another or turned into another form of energy [1].During this experiment the heat capacity of the calorimeter is calculated and then the specific heat capacity of a metal is measured. A hypothesis for the part when calculating the heat capacity of the calorimeter is- temperature will increase by half of ΔThot. For the first experiment- dependent variable: ΔTcalorimeter, independent variable: the volume of water, control variable: surrounding temperature, thermometer used, and frequency stirred per 10 seconds. A reasonable hypothesis for experiment 2 is that the metal will heat the water by between 4.5°C and 8°C. When finding the specific heat capacity of the metal the independent variable: time, the dependent variable: temperature, control variable: surrounding temperature, the time intervals, the insulation, and thermometer used. To calculate the heat capacity of the calorimeter the equation: –Qhotwater = Qcoldwater + Qcalorimeter is used and thus the equation Qcalorimeter = HCcalorimeter x ΔTcalorimeter can be used to determine the heat capacity. To work out the specific heat capacity for the metal the equation Mmetal x Cp metal x ΔTmetal = -Mwater x Cp water x ΔTwater +                                                                    HCcalorimeter x ΔTcalorimetry

# Results

Measuring the heat capacity of the calorimeter:

Thot = ____60°C______            mhot water = ____50g_______       ΔThot water = ____-15°C______

Tcold = ____21.5°C_____         mcold-water = ____50g_____       ΔTcold water = ____23.5°C______

Tmax = _____45°C______                                                        ΔTcalorimeter = ____23.5°C______

Qcold-water = ____4.92KJ____       Qhot water = _____-3.14KJ_____        Qcalorimeter = ____-1.78KJ_____

HCcalorimeter = ____70.36J/°C____

Thot = ____60°C______            mhot water = ____50g_______       ΔThot water = ____-15°C______

The calculations used to obtain these results are as follows:

Qhw = 50g x 4.186J/g-°C x -15°C =	-3139.5J =	-3.14KJ

Qcw =50g x 4.186J/g-°C x 23°C = 	4918.6J = 	4.92KJ

-Qhw = Qcw + Qcalorimeter

So: Qcal = -Qhw – Qcw

= - (-3.13) – 4.92 = 	-1.78KJ

To calculate the HC of the calorimeter: HCcalorimeter = Qcalorimeter/ΔTcalorimeter

= -1.78/25.3 =	 -0.070356KJ/°C

Measuring the heat capacity of the metal:

| Time (s) | Temperature Water (°C) |
| --- | --- |
| 10 | 26 |
| 20 | 27 |
| 30 | 28 |
| 40 | 28.5 |
| 50 | 29 |
| 60 | 28.5 |
| 70 | 28 |
| 80 | 28 |
| 90 | 28 |
| 100 | 28 |
| 110 | 28 |
| 120 | 28 |
| 130 | 27.5 |
| 140 | 27.5 |
| 150 | 27.5 |
| 160 | 27.5 |
| 170 | 27.5 |
| 180 | 27.5 |
| 190 | 27.5 |
| 200 | 27.5 |
| 210 | 27 |

Table 1

Thot = ___78°C   ___        mmetal = ____18.6g____             ΔTmetal  = ____-49°C_____

Tcold = ____21.5°C___       mwater = _____30g______              ΔTwater  = _____7.5°C______

Tmax = ____29°C____                                                           cp metal  =           1.024J/g-°C        .

These results can be obtained using the following formulae and equations:

q(metal) = mass  T  Cp =

=18.6g  -49°C  Cp metal

= -911.4   Cp metal J

q(water) = mass  T  Cp =

= 30 g  7.5°C  4.148 J/g- °C =

= 933.3 J

Due to first law of thermodynamics it is said: q (system) = - q (surroundings)

Thus: q(metal) = q(water)

-911.4   Cp metal J = -933.3 J

Therefore Cp metal = -933.3/-911.4

Cp Metal = 	1.024J

# Discussion

This experiment is simple, however there are limitations. Having a lid helps keep the heat in however the equipment provided was not a perfect fit and meant that heat could escape through the holes used for the thermometer and stirring rod. Using a more secure lid would ensure greater accuracy. For example during an experiment using a bomb calorimeter the lid is “secured by 12 1-cm-diam stainless steel bolts inserted through the lid flange and sealed with a Heoprene O-ring”. [2] In this case the bolts hold the lid on and the O-rings stop any air from escaping through small gaps. One Styrofoam cup inside another will provide some insulation however not a great deal, a bomb calorimeter has measures in place to prevent heat escaping. “The vessel is enclosed by a “jacket” which is separated from the vessel by an air space about 1 cm thick…kept at a constant temperature by means of a thermostat”, [3] this is a much more effective way of ensuring that extra heat is not lost to surroundings and that the surrounding temp is controlled. Although a bomb calorimeter generates much more reliable data, it is much more costly so would only be beneficial for specialists. In order to improve the simplistic design a smaller Styrofoam cup placed inside a relatively larger cup could be used (this would replicate the “jacket” that is seen in a bomb calorimeter), a lid could be placed on both cups, this limits the amount of heat that is lost to the surroundings.

One use that calorimetry has in OME is that “single, double and triple based propellants, containing nitrate esters, will undergo thermal decomposition, even at normal magazine storage temperatures, and this process can be observed directly by heat conduction calorimetry” [4] this is very useful for manufacturers and buyers as the life expectancy of the munition is known and thus can be stored accordingly. It is also a much quicker way of testing than actually storing the explosives for however many years it may take for thermal decomposition to start. “Fundamental information about the detonation process and its chemistry can be obtained by combining calorimetric measurements with product analysis of appropriately designed experiments”[5]. It is vital when handling and manufacturing OME that the detonation heat is known for not only safety but also efficiency. By using calorimetry to measure this a result within a very small degree of accuracy can be produced.

# Conclusion

From the results it can be said that one hypothesis was correct- the metal did warm the water by between 4.5°C and 8°C. The other hypothesis was incorrect, temperature increased by 23°C, for this hypothesis to have been correct temperature would have had to have increased by 30°C. The overall outcome of discovering the specific heat capacity of the metal was achieved with some degree of accuracy.

# Bibliography

[1] S. Singh, First Law of Thermodynamics – Equation, Statements, Examples [Online]. Available: First Law Of Thermodynamics - Equation, Statement, Examples (byjus.com)

[2] D. L. Orneleas, “Calorimetric determination of the heat and products of detonation of an Unusual CHNOFS Explosive”, Propellants, Explosives, Pyrotechnics, vol. 14, 3, pp. 122-123, June 1989.

[3] R. S. Jessup, Precise Measurement of Heat of Combustion with a Bomb Calorimeter. USA: Nation Institute of Standards and Technology, 1960.

[4] P. Bunyan, C. Baker, and N. Turner, “Application of heat conduction calorimetry to high explosives,” Thermochimica Acta, vol. 401, no. 1, pp. 9–16, May 2003

[5] D. L. Orneleas, “Calorimetric determination of the heat and products of detonation of an Unusual CHNOFS Explosive”, Propellants, Explosives, Pyrotechnics, vol. 14, 3, pp. 122-123, June 1989.

# Appendix

Equipment for experiment 1: 100ml of water, two Styrofoam cups, a lid with thermometer and stirrer hole, thermometer, stirring rod, beaker, Bunsen burner, correct apparatus for Bunsen burner ignition (lighter, gas tap), tripod, gauze, heat mat, measuring cylinder.

Equipment for experiment 2:  Metal sample, around 60ml of water, Bunsen burner, correct apparatus for Bunsen burner ignition (lighter, gas tap), heat mat, tripod, gauze, beaker, two Styrofoam cups, lid with thermometer and stirrer hole, tongs, measuring cylinder.

Experiment 1 – Measure the value of the heat capacity of the calorimeter:

- Place one Styrofoam cup into the other and add 50 ml of water to the top cup, close the lid and add thermometer and stirrer.

- Put the other 50ml of water into a beaker and heat using the Bunsen burner. Heat to roughly 80°C, turn off Bunsen and let temperature reach around 60°C.

- Record the temperature of both the water in the Styrofoam cup (T-cold) and the water in the beaker (T-hot).

- Add the hot water to the Styrofoam cup with the cold water, put lid on, stir, and watch temperature.

- Record the highest temperature that the mixture reaches before it starts to cool again (T-max).

Experiment 2 – Measure the specific heat capacity of a metal:

- Weigh the metal sample (M-metal).

- Place enough water in a beaker so that when then metal is placed in it is fully submerged.

- Heat the water in the beaker (without the metal in it) until it is boiling, then add the metal.

- Boil for another 5 minutes, then turn off heat for 2 minutes, (this allows the metal to reach the same temp as the water).

- Place two Styrofoam cups together and measure 30ml of water into the top one, record the temp of the water (T-cold).

- After 2 minutes record the temperature of the water (T-hot).

- Quickly add the metal to the Styrofoam cup using the tongs.

- Stir and record the temperature.

- Record the temperature ever ten seconds.

- Stop recording once the temperature has reached a maximum point and then cooled 2°C below that maximum point.