| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| January 2024 | 2024 |

MODULE CODE: ACDA4006

Electrical and Electronic Principles

TIME:  2 HOURS

Maximum Marks : 100

answer Any 4 questions

Equipment or materials required – Calculator, Graph sheet,

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

ANSWER ANY 4 QUESTIONS

Question 1 [25 Marks]

- An indicator light requires a 330Ω resistor to limit current. The voltage across the current limiting resistor is 3V. What is current flowing through the resistor? Also, calculate how the current change if a 270Ω resistor is used instead?											[4 Marks]

Solution:

Given, Voltage = 3V

Resistor = 330Ω

Current flowing through the resistor = VR=3330=0.0091A			[2 Marks]

When the resistor is 270Ω,  Current  =VR=3270=0.0111A			[2 Marks]

- A solar yard light has a solar collector that can provide 1.0W of power for charging the 3.0V batteries. What is the maximum charging current the solar collector could supply to the fully discharged 3.0V batteries?		[2 Marks]

Solution:

I=PV=13=0.33V 								[2 Marks]

- Design an equivalent circuit with a single voltage source, for the circuit shown in fig.1,										[7 Marks]

Fig: 1

Solution:

Convert the circuit into an equivalent current source circuit.

I1=V1R1=63=2A								[1 Mark]

I2=V2R2=186=3A								[1 Mark]

Simplifying the circuit.

I=I2-I1=3-1=1A 							[1 Mark]

R=3×63+6=189=2Ω 								[1 Mark]

Equivalent circuit

V=IR=2×1=2V							[1 Mark]

[2 Marks]

- For the circuit shown in fig.2, calculate the following.

- Equivalent Resistance,						[6 Marks]

- Current flowing through 20Ω Resistor				[2 Marks]

Fig:2

Solution:

- Equivalent Resistance

R4+R3 in parallel with R5

Req1=(R3+R4)×R5R3+R4+R5=30+50×5030+50+50=30.369Ω					[2 Marks]

Req1+R2 in parallel with R6

Req2=(Req1+R2)×R6Req1+R2+R6=30.369+25×5030.369+25+50=26.274Ω				[2 Marks]

Req2 is series with R1

Req=R1+Req2=20+26.264=46.264Ω 				[2 Marks]

- Current flowing through 20Ω Resistor

I=VReq=1546.264=0.324A							[2 Marks]

- A certain power supply requires 25W of input power and produces an output power of 20W. Calculate its efficiency in percentage and the total power loss?											[4 Marks]

Solution:

Efficiency =POUTPIN= 2020=0.8							[1 mark]

% Efficiency = 0.8×100=80%						[1 mark]

Total Power Loss PLoss=PIN-POUT=25-20=5W			[2 marks]

Question 2 [25 Marks]

- Determine the voltage across each resistor in the circuit shown in fig.3.

[3 Marks]

Fig.3

Solution:

As all the resistors are in parallel, the Voltage across the resistor is the same.

V1=V2=V3=V4=V5=10V							[3 Marks]

- Three resistors (RA RB and Rc)are connected in series to a 250V Voltage Source. Where, Rc=50Ω, VB=80V and Total current = 2A. Calculate the following.

- Voltage across RA 							[2 Marks]

- Voltage across RC 							[2 Marks]

- Resistance value of RA and RB						[4 Marks]

Solution:

- Voltage across RA

V=VA+VB+VC

250=VA+80+100

VA=250-180=70V							[2 marks]

- Voltage across RC

VC=I×Rc=2×50=100V						[2 Marks]

- Resistance value of RA and RB

RB=VBI=802=40Ω								[2 Marks]

RA=VAI=702=35Ω								[2 Marks]

- Define the following terms

- Resistance								[2 Marks]

- Impedance								[2 Marks]

- Reactance								[2 Marks]

- Doping									[2 Marks]

- Voltage Rating								[2 Marks]

Solution:

- Resistance

Opposition to flow of current. Unit is Ohm - DC circuit property		[2 Marks]

- Impedance

Opposition to change of flow of current. Unit is Ohm- AC circuit property 	[2 Marks]

- Reactance

Vector sum of resistance and reactance – Describes total opposition to current – AC and DC circuit property.								[2 Marks]

- Doping

It is process of adding impurities to the pure form of Semiconductor.		[2 Marks]

- Voltage Rating

Maximum voltage an electrical or electronic appliances can withstand for a duration of time.										[2 Marks]

- Differentiate Step- up Transformer and Step-Down Transformer (minimum of two differences)								[4 Marks]

Solution:

| Sl. No | Step-up Transformer | Step-down Transformer |
| --- | --- | --- |
| 1 | Input voltage is lower than the output voltage | Input voltage is higher than the output voltage |
| 2 | Number of Secondary winding is higher than the primary winding | Number of Secondary winding is less than the primary winding |
| 3 |  |  |

[Each difference two marks, Maximum of 4 marks]

Question 3 [25 Marks]

- Determine the current across each resistor in the circuit shown in fig.4, using mesh analysis (Kirchhoff’s Voltage Law)		[17 Marks]

Fig.4

Solution:

[1 Mark]

Apply KVL in ABEFA

-5I1-3I1-I2-5-2I1+20=0 						[3 Marks]

-10I1+3I2=-15 …………(1)						[2 Marks]

Apply KVL in BCDEB

-6I2-7I2-2I2+5+3I1-I2=0 						[3 Marks]

-3I1-18I2=-5 …………(2)							[2 Marks]

Simplifying Equation (1) and (2)

I1=1.67A									[2 Marks]

Substitute, I1 in equation (1)

-10 1.67+3I2=-15

-16.7+3I2=-15

3I2=-15+16.7

I2=1.73=0.567A									[2 Marks]

Current across branch BE

I1-I2=1.67-0.567=1.1A							[1 Mark]

Current Across R1and R3=I1=1.67A

Current Across R2=I1-I2=1.1A

Current Across R4 R5 and R6=I2=0.567A					[1 Mark]

- In the RC circuit shown in fig 5. Calculate the following.

- Capacitive Reactance (XC)		[3 Marks]

- Impedance (Z)		[3 Marks]

- Current across the circuit.		[2 Marks]

Fig. 5

Solution:

- Capacitive Reactance (XC)

XC=12πfC=12π×1.5×1000×0.022×10-6						[2 Marks]

XC=4.82KΩ									[1 Mark]

- Impedance (Z)

Z=R2+XC2=(2.2×1000)2+(4.82×1000)2				[2 Marks]

Z=5.30KΩ									[1 Mark]

- Current across the circuit.

I=VZ=105.30×1000									[1 Mark]

I=1.89mA									[1 Mark]

Question 4 [25 Marks]

- The alternating current is given by i=40 sin314t. Determine the following

- Maximum Value of current		[1 Mark]

- RMS Value of Current		[2 Marks]

- Frequency		[2 Marks]

Solution:

- Maximum Value of current

From the given equation, Vm=40V						[1 Mark]

- RMS Value of Current

VRMS=Vm2=402									[1 Mark]

VRMS=28.284V									[1 Mark]

- Frequency

From the equation, ω=314

We know that, ω=2πf

314=2πf 									[1 Mark]

f=3142π=50Hz									[1 Mark]

- Differentiate P-type and N-Type Semiconductor.		[4 Marks]

Solution:

| Sl.No | N type semiconductor | P type semiconductor |
| --- | --- | --- |
| 1 | Pentavalent impurities are added | Trivalent impurities are added |
| 2 | Doping agent: Arsenic, Antimony | Doping agent: Gallium, indium |
| 3 | Electrons- Majority charge carriers Holes- minority charge carriers | Electrons- minority charge carriers Holes- majority charge carriers |

[Each difference 2 marks, Maximum of 4 marks]

- List any four types of Transistors.		[4 marks]

Solution:

- BJT (Bipolar Junction Transistor) – NPN

- BJT (Bipolar Junction Transistor) – PNP

- N-Channel JFET (Junction Field Effect Transistor)

- P-Channel JFET (Junction Field Effect Transistor)

- N-Channel MOSFET (Metal Oxide Semiconductor Field Effect Transistor)

- P-Channel MOSFET (Metal Oxide Semiconductor Field Effect Transistor)

[1 mark each, Maximum of 4 Marks]

- What is the use of the circuit shown in fig.6?  Describe the functions of each stage of the circuit with the necessary diagrams.		[12 marks]

Fig.6

Solution:

The Circuit shown in Fig 6 is a Bridge Rectifier circuit, It is used to convert AC to DC.	[2 Marks]

Transformer:

The input to the bridge rectifier is  provided by a transformer. The transformer steps down the voltage from the mains to a suitable level.		[1 Mark]

Diodes (4 Diodes):

A bridge rectifier consists of four diodes arranged in a specific configuration.					[1 Mark]

Working Principle:

AC Input:

The alternating current (AC) input from the transformer is applied to the four diodes in the bridge rectifier circuit.		[1 Mark]

Diode Operation:

Depending on the polarity of the input AC voltage, two diodes conduct during each half-cycle.		[1 Mark]

When the voltage is positive, Diodes D2 and D4 conduct, and when the voltage is negative, Diodes D1 and D3 conduct.	[3 Marks]

Output Voltage:

As a result, the output is a pulsating DC voltage that maintains the same polarity regardless of the input AC voltage.	[2 Marks]

Waveform:

[1 Mark]

Question 5 [25 Marks]

- A 12 V Battery has an internal Resistance of 12Ω, Calculate the power dissipated by various load resistances given in the below table and complete the table.		[15 Marks]

| Load Resistance (Ω) | Load power (W) |
| --- | --- |
| 4 |  |
| 8 |  |
| 12 |  |
| 16 |  |
| 20 |  |

- Plot a graph of the load Power against the load resistance,		[8 Marks]

Solution:

| Load Resistance (Ω) | Load power (W) |
| --- | --- |
| 4 | 2.25 |
| 8 | 2.88 |
| 12 | 3 |
| 16 | 2.938 |
| 20 | 2.813 |

[3 marks for each, maximum of 15 marks]

- Plot a graph of the load Power against the load resistance,

[labelling X axis, Labelling Y axis, title, scale – 1 Mark each, Plotting- 4 Marks]

- List any two applications of Transistor.		[2 Marks]

Solution:

- Transistor as an amplifier							[1 Mark]

- Transistor as switch								[1 Mark]

Diwedd/End