| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2018 | mai 2018 |

MODULE CODE: acda4002

Electrical and Electronic Principles

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator

Adnoddau neu ddefnyddiau sydd eu hangen - Dim

ANSWER ALL QUESTIONS

- For the circuit shown below assume Vs = 50V

- Calculate the total circuit resistance ( RT ).

[2 marks]

- Calculate the supply current ( Is)

[1 mark]

- Calculate the total power dissipated.

[2 marks]

Oscilloscope settings:

2V/div

10ms/div

An oscilloscope display shows the above voltage waveform. The oscilloscope settings are shown in the box next to the waveform.

- Estimate the period of the waveform.

[1 mark]

- From the estimated period, determine its frequency.

[1 mark]

- Determine its peak-peak amplitude

[1 mark]

- Determine the r.m.s. value of the waveform.

[2 marks]

- For a series RLC circuit where R=250Ω, L= 100mH, and C=100µF supplied from a 120V 60Hz supply,

  - calculate the voltage across:

    - The resistor

[2 Marks]

    - The capacitor

[2 Marks]

    - The inductor

[2 Marks]

  - Calculate the resonant frequency of the circuit

[2 Marks]

  - At the resonant frequency, calculate the circuit current

[2 Marks]

- Using any appropriate method, determine the current flowing in the 20Ω resistor in the circuit shown below.

[5 Marks]

- A 230 V step-down mains transformer has a 20W lamp connected to its secondary winding. The turns ratio of the transformer is 1/20.

- Sketch the circuit

[1 Mark]

- Calculate the secondary current supplied to the load

[2 Marks]

- The primary current taken from the supply.

[2 Marks]

Diwedd/End