Applied Computing

Wales Institute of Science and Art (WISA)

# Networks and Cybersecurity Essentials (ACCA4024)

Lab 7: Tools to check on Threats and Vulnerabilities

n.kaliyamurthy@uwtsd.ac.uk                                                                                                                            1

Objectives

Using Packet Capturing tools and network monitoring tools.

Tasks

1. Use Nmap to scan 2.

Required Resources

- PC with Ubuntu 16.0.4 LTS installed in a VirtualBox or VMware virtual machine. (Alternatively, Any Linux Platform)

- Internet Connection to refer the outcomes o References: https://nmap.org/

Task 1

- Login as User Cisco.

- Go to Terminal and Type the following commands and examine  o To do that we are using Nmap.

 Network Mapper, or Nmap, is an open source utility used for network discovery and security auditing. Administrators also use Nmap for monitoring hosts or managing service upgrade schedules.   Nmap determines

        - what hosts are available on a network,

        - what services are running,

        - what operating systems are running,  • What packet filters or firewalls are running?

o So, basically if you could get all the above details using Nmap, you could get few information which could be used later in performing attacks or mitigating attacks.

- Commands to execute o Step 1:

 nmap localhost

(This is a scan performed to get information about the services that are bound to the localhost)

o Step 2:

 Replace the above command with your IP Address and other hosts (if connected/ else ignore) in your network to check the outcomes.

Type the command you used to find out your host IP Address

ifconfig

Type the command you used to scan after finding your host IP Address

nmap 127.0.0.1

Record the outcomes of the above command here

o Step 3: Analyse the outputs/outcomes from Step 1 and 2 and record your reflections. (Hint: Analyse what is the difference between local host and ip address of the host, If ports displayed are different, Why is it different? etc (not limited to the hints))

Localhost is used to refer to the computer from its internal IP not from any external ips of the device. So, the ping packets don’t pass through any physical network interfaces only from port to port.

- Step 4: Why is the nmap scan only scans the first 1000 ports? What will be the command to scan all the ports?

This is an optimisation technique. It will scan the most common 1000 ports as it can take a considerable amount of time to scan every single port. A command to scan all ports would be: nmap -p0-65535

- Step 5: Scan the UDP Ports

    - nmap –sU localhost

(This is a scan performed to get information about the UDP ports and to execute this command you need root privileges. So, try)

    - sudo nmap –sU localhost

Record the outcomes of the above command here (Hint: what UDP ports could you find open through your scan)

- Step 6: From the above outcomes select any ports which is open

 sudo nmap –p137 localhost

(Change the port number 137 to any port which you found as open in the previous step (5).)

Record the outcomes of the above command here (Hint: Is the port open or closed? Why? Also, record the use of –p command)

The port is closed as it is not being used for anything.

- Step 7: Try other port related commands and record its purpose.

    - sudo nmap –pportnumber host_ip

    - sudo nmap –pportnumber_startrange-portnumber_endrange host_ip

    - sudo nmap –pportnumber,portnumber host_ip

    - not limited to the above (Do a google search, refine your search and find out other port related commands, Hint:nmap.org)

Record the outcomes of the above commands here

- sudo nmap –pportnumber host_ip:

This command brings back the port, its current state and the service on a specific IP address.

- sudo nmap –pportnumber_startrange-portnumber_endrange host_ip:

Will scan a set range of port numbers on a chosen ip.

- sudo nmap –pportnumber,portnumber host_ip:

Will scan multiple ports specified.

- Step 8: Detecting the version is a very important part of information gathering. Try to find out the version.  nmap –sV localhost

Record the outcomes of the above commands here

- Step 9: Try other commands to detect other versions and record its purpose.

      - Command to identify the Operating System version

Record the outcomes of the above commands here

      - Command to identify the Service Version

Record the outcomes of the above commands here

Learnings from tutorials so far:

Part 1 (Packet Tracer, Host Machines)

  - Build a home Network

  - Understand and configure IP Address

  - Use Ping/ Trace route Utility to check the connection status

  - Use ipconfig /all command to determine the MAC address of a Windows computer on an Ethernet network.

  - Analyze a MAC address to determine the manufacturer.

  - Analyze the Wired and Wireless Network connection.

  - Analyze Address Resolution Protocol (ARP).

  - Connect to a Wireless Router and configure the basic settings in a wireless router and add security features.

  - Use nslookup command to understand the url to ip conversion.

  - Configure basic connectivity settings in Intermediate devices (Switch/ Router).

  - Work with SBA Packet Tracer Files/ environment

  - Configure ftp, web, email, dns, ntp and aaa Servers Part 2

  - Set up a virtual lab environment

  - Troubleshooting the virtual lab environment

  - Working with virtual machines

  - Authentication, Authorization and Accounting

  - Threats and Vulnerabilities – Using NMAP