University of Wales Trinity Saint David

Motorsport Engineering

Engine Performance Characteristics

Harri Evans

2500728

22/03/2025

Powertrain Technology - ACHA4029

Word Count - 1840

Contents Page

1.Introduction…………………………………………………………………3

2.Testing Equipment and Method…………………………………………….4

3. Spreadsheet Calculations and Processed Results………………………...5-6

4. Graphical Analysis……………………………………………………….7-9

5. Conclusion……………………………………………………..……….…10

6. Evaluation………………………………………………………………….11

References……………………………………………………………………12

1.Introduction

In this assignment I will evaluate the performance characteristics of a Honda engine using data from an engine dynamometer. Using the data gathered from the test I can calculate corrected power, torque, BMEP, SFC and overall thermal efficiency, these results are then plotted in graphs which are analysed to identify how the engines performance changed with speed and why the change occurred. These calculations are important because torque shows the turning force from the engine, brake power shows the rate at which work is done, BMEP allows comparison between different engine sizes, SFC indicates fuel economy, thermal efficiency shows how effectively the engine converts chemical energy into power output.

Engine dynamometer testing is a fundamental way of testing in automotive engineering due to the controlled and reliable environment that allows us to assess how effectively an engine turns chemical energy in fuel into mechanical energy through measuring the output power and torque whilst under load.

I will also draw conclusions form my test results and write a evaluation on how I feel I have performed to complete the tasks set and what I could have improved to validate the results and the investigation as a whole.

2.Testing Equipment and Method

The test was conducted using a 1.6L Honda engine and a engine dynamometer. The main equipment used were the engine and dynamometer along with its torque arm and load measurement system, also a fuel flow measurement system with sensors for engine operating conditions and a data recording system.

The operating principle of the dynamometer is that the engine is run against a variable resistance. As the crankshaft turns, the dynamometer absorbs the engine’s output using a magnetic field generated by stationary coils that react with the internal rotor, creating a magnetic braking force that opposes rotation. The reaction force is measured through the strain gauge which sends a signal between 0 and 10v. This measured load acts at a known radius, allowing torque to be calculated, once torque and engine speed are received, power can then be calculated. Because fuel flow was also recorded it is possible to measure thermal efficiency rather than considering power output alone.

The method used during the investigation was steady-state testing across a series of engine speeds. First the engine was warmed to normal operating temperature so that the results would be more representative and less affected by cold-running conditions. The engine was then held at specific speeds while the dynamometer applied a load and stabilised the operating condition. At each test point the force on the dynamometer, engine speed and fuel flow data were recorded. These raw figures were later entered into a spreadsheet, where the required calculations were conducted automatically using the correct engineering formulae. Killedar (2012, pp.29–40)

Using multiple tests points it produces a consistent set of measurements across the operating range of the engine. By progressing through multiple speeds it becomes possible to find trends rather than relying on one isolated test value. Because atmospheric conditions influence engine output the computer will implement a correction factor to compare power under standard conditions. Which is why the power values were also corrected using the DIN style atmospheric correction based on ambient pressure and temperature. This correction is intended to make results more comparable when tests are carried out in different weather conditions.

3. Spreadsheet Calculations and Processed Results

Once the data from the dynamometer has been entered into the spreadsheet the first calculation can be carried out which was torque. As shown in the handout torque is equal to the applied load multiplied by the torque arm radius, which in this case the torque arm length is 0.815776m.

Brake power was then calculated using engine speed and torque. The handout shows that power in a rotating system depends on the rate at which work is done and therefore on both torque and rotational speed. This is why the brake power curve does not follow the torque curve exactly. Even if torque falls after a certain point brake power may continue to rise provided engine speed is still increasing strongly enough.

BMEP was also calculated because it is a more universal way of expressing engine performance. The handout describes BMEP as the equivalent mean pressure available to do work, and notes that it is proportional to torque. This makes it helpful for comparing engines regardless of size. The peak BMEP found from the spreadsheet was approximately 812.5 kPa, which occurred at the same engine speed as maximum torque, namely 2350 rpm. This would be expected because BMEP and torque are directly related for a given engine displacement.

Specific fuel consumption was then calculated using brake SFC as the kilograms of fuel used per kilowatt hour of power which is an important indicator of how efficiently the engine is producing output power. A higher SFC means the engine requires more fuel to generate the same amount of work which is not the desired result. The spread sheet results show a minimum SFC of 0.261 Kg/KWh at 1750 rpm. At 1000rpm and 4700 rpm the SFC reading was higher which would indicate that the engine has a specific optimum efficiency region in the lower to mid range.

Thermal efficiency was then calculated using power, fuel flow and fuel calorific value. Brake thermal efficiency will act as an overall measurement of the engines ability to convert chemical and heat energy into mechanical output. From the handout I can see that petrol engines typically operate with a thermal efficiency of 20% to 30%. Using the data from the spread sheet we can see that the thermal efficiency of the Honda engine was approximately 31.3% which is very close to the expected range for a petrol engine.

Power and Torque was also calculated using atmospheric correction, this uses ambient pressure and temperature to adjust the measured power to standard DIN specifications which will allow result to be compared accurately. The peak power produces by the in was 44.4Kw at 4700rpm and the peak torque was 103.5Nm at 2350rpm. (Ferguson and Kirkpatrick, 2016)

4. Graphical Analysis

4.1 SFC against Engine Speed

The SFC against Engine Speed graph (Figure 1) shows that fuel consumption was the worst at high engine speeds and best at lower engine speeds. At 1000rpm the SFC was 0.362Kg/KWh but then decreased as the engine rpm increased until reaching a minimum of 0.261Kg/KWh at 1750rpm before increasing again at 5500rpm to 0.376Kg/KWh.

This can be explained by considering combustion quality and engine losses as at low engine speeds the engine produces low power yet it still has to overcome the internal friction and pumping losses which will make the amount of fuel use per KWh high. When the engine rpm increases it will become more efficient as cylinder improves and becomes more stable a larger portion of the chemical energy is converted into power output which will give low SFC values.

Figure 1

4.2 Torque against Engine Speed

As shown in the torque graph (Figure 2) the torque from 83.43Nm at 1000rpm to a maximum of 103.45Nm at 2350 rpm before slowly decreasing to 72.31Nm at 5500rpm.

This is natural from a petrol engine as at low engine speed airflow through the intake system is low which will result in poor cylinder filling and combustion conditions for maximum power. As the engine rpm increases the intake and exhaust process becomes more efficient resulting in good volumetric efficiency from the valvetrain. At higher rpm the torque begins to reduce due to the time for cylinder filling and emptying reducing with an addition to frictional losses caused by high engine rpm.

Figure 2

4.3 Brake Power against Engine Speed

Brake power increases from 8.79Kw at 1000rpm to a maximum of 44.4Kw at 4700rpm before dropping off to 41.9Kw at 5500rpm as shown in the graph (Figure 3).

This result is different from the torque curve as brake power depends on both torque and engine speed which is shown by the maximum torque at 2350rpm but the constant increase in engine rpm causes the brake power to increase despite the engine producing slightly less turning force per revolution after the torque peak.

Figure 3

5. Conclusion

The data gathered from the dynamometer successfully allowed me to evaluate the performance characteristics of a 1.6L Honda engine throughout its rpm range. Using the correct torque arm length of 0.81776 I found that the engine produced a peak brake power of 44.4Kw at 4700rpm and a maximum torque of 103.5 at 2350rpm. Specific fuel consumption was at its lowest at 0.261Kg/KWh and 1750rpm which is when the engine is at its most efficient at turning chemical energy into power and is typical for a petrol engine. SFC in not only useful for checking an engines efficiency but also for determining the estimated running cost of an engine and its most economical operating region.

The trends shown in the graphs were consistent with established engine theory, as torque increased through the lower engine rpm range and volumetric efficiency increased then fell at higher rpm as volumetric efficiency reduced and frictional losses increased. Although brake power continued to increase above torque as it relies on both torque and rpm. Specific fuel consumption was at its most efficient in the lower to mid rpm range showing the engine is at its most efficient with the least amount of fuel used when at a steady rpm which is consistent with previous theory.

Overall the desired results were achieved using raw data from the dynamometer to calculate the main performance characteristics from the Honda engine.

6. Evaluation

In terms of personal evaluation I believe I completed each task in a structured and methodical way, the raw data was entered into a spreadsheet and the correct formulae were used and the results were plotted on a well presented graph with clear titles. This gave a clear example to fully understand the engine performance characteristics also in terms of how data can be processed into meaningful results such as power, torque, SFC and thermal efficiency for better understanding. However there are still some points I could have improved on such as the spreadsheet could have been laid out clearer so that each stage of the calculations was easier to follow.

Due to the dynamometer being non operational during this period and having to use pre recorded data it would have been greatly beneficial and improved my understanding of the system and test results, however the pre recorded data proved to be adequate to carry out the calculations but may have included possible errors.

However if I had conducted the test myself I could have used more test points to improve result accuracy and reliability to reduce the chances of inaccurate results around the maximum output for torque and power. To further validate the test results you could carry out repeated runs under the same operating conditions and take an average result. Re calibration of the sensors would have also been beneficial as this will ensure a true accurate reading which will avoid errors in the data gathered.

References

Ferguson, C.R. and Kirkpatrick, A. (2016). Internal combustion engines : applied thermosciences. Chichester, West Sussex, United Kingdom: John Wiley & Sons, Inc.

Killedar, J.S. (2012). Dynamometer : theory and application to engine testing. Xlibris, pp.29–40.

Nwufo, O.C., Nwaiwu, C.F., Ononogbo, C., Igbokwe, J.O., Nwafor, O.M.I. and Anyanwu, E.E. (2017). Performance, emission and combustion characteristics of a single cylinder spark ignition engine using ethanol–petrol-blended fuels. International Journal of Ambient Energy, 39(8), pp.792–801. doi:https://doi.org/10.1080/01430750.2017.1354318.

Figure 1 (Harri Evans)

Figure 2 (Harri Evans)

Figure 3 (Harri Evans)