University of Wales Trinity Saint David

School of Engineering

BSc AUTOMOTIVE ENGINEERING

Investigating intake and exhaust systems.

Gareth England

1702839

Powertrain Technology

M5X02200

Contents

Introduction.	3

Research.	4

Methodology.	6

Results.	8

Analysis.	9

Evaluation.	10

Modifications.	13

Conclusion.	14

Works Cited	15

# Introduction.

this is a report about the investigation into the design and operations of the induction and exhaust manifolds and what factors influences them. This will be done by analysing and producing graphs based on data derived from testing the flow characteristics of the cylinder head.

The flow characteristics will be determined by placing the cylinder head onto a flow bench and recording the rate of air flow through the bare cylinder head, a cylinder head with an inlet manifold and a cylinder head with a flared entry port.

# Research.

Gas motion within the engine cylinder is one of the major factors that controls the combustion process in spark-ignition (SI) engines, and the fuel-air mixing and combustion processes in diesel engines. It also has a significant impact on engine heat transfer. Both the bulk gas motion and the turbulence characteristics of the flow are important. The initial in-cylinder flow pattern is set up by the intake process. It may then be substantially modified during compression. This chapter reviews the important features of gas motion within the cylinder set up by flows into and out of the cylinder through valves or ports, and by the motion of the piston.

The engine intake process governs many important aspects of the flow within the cylinder. In four-stroke cycle engines, the downward motion of the piston pulls air into the cylinder. The intake flow velocities scale with piston speed. For much of the intake stroke, the annular gap between the intake valve head and valve seat is the minimum area for the flow (see Sec. 6.3) so gas velocities through this gap are the highest velocities set up during the intake process. The gas issues from the valve opening into the cylinder as an annular conical jet and the radial and axial velocities in the jet are about 10 times the mean piston speed. Figure 8.1 shows the radial and axial velocity components close to the valve exit, measured during the intake process, in a motored model engine with transparent walls and single valve located on the cylinder axis, using laser doppler anemometry (see next section).1 The jet separates from the valve seat and head, producing shear layers with large velocity gradients which generate turbulence. This separation of the jet sets up recirculation regions beneath the valve head and in the corner between the cylinder wall and cylinder head. The motion of the intake jet into the cylinder is shown in the schlieren photographs in Fig. 8.2 taken in a firing transparent engine. This engine had a square cross-section cylinder made up of two quartz walls and two steel walls, to permit easy optical access. The schlieren technique makes regions with density gradients in the flow show up as lighter or darker regions on the film.2 The engine was throttled to one-half an atmosphere intake pressure, so the jet starts after the intake stroke has commenced, at 35° after top center (ATC), following the backflow of residual into the intake manifold. The front of the intake jet can be seen propagating from the valve toward the cylinder wall at several times the mean piston speed. Once the jet reaches the wall (θ > 41° ATC), the wall deflects the major portion of the jet downward toward the piston; however, a substantial fraction flows upward toward the cylinder head. The highly turbulent nature of this intake jet is evident.

8.1 Intake-Generated Flows 349 Figure 8.3 Large-scale rotating flow pattern setup within the cylinder by the intake jet. Photograph of streak lines in water flow into model engine cylinder with axisymmetric valve.3 The interaction of jet with the wall produces large-scale rotating flow patterns within the cylinder volume. These are easiest to visualize where the engine geometry has been simplified so the flow into the cylinder is axisymmetric. The photograph in Fig. 8.3 of a water analog of an engine intake flow was taken in a transparent model of an engine cylinder and piston. The valve is located in the center of the cylinder head, and the flow into the valve is along the cylinder axis. The experimental parameters have been scaled so that the appropriate dimensionless numbers which govern the flow, the Reynolds and Strouhal numbers, were maintained at typical engine values. The photograph shows the major features of the intake-generated flow in a thin illuminated plane through the cylinder axis. The streaks are records of the paths of tracer particles in the flow during the period the camera shutter is open. The bulk of the cylinder as the piston moves down is filled with a large ring vortex, whose center moves downward and remains about halfway between the piston and the head. The upper corner of the cylinder contains a smaller vortex, rotating in the opposite direction. These vortices persist until about the end of the intake stroke, when they became unstable and break up.3 With intake valve location and port geometry more typical of normal engine practice, and with two intake valves per cylinder, the intake-generated flow is more complex. However, the presence of these large-scale rotating flow patterns, produced by the airflow into the cylinder, can still be discerned. Such flows are inherent to the poppet valve geometry in the cylinder head: they bring into the cylinder significant fluid kinetic energy and create shear, both of which can generate turbulence. The intake valve(s) and port geometries can be designed to augment these jet-created flows by generating large-scale in- cylinder rotating flows. Two of these, swirl and tumble, are illustrated in Fig. 8.4 by the flow streak lines highlighted by flow tracers in water analog studies of real cylinder airflows. Swirl, Fig. 8.4a, organized fluid rotation about the cylinder axis, is usually generated by bringing the air into the cylinder with net angular momentum about the cylinder axis. It is used in smaller higher speed diesel engines to promote more rapid fuel-air mixing, and also in spark-ignition engines to increase the engine’s combustion rate. Tumble, Fig. 8.4b, is the name given to flow rotation about an axis orthogonal to the cylinder axis. It is generated by using closer to horizontal intake ports which direct the intake airflow across the cylinder head and down the far side of the liner as illustrated. Tumble is used to increase the kinetic energy stored in the in-cylinder charge during intake: the upward motion of the piston during compression breaks up this tumbling flow which generates additional turbulence, and as a result speeds up the spark-ignition engine combustion process.

(Heywood, 2018)

Mass flow of engine inlet can be calculated by isentropic port equation 1; the port flow efficiency can be obtained by steady flow test equipment.

(1)dm/dt=Aeff⋅po1⋅√2/Ro⋅To1⋅Ψ

The effective flow area in equation 2 is decided by the flow coefficient:

(2)Aeff=μσ⋅dvi2⋅π/4

The air charged into cylinder can be obtained by the instantaneous flow integrated. Then the engine volumetric efficiency can be obtained by equation 3.

(3)ηv=m1/ms=(Pa/Ps)⋅(Ts/Ta)⋅(Va/VS)

(Xu, 2016)

# Methodology.

Start by Weighing all moving components in the cylinder head, this gives us the basis to work out the moving mass for future calculations, such as contact force. The components that need to be measured are as follows:

- Camshaft

- Valves

- Springs

- Spring retainers

- Collets

- Valve buckets

Next the Inlet and Exhaust valve diameter must be carefully measured for future calculations such as CFM per In^2, the values are also needed when determining what size valves can be used in the cylinder head.

Next the bore and stroke of the engine were measured along with the compression ratio; this will allow us to calculate the cubic capacity of the engine and cylinders.

Then the camshaft profile of the induction system was measured to be used to plot graphs of valve lift against the crank shaft angle.

The next step was to place the cylinder head onto a flow bench and record the rates of flow through the inlet ports. Specific guidelines must be followed to accurately measure the rates. Three tests would be carried out, test one would be the rate of flow through the bare cylinder head inlet ports. Test 2 would be the same test but with the inlet manifold attached to the head and test 3 would be the bare cylinder head with a flared inlet port attached, the inlet port was made from blu tack as the properties allowed it to be easily manipulated into shape and attached to the head. The measurements would be taken at every 0.5 of a millimetre increment that the valve opens.

With these results graphs can be plotted ready to be analysed, the first graph to be plotted is CFM against valve lift, CFM is cubic feet per minute this refers to the amount of air flowing into the cylinder head. Upon plotting the graph, the flow was measured at a valve lift that was equal to a quarter of the valves diameter.

From the flow bench measurements, plot a graph for flow coefficient against valve lift and repeat for all the cylinder head stages. each graph should indicate maximum practical flow coefficients for the cylinder head. These values along with a thorough investigation of the cylinder head can allow you to estimate and determine any theoretical modifications that can be made, such as increasing the valve diameter.

Next determine the loss in flow coefficient for the induction system that is caused by adding an inlet manifold and also establish the gained flow coefficient from adding a flared entry port.

These results can be used to plot graphs of inlet flow coefficient (CF) against crank angle And should be repeated for all induction stages.

Using the plotted graph of CF, the mean flow of coefficient can be worked out for both the bare head and the cylinder head with the inlet manifold attached.

Furthermore, Using the mean flow coefficient, the inlet valve diameter and displacement to determine the engine speed maximum power of the road engine. Discuss how close this speed is to the stated original manufacturer specifications.

Lastly by utilising a combination of the maximum available inlet valve area and the highest practical flow coefficient for your head type, calculate and plot a graph of the required inlet valve lift profile (graph of lift against crank angle) needed to achieve the required mean flow coefficient to raise the RPM of maximum power.

# Results.

| Cam Lift | bare head % | L/s | CFM |  | inlet manifold % | L/s | CFM |  | flared port % | L/s | CFM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.5 | 38/16.5 | 6.27 | 2.96571 |  | 40/16.5 | 6.6 | 3.1218 |  | 42/16.5 | 6.93 | 3.27789 |
| 1 | 81/16.5 | 13.365 | 6.321645 |  | 75/16.5 | 12.375 | 5.853375 |  | 81/16.5 | 13.365 | 6.321645 |
| 1.5 | 53.5/33.2 | 17.762 | 8.401426 |  | 60/33.2 | 19.92 | 9.42216 |  | 62/33.2 | 20.584 | 9.736232 |
| 2 | 74/33.2 | 24.568 | 11.62066 |  | 76/33.2 | 25.232 | 11.93474 |  | 77/33.2 | 25.564 | 12.09177 |
| 2.5 | 41/69.7 | 28.577 | 13.51692 |  | 95/33.2 | 31.54 | 14.91842 |  | 48/69.7 | 33.456 | 15.82469 |
| 3 | 54/69.7 | 37.638 | 17.80277 |  | 54/ 69.7 | 37.638 | 17.80277 |  | 57/69.7 | 39.729 | 18.79182 |
| 3.5 | 63/69.7 | 43.911 | 20.7699 |  | 59/69.7 | 41.123 | 19.45118 |  | 65/69.7 | 45.305 | 21.42927 |
| 4 | 67/69.7 | 46.699 | 22.08863 |  | 66/69.7 | 46.002 | 21.75895 |  | 73.5/69.7 | 51.2295 | 24.23155 |
| 4.5 | 73/69.7 | 50.881 | 24.06671 |  | 70/69.7 | 48.79 | 23.07767 |  | 78.5/69.7 | 54.7145 | 25.87996 |
| 5 | 78/69.7 | 54.366 | 25.71512 |  | 74/69.7 | 51.578 | 24.39639 |  | 83/69.7 | 57.851 | 27.36352 |
| 5.5 | 81/69.7 | 56.457 | 26.70416 |  | 76/69.7 | 52.972 | 25.05576 |  | 87/69.7 | 60.639 | 28.68225 |
| 6 | 85/69.7 | 59.245 | 28.02289 |  | 78/69.7 | 54.366 | 25.71512 |  | 90/69.7 | 62.73 | 29.67129 |
| 6.5 | 87/69.7 | 60.639 | 28.68225 |  | 80/69.7 | 55.76 | 26.37448 |  | 92/69.7 | 64.124 | 30.33065 |
| 7 | 89/69.7 | 62.033 | 29.34161 |  | 81/69.7 | 56.457 | 26.70416 |  | 94/69.7 | 65.518 | 30.99001 |
| 7.5 | 91/69.7 | 63.427 | 30.00097 |  | 82/69.7 | 57.154 | 27.03384 |  | 96/69.7 | 66.912 | 31.64938 |
| 8 (max cam lift) | 92/69.7 | 64.124 | 30.33065 |  | 83/69.7 | 57.851 | 27.36352 |  | 48/138.2 | 66.336 | 31.37693 |
| 8.5 | 93/69.7 | 64.821 | 30.66033 |  | 83/69.7 | 57.851 | 27.36352 |  | 48.5/138.2 | 67.027 | 31.70377 |
| 9 | 94/69.7 | 65.518 | 30.99001 |  | 84/69.7 | 58.548 | 27.6932 |  | 49/138.2 | 67.718 | 32.03061 |
| 9.5 | 95/69.7 | 66.215 | 31.3197 |  | 84/69.7 | 58.548 | 27.6932 |  | 49/138.2 | 67.718 | 32.03061 |
| 10 | 42/138.2 | 58.044 | 27.45481 |  | 85/69.7 | 59.245 | 28.02289 |  | 49.5/138.2 | 68.409 | 32.35746 |
| 10.5 | 42/138.2 | 58.044 | 27.45481 |  | 85/69.7 | 59.245 | 28.02289 |  | 49.5/138.2 | 68.409 | 32.35746 |

Table 1

| component weights and measurements |  |
| --- | --- |
| valves | 38 grams |
| springs | 20.4 grams |
| spring retainers | 7.7 grams |
| collets x2 | 1 gram |
| spring buckets | 23.8 grams |
| inlet valve diameter | 24mm |
| exhaust valve diameter | 30mm |
| cylinder bore | 78.85mm |
| piston stroke | 81.4mm |
| head gasket | 0.38mm |
| compression ratio | 11.0 : 1 |

Table 2

# Analysis.

Table 1. Above are the results gathered from the 1.6 ford sigma zetec SE engine cylinder head from the tests preformed on the flow bench. Column one (left to right) is the valve lift, this was achieved by turning a bolt fixed to the cylinder head half a millimetre. The bolt acts like the cam shaft opening the valve. The results end at 10.5 millimetres because that was the max stress we could put on the valves opening the valves any further would have damaged the valves. The max lift the valves would usually do when fixed in a vehicle is 8 millimetres, this is known because the measured cam lobe lift was 8 millimetres.

Columns 2, 6 and 10 are the results the flow bench produced at the half a millimetre intervals we set. As seen in the table the results are set out as 38 / 16.5 for example, the 38 is the number retrieved from the test on the flow bench these are percentages, the dash means its divided by the last number which is the conversion rate, the conversion rate ascertains to the stage the flow bench is set at 16.5 was stage one. The stages increase as the percentage reaches its maximum.

Column 2 is the bare cylinder head with nothing attached to the inlet port, column 6 is the cylinder head with the stock inlet manifold attached to it and column 10 is the cylinder head with a flared entry attached to the outer edge of the inlet port made from blu tack.

# Evaluation.

Above is a graph plotted to show Cam lift in relation to the angle of the crank. Measurements were recorded at every two-degree interval for a full 360-degree cycle which depicted the maximum lift of the cam for a standard 1.6 Zetec SE sigma engine, which was 8.03 millimetres.

the graph above depicts CFM (Cubic feet per minute) in relation to the valve lift. the CFM was recorded from the cylinder head flow testing. from the graph it’s clear that as the valve lifts the port flow area increases allowing more air to pass through into the engine cylinder. The tests were repeated for each cylinder head stage and as visible by the graph the bare cylinder head drops at the end this may be due to a testing error where we increased the stage too soon.

Whilst testing the cylinder head we surpassed the max cam lift to provide a wider scope of data so that a more informed decision could be made on the improvements and modifications of the induction system.

One of the tasks of this assignment was to find the CFM and a flow rate that was a quarter of the valve diameter, the valve was measured at 24mm so a quarter would equal 6mm. The CFM at 6mm for the cylinder head with the intake manifold was 25, for the bare cylinder head the CFM measured 28 and lastly the head with the flared entry port was 30 CFM. The flow is at its greatest with a flared entry because the air can flow better around the curved edges, which is the reason intake trumpets are used their curved outer ring provides a smooth surface for air to be drawn across and in.

The standard inlet manifold reduces the flow because the air has to be drawn through a lengthy tube, this reduces the airs momentum, because inlet manifold is designed for control over power. The tubes on the inlet manifold are designed to draw in equal amounts of air for each cylinder.

Above is the CFM per square inch of air flow in relation to valve lift. This graph was achieved using values worked out using the normal CMF and dividing it by the area of the port. First the area had to be calculated this was done using the equation pi times diameter squared divided by four. Before this we needed to know the diameter, which was 24mm but converted to inches was 0.945. inputting these values equalled an area of 0.701. the results are a rough estimate as the valve stem was not factored in. by reviewing the graph the bare cylinder head reaches a greater peak flow, but the intake manifold has a more controlled incline.

| Flow Co-efficient |  |  |  |
| --- | --- | --- | --- |
| perf. CFM/meas. CFM (bare) | perf. CFM/meas. CFM (manifold) | Flow Coefficient loss | Flow Coefficient added |
| 0.030657136 | 0.032270669 | -0.001613533 | 0.003227067 |
| 0.065348105 | 0.060507505 | 0.0048406 | 0 |
| 0.086847216 | 0.097398747 | -0.010551531 | 0.013798156 |
| 0.120125121 | 0.123371746 | -0.003246625 | 0.004869937 |
| 0.139727108 | 0.154214683 | -0.014487575 | 0.023855848 |
| 0.184030826 | 0.184030826 | 0 | 0.010223935 |
| 0.21470263 | 0.201070717 | 0.013631913 | 0.006815957 |
| 0.228334543 | 0.224926565 | 0.003407978 | 0.022151859 |
| 0.248782412 | 0.238558478 | 0.010223935 | 0.01874388 |
| 0.265822304 | 0.252190391 | 0.013631913 | 0.017039891 |
| 0.276046238 | 0.259006347 | 0.017039891 | 0.02044787 |
| 0.289678151 | 0.265822304 | 0.023855848 | 0.017039891 |
| 0.296494108 | 0.27263826 | 0.023855848 | 0.017039891 |
| 0.303310064 | 0.276046238 | 0.027263826 | 0.017039891 |
| 0.310126021 | 0.279454217 | 0.030671804 | 0.017039891 |
| 0.313533999 | 0.282862195 | 0.030671804 | 0.010815564 |
| 0.316941977 | 0.282862195 | 0.034079783 | 0.010786227 |
| 0.320349956 | 0.286270173 | 0.034079783 | 0.01075689 |
| 0.323757934 | 0.286270173 | 0.037487761 | 0.007348911 |
| 0.283805867 | 0.289678151 | -0.005872284 | 0.050679619 |
| 0.283805867 | 0.289678151 | -0.005872284 | 0.050679619 |

Above is the graph that shows the coefficient of flow, the CF is calculated by using the cubic feet per minute per square inch and dividing it by the perfect CFM per square inch. The perfect CFM/In^2 is dictated as 138 by the super flow manual.

| mean CFM (bare head) | 38.96374852 |
| --- | --- |
| mean CFM (manifold) | 39.76352048 |

To calculate the mean flow coefficient (Cf). You must take the highest Cf value and the lowest CF value and minus them then divide by the collective amount of values there are, in this case there were 21 measurements.

# Modifications.

Under careful inspection there wasn’t a lot of modification options for the exhaust side as the valves were as big as they could be. There could be options such as a lighter material or thinner valves made of stronger materials.

The induction aspect could be improved for power by increasing the valve diameter as the valves were quite small. Another option could be to open the valves at a larger depth because from our result we saw that even after max cam lift the CFM still increased meaning more air can be brawn in if given the chance. I wouldn’t decrease the valve size as I believe they are at a peak diameter if the modifications are meant to be economical.

Both the inlet and exhaust ports are oval shaped, and the exhaust port is nearly to the point where it can be made larger, but the shapes of the ports could be changed and made more square for example to increase flow area.

Also the finish in the ports could be changed and smoothed for better flow but that increases the likelihood that the air doesn’t swirl as much causing a bad air fuel mixture which would have to rectified else where such as by adding swirl flaps or shaping the piston crown for a swirl effect.

# Conclusion.

This engine can be modified exponentially but it ends up being an ultimatum. You can modify it to be economical or you can modify it to be more powerful, but you can have both. The ford engineers have designed the engine to hit that sweet spot where its economical to drive without compromising to greatly on power, that being said if you are looking to make this engine for a specific use there are modification options that will allow that.

# Works Cited

Heywood, J. (2018). Internal combustion engine fundamentals (Second edition. ed.). New York: McGraw-Hill.

Xu, J. (2016). Effects of electromagnetic intake valve train on gasoline engine intake charging. China: Elsevier B.V.