Distortion of Materials

Examples

- Calculate the average shear stress in a solid gudgeon pin of 30mm diameter if the load on the pin is 22kN. (Double Shear)

- A round bar has a length of 270mm and diameter 15mm.  A tensile load is gradually applied to the bar.  A load of 25kN causes an extension of 0.2mm.  Find the value of Young’s modulus of elasticity for the bar material.

- A push rod has a diameter of 5mm.  At a certain point in the operation of the valve the force on the rod is 2.5kN.  Calculate the compressive stress in the rod.

- A piston has a force of 4kN acting on the crown at TDC.  The cross sectional area of the connecting rod is 180mm2.  Calculate the compressive stress in the material.

- A steel bar has a diameter of 50mm and is 1m long.  The bar supports a tensile load of 50kN.  Calculate the resilience in the bar if E=200GN/m2.

- A cast iron cylinder has an outside diameter of 120mm and an inside diameter of 100mm, and supports a load of 100kN.  Find the compressive stress in the material.

7m

F

A beam of length 7m is simply supported at each end and carries a concentrated load 3m from one end.  If the bending moment is 50kNm at the section where the load is concentrated, find the magnitude of the load and the reaction forces.

A     3m	              			              B

Bending moment @ F = 50x103Nm

Forces up = Forces down

RA+ RB=F_(i)

Taking moments about A

Taking moments about B

Therefore:

RA+ RB=F

- A beam is 200mm deep by 100mm wide and subject to a maximum bending moment of 300kNm.  What is the maximum stress?

d=200mm

b=100mm

- A steel rectangular bar of thickness 12mm is bent into an arc until the steel yields at the top surface.  The yield stress of the material is 300MN/m2 and E = 200GN/m2.  Find the radius of curvature.

- A solid steel shaft has a diameter of 300mm and twist through 1˚ over a length of 5m while rotating at a speed of 100rev/min,  The value of G = 85 GN/m2.  Calculate the torque imposed on the shaft.

- A torque of 100Nm is applied to a shaft 2m in length.  The angle of twist is 0.5˚.  The value of G=85GN/m2.  Calculate the diameter of the shaft.

100N		      200N	                300N

A beam of length 5m is simply supported at the ends.  It carries concentrated loads of 100N, 200N and 300N at distances of 1m, 2.5m and 4 from one end, respectively.  Calculate the reaction forces.

4m

2.5m

1m

5m

A							            B

- Find the twisting moment of a solid shaft of diameter 100mm when the angle of twist is 0.75˚ over a length of 2m.  For the shaft material, G=100GN/m2.

- A hollow shaft has an outside diameter of 350mm and an inside diameter of 250mm.  It transmits a torques of 400kNm.  Calculate the shear stress if G = 90GN/m2.