# ACDA5001 Electrical Engineering

# Assignment

Joseph Samuel Roan

2411525

05/01/2026

Table of Contents

1.	Abstract	6

2.	Introduction	7

2.1	What is an AC Circuit?	7

2.2	What is am RLC Circuit?	7

2.3	What is Power Factor?	7

2.4	What is Resonance?	7

2.5	What is Maximum Power transfer?	7

2.5.1	AC Circuits	7

2.5.2	DC Circuits	7

2.6	What is Power Factor Correction?	8

3.	Method	9

3.1	RL Circuit	9

3.2	RLC Circuit	10

3.2.1	Calculations for the Capacitor:	11

3.3	Power	16

3.3.1	Apparent Power	16

3.3.2	Reactive Power	16

3.3.3	Active Power	16

3.4	Power Factor	16

3.5	Phase Angle	18

3.6	What happens to energy stored in Capacitor and Inductor as the Power Factor Reaches 1?	20

4.	Results	21

4.1 Phasor Diagrams and Results	21

4.1.1	Resonance	21

4.1.2	25 Hz Below Resonance	22

4.1.3	25 Hz Above Resonance	25

4.1.4	50 Hz	27

5.	Discussion	29

5.1	How the PFC relates to Resonance of the Circuit	29

5.2	How the PFC relates to Achieving Maximum Power Transfer	30

5.3	How the PFC relates to Energy Efficiency	30

5.4	Why is Electricity Three-Phase within Developed Countries?	30

5.5	Why do systems generally not go beyond 3 phases?	30

5.6	Significance of PF and PFC in Electrical Engineering Systems	31

6.	Conclusion	32

7.	References	33

List of Figures

Figure 1 - RL Circuit Design	9

Figure 2 - RL Circuit Graph	9

Figure 3 – RLC Circuit Design	10

Figure 4 – RLC Circuit Graph	10

Figure 5 - Phasor Diagram	12

Figure 6 - Triangle OAC	13

Figure 7 - Triangle OAB	14

Figure 8 - Graph of Power Factor in relation to Capacitor Value	17

Figure 9 - Graph of Phase Angle in Relation to Capacitor Value	19

Figure 10 - 25 Hz Below Resonance Phasor Diagram	22

Figure 11 - 25Hz Above Resonance Phasor Diagram	25

Figure 12 - Phasor Diagram For 50Hz	27

Figure 13 - Resonance in PFC [21]	29

Table 1 - PFC	18

Equation 1 - Vrms	11

Equation 2 - Irms	11

Equation 3 - ∅1 Calculation	11

Equation 4  - Power Factor	11

Equation 5 - Cos ∅2 Calculation	11

Equation 6 - ∅2 Calculation	11

Equation 7 - Cos  ∅1	13

Equation 8 -OA	13

Equation 9 - Sin  ∅1	13

Equation 10 - Ac	13

Equation 11 - ∅1	14

Equation 12 - OB	14

Equation 13 - Sin ∅2	14

Equation 14 - AB	14

Equation 15 - BC	15

Equation 16 - Xc	15

Equation 17 - Xc	15

Equation 18 - Formulae for Capacitor	15

Equation 19 - Capacitor	15

Equation 20 - Apparent Power	16

Equation 21 - Reactive Power	16

Equation 22 - Active Power	16

Equation 23 - Formulae for Fr	18

Equation 24 - Calculation for Fr	18

Equation 25 - Xl Calculation	18

Equation 26 - ∅LR Formulae	18

Equation 27 - ∅LR Calculation	18

Equation 28 - Equation for Fr	21

Equation 29 - Fr	21

Equation 30 - Frequency	22

Equation 31 – Xc Calculation	22

Equation 32 – Xl Calculation	22

Equation 33 - Formulae For ZLR	22

Equation 34 – ZLR Calculation	23

Equation 35 - ILR Formulae	23

Equation 36 – VLR Calculation	23

Equation 37 – ILR Calculation	23

Equation 38 - Ic Calculation	23

Equation 39 - ∅LR Calculation	23

Equation 40 - ∅ Formulae	23

Equation 41 - ∅ Calculation	23

Equation 42 - ILR  Sin ∅LR Calculation	24

Equation 43 - I Calculation	24

Equation 44 - Frequency	25

Equation 45 - Xc Calculation	25

Equation 46 - Xl Calculation	25

Equation 47 - ZLR	25

Equation 48 – ILR Calculation	26

Equation 49 – Ic Calculation	26

Equation 50 - ∅LR Calculation	26

Equation 51 - ∅ Calculation	26

Equation 52 - ILR  Sin ∅LR Calculation	26

Equation 53 – I Calculation	26

Equation 54 - Xc Calculation	27

Equation 55 - Xl Calculation	27

Equation 56 - ZLR Calculation	27

Equation 57 - ILR Calculation	27

Equation 58 - Ic Calculation	28

Equation 59 - ∅LR Calculation	28

Equation 60 - ∅ Calculation	28

Equation 61 - ILR  Sin ∅LR Calculation	28

Equation 62 - I Calculation	28

# Abstract

Throughout this experiment, the author has created a RL and an RLC Circuit, calculated the capacitor to create the RLC Circuit, given graphs of this too. Additionally, the author has calculated voltages at different at resonance and at different frequencies relating to resonance, while also creating phasor diagrams too. The results outputted, in reference to PFC, determine that when the capacitor value increases, so does the Power Factor, however the phase angle will decrease, this is also true in vice-versa.

# Introduction

## What is an AC Circuit?

‘AC’ Stands for Alternating Current [1], which is described a circuit, which can be single or three phase, with the produced voltages being at 120° [2]. They are made up of windings, which can be Star or Delta [2]. These can be used in various applications, for example Power Generation, Power Distribution, Motors, Heating and Lighting.

## What is am RLC Circuit?

An RLC Circuit is a circuit containing a Resistor (Described as ‘R’), an Inductor (Described as ‘L’) and a Capacitor (described as ‘C’) [3]. They are generally used in Filters, Voltage Stabilization and Medical Application.

## What is Power Factor?

Power Factor (PF), refers to the measurement of the effectiveness of electrical power being converted into output that is used within an electrical system [4]. It is the Ratio of Real Power to apparent power, which indicates the efficiency of power usage within an electrical system [4]. The highest value of PF is 1 [5], this means maximum efficiency, therefore the larger the PF is, the higher the efficiency. A method of increasing this in an electrical circuit is PFC, See section 2.6 below, for an explanation.

## What is Resonance?

Resonance or Resonance Frequency is the maximum amplitude response [6].  It occurs when a force is applied periodically at the natural frequency of the system, this results in larger amplitudes in comparison to other frequencies, therefore being the maximum amplitude response [7].

## What is Maximum Power transfer?

Maximum Power Transfer differs in AC and DC Circuits, they are explained below in sections 2.5.1 & 2.5.2.

### AC Circuits

In AC Circuits it’s when the maximum power is transferred to the load, when its impedance is equal to the complex conjugate [8].

### DC Circuits

Additionally, within DC Circuits it’s when the maximum power transferred to the load, when the loads resistance equals the internal resistance of the source [9].

## What is Power Factor Correction?

The purpose of Power Factor Correction (PFC) is to bring the Power Factor to close to ‘Unity’ as possible [10]. PFC is a method that will increase the efficiency of electrical power [4], a PFC Circuit will shape the current input, so that it will be in phase with the line voltage and reduce the apparent power used [11]. Moreover this uses capacitors to reduce reactive power, as it will reduce current and improve its efficiency  [12].

# Method

## RL Circuit

Figure 1 - RL Circuit Design

This circuit was created by the author, the values for the components were derived from their student number (2411525). This is shown below in Figure 1. The Graph of results for this is shown below in Figure 2.

Figure 2 - RL Circuit Graph

## RLC Circuit

Figure 3 – RLC Circuit Design

This Circuit was designed by the author, the only difference between this and the RL Circuit created above in section 3.1, was the Capacitor. The value for this was calculated by the author, see Equation 19 below. The Circuit Design is shown below in Figure 3 and the Graph is shown below in Figure 4. The Calculations for the Capacitor is shown below in Section 3.2.1.

Figure 4 – RLC Circuit Graph

### Calculations for the Capacitor:

Equation 1 - Vrms

Vrms=Vm√2=424√2=6.56A

Equation 2 - Irms

Irms=Im√2=9.28√2=299.81

Equation 3 - ∅1 Calculation

∅1=2.94×10-3 20×10-3

Equation 4  - Power Factor

Power Factor=Cos∅1=Cos (52.92)=0.60

The P.F. needed is 0.95, therefore:

Equation 5 - Cos ∅2 Calculation

Cos ∅2=0.95

Equation 6 - ∅2 Calculation

∅2=Cos-1(0.95) = 18.2°

Figure 5 - Phasor Diagram

In Triangle AOC:

Equation 7 - Cos  ∅1

Cos ∅1=OAOC

Equation 8 -OA

OA=6.56×Cos 52.92=IR=3.96A

Equation 9 - Sin  ∅1

Sin ∅1=AcILR

Equation 10 - Ac

Ac=6.56 ×Sin 52.92=Ic=5.23A

Figure 6 - Triangle OAC

In Triangle AOB:

Equation 11 - ∅1

Cos ∅2=OAOB

Equation 12 - OB

OB=I=3.96Cos (18.2)=4.17A

Equation 13 - Sin ∅2

Sin ∅2=ABOB

Equation 14 - AB

AB=Sin 18.2×4.17=1.30

Figure 7 - Triangle OAB

Equation 15 - BC

BC=AC-AB

Equation 16 - Xc

Xc=299.813.93=76.29Ω

Equation 17 - Xc

Xc=12πFC

Equation 18 - Formulae for Capacitor

C=12πFXC

Equation 19 - Capacitor

C=12π 6.56(76.29)=318.02µF

## Power

### Apparent Power

Waldo V. Lyon, an expert and author in electrical engineering [13], describes Apparent Power as “The Greatest” power that can be taken by a load, that has the same RMS line Currents and Voltages [14]. The authors calculations for Apparent Power are shown below in equation 20.

Equation 20 - Apparent Power

S=VI=299.81×6.56=1966.75

### Reactive Power

Reactive power is described as the “product of the voltage and current”, whilst being measured at the same point in the circuit, this is calculated using the sin function [15]. The authors calculations for Reactive Power are shown below in equation 21.

Equation 21 - Reactive Power

Q=299.81×6.56×Sin 52.92=1569.07

### Active Power

Similar to Reactive power, Apparent power refers to the “product of the voltage and current”, whilst being measured at the same point in the circuit, however this is calculated using the cos function [15]. The authors calculations for Active Power are shown below in equation 22.

Equation 22 - Active Power

P=VI Cos ∅ =299.81×6.56×Cos 52.92=1185.81

## Power Factor

The Equation the author used to Calculate the Power Factor is shown above in equation 4. With the calculations of the Power Factor, the author created a graph of Power Factor in relation to the value of the capacitor. This is shown below in Figure 8. The Table of the results are also shown in table 1, below.

Figure 8 - Graph of Power Factor in relation to Capacitor Value

## Phase Angle

Phase Angle is a measurement of angle between resistance and ratio of reactance with the same frequency [16]. The method in which the author calculated this, is shown below in Equations 23 to 27, this method is repeated from 5uF to 35uF. With the results of the Phase angle, the author created a table and a graph of Phase angle in relation to the capacitor value. These are shown below in Table 1 and Figure 9.

Equation 23 - Formulae for Fr

Fr=12π 1Lc-R2L2

Equation 24 - Calculation for Fr

Fr=12π 1115×10-3 5×10-6-252(115 ×10-3)2= 207.02

Equation 25 - Xl Calculation

Xl=2π×207.02×115×10-3=149.59 Ω

Equation 26 - ∅LR Formulae

∅LR=Tan-1 XL R

Equation 27 - ∅LR Calculation

∅LR=Tan-1 149.59 25=80.51°

Table 1 - PFC

Figure 9 - Graph of Phase Angle in Relation to Capacitor Value

## What happens to energy stored in Capacitor and Inductor as the Power Factor Reaches 1?

When Power Factor reaches 1, the power is at maximum efficiency, as only the useful power is being used (without any waste) [17]. This Happens due to the active power and reactive power being equal [18]. In regards to energy stored in a Inductor, when the Power Factor reaches 1, the current is in phase with the voltage, resulting in only the useful power being used [19]. This means that it’s at maximum efficiency and also has no reactive power within the circuit [19]. In regards to energy stored in a Capacitor, when the Power Factor reaches 1, it will result in the circuit being fully resistive, this is because it will consume all the power from the source, however will not consume any reactive power [20].

# Results

# 4.1 Phasor Diagrams and Results

The Phasor Diagrams showing the component voltages and current that were calculated for the RLC Circuit, see section 3.2. are shown below. Additionally the Results of the calculations are shown below:

## Resonance

The definition of resonance is outlined in the author above in section 2.4. The authors calculations for resonance are shown below:

Equation 28 - Equation for Fr

Fr=12π 1115×10-3 318.02×10-6-252(115 ×10-3)2

Equation 29 - Fr

Fr=34.59

## 25 Hz Below Resonance

For the Phasor diagram refer to Figure 9 below. The authors calculations for 25 Hz below resonance is shown below:

Figure 10 - 25 Hz Below Resonance Phasor Diagram

Equation 30 - Frequency

34.59-25=9.41Hz

Equation 31 – Xc Calculation

Xc=12π 9.41(318.02×10-6)=53.18 Ω

Equation 32 – Xl Calculation

Xl=2π×9.41×115×10-3=6.80 Ω

Equation 33 - Formulae For ZLR

ZLR= R2+XL2

Equation 34 – ZLR Calculation

ZLR= 252+6.802 =25.91Ω

Equation 35 - ILR Formulae

ILR=VLRZLR

Equation 36 – VLR Calculation

VLR=4242=299.81V

Equation 37 – ILR Calculation

ILR=299.8125.91=11.57A

Equation 38 - Ic Calculation

IC=VCXC=299.8153.18=5.64A

Equation 39 - ∅LR Calculation

∅LR=Tan-1 6.80 25=15.22°

Equation 40 - ∅ Formulae

∅=Tan-1 ILR Sin ∅LR~ IC ILR Cos ∅LR

Equation 41 - ∅ Calculation

∅=Tan-1 11.57 (Sin 15.22)~ 5.6411.57 Cos 15.22=13.12° [Leading]

Equation 42 - ILR  Sin ∅LR Calculation

ILR Sin ∅LR=11.57 Sin 15.22=3.04A

IC>ILR Sin ∅LR

Equation 43 - I Calculation

I= ILR Cos∅LR2+ ILR Sin∅LR~IC2=11.28A [Leading]

## 25 Hz Above Resonance

For the Phasor diagram refer to Figure 10 below. Additionally, the authors calculations for 25 Hz below resonance is shown below:

Figure 11 - 25Hz Above Resonance Phasor Diagram

Equation 44 - Frequency

34.59+25=59.59Hz

Equation 45 - Xc Calculation

Xc=12π 59.59(318.02×10-6)=8.40 Ω

Equation 46 - Xl Calculation

Xl=2π×59.59×115×10-3=43.06 Ω

Equation 47 - ZLR

ZLR= 252+43.062 =49.80Ω

Equation 48 – ILR Calculation

ILR=299.8149.80=6.02A

Equation 49 – Ic Calculation

IC=VCXC=299.818.40=35.69A

Equation 50 - ∅LR Calculation

∅LR=Tan-1 43.06 25=59.86°

Equation 51 - ∅ Calculation

∅=Tan-1 6.02 (Sin 59.86)~ 35.69 6.02 Cos 59.86=84.34° [Leading]

Equation 52 - ILR  Sin ∅LR Calculation

ILR Sin ∅LR=6.02 Sin 59.86=5.21A

IC>ILR Sin ∅LR

Equation 53 – I Calculation

I= ILR Cos∅LR2+ ILR Sin∅LR~IC2=30.48A

## 50 Hz

For the Phasor diagram refer to Figure 11 below. The authors calculations for 50 Hz is shown below:

Figure 12 - Phasor Diagram For 50Hz

Equation 54 - Xc Calculation

Xc=12π 50(318.02×10-6)=10.01Ω

Equation 55 - Xl Calculation

Xl=2π×50×115×10-3=36.13 Ω

Equation 56 - ZLR Calculation

ZLR= 252+36.132 =43.94Ω

Equation 57 - ILR Calculation

ILR=299.8143.94=6.82A

Equation 58 - Ic Calculation

IC=VCXC=299.8110.01=29.95A

Equation 59 - ∅LR Calculation

∅LR=Tan-1 36.13 25=55.32°

Equation 60 - ∅ Calculation

∅=Tan-1 6.82 (Sin 55.32)~ 29.95 6.82 Cos 55.32=80.94° [Leading]

Equation 61 - ILR  Sin ∅LR Calculation

ILR Sin ∅LR=6.82 Sin 55.32=5.61A

IC>ILR Sin ∅LR

Equation 62 - I Calculation

I= ILR Cos∅LR2+ ILR Sin∅LR~IC2=24.65A

# Discussion

The author has calculated a capacitor value to correct the power factor to ≥0.95, see section 3.2.1. Below the author has described how this relates to:

## How the PFC relates to Resonance of the Circuit

Refer to section 2.6 for a description of PFC, also, refer to section 2.4 for a description of resonance. However, in regards to how PFC relates to resonance of the circuit, if the PFC is installed in a system with non-linear loads, the capacitors can cause harmonic resonance [21]. This occurs when the system inductance and the capacitance that has been added creates a circuit, which causes the flow of harmonic currents [22]. This will result in high voltage and current at the resonant frequency, which could overload the capacitors and lead to a failure [22]. The self-healing process of PFC capacitors can change the capacitance, creating a damaged resonant frequency [22]. A graph showing Resonance in PFC is shown below in Figure 13.

Figure 13 - Resonance in PFC [22]

## How the PFC relates to Achieving Maximum Power Transfer

Refer to section 2.6 for a description of PFC and refer to section 2.5 for a description of Maximum Power Transfer. In regards to how PFC relates to achieving maximum power transfer, its important that the power factor be as close to unity as possible [23]. This is because, when the power factor is less than 1, the current and voltage are not in phase [10], as a result less of the supplied power is being used efficiently and potentially disruptions to the device can occur [23], which can have negative effects from a business’s commercial aspect, as it can lead to increased operational costs, due to utility charges and distribution losses [24]. However if the PFC is utilised, the efficiency will be improved, due to lower distribution losses, as this will lower energy consumption also prolonging equipment’s lifespan [24], which will mitigate this risk.

## How the PFC relates to Energy Efficiency

Refer to section 2.6 for a description of PFC, Energy Efficiency is providing a service with the same or better quality, while using less energy [25]. Energy Efficiency is imperative in today’s society and on any business [26] [27]. Some Society benefits include, reduce climate change for later generations, Economic growth, provide Jobs, Affordability and Public Health [28]. For Businesses, some benefits include, Cost savings, Economic growth and following Government Legislations [29] [30]. Regarding how PFC relates to energy efficiency, PFC is tremendously important in improving energy efficiency as when PFC is applied, it will create a high Power Factor, a high power factor, as explained in section 2.6, it will increase energy efficiency, this is as it reduces energy losses [31]. Additionally, if there is a high Power Factor, it can increase the efficiency of electrical appliances, make them more reliable for use in the future [32], making them more sustainable and energy efficient.

## Why is Electricity Three-Phase within Developed Countries?

Electricity in the United Kingdom and throughout the developed world, is transmitted on a three-phase basis, it’s the method used by the national grid [33]. This because, its more efficient than single phase in regards to power, as they can produce more power with less current [34]. Additionally, they are proficient at power distribution, as they can distribute power over long distances without the need for additional transformers [35].

## Why do systems generally not go beyond 3 phases?

However, as useful as a 3 -Phase system is, a system using more than 3 phases are not used for several reasons, for instance the cost effectiveness, as it would require a greater number of electrical components, which would come to a much larger cost [36]. Additionally, there would be limited gain as 6 & 12-Phase Systems do exist in rare cases, for example very high voltages. For normal voltages, they would not be suitable for power transmission [37], however they have an issue with dynamic overvoltage’s [37].

## Significance of PF and PFC in Electrical Engineering Systems

For an description of PF refer to section 2.3. For a description of PFC refer to section 2.6. Electrical Engineering systems refers to an electrical system that manage and distribute electrical power [38]. Electrical systems engineering covers the design, manufacture and maintenance of electrical systems [39]. These are used in many different industries, for example; construction, aviation, domestic, aerospace, motorsport, and many more [40]. The significance of PF in relation to Electrical Engineering Systems, is that it is very critical, as it indicates the efficiency of electrical power usage, as detailed in section 2.3. Additionally, PF is the relationship between active and apparent power and is essential for optimising the efficiency of electrical systems and it can prevent issues in the systems, for example, energy losses and conductor overheating [41]. The results of these can lead to equipment failure and higher utility charges [41], which for a business are risks they would like to mitigate, as a result the PF plays a significant role in electrical systems. The significance of PFC in relation to Electrical Engineering Systems, is that it is very important, as PFC is the method used to increase the power factor, to make it as close to 1 as possible, as detailed in section 2.6. PFC is significant in relation to Electrical systems as, if the PF is increased due to the PFC, this means the issues highlighted in regards to a lower PF (energy losses and overheating), wont occur and would be a method of eliminating/mitigating these risks.

# Conclusion

From the experiment, it has been determined that, when the capacitor value increases, so does the Power Factor, however the phase angle will decrease, this is also true in vice-versa, this is in relation to PFC. Additionally, with a higher frequency, this will create a higher Power Factor value. Additionally, the author has created a RLC Circuit, via PFC, using the value calculated for the capacitor to do this. Using this value the author calculated the component voltages and current for the RLC Circuit, using phasor diagrams to demonstrate the values. Additionally, explaining the relation between PFC and different subjects within electrical engineering. While also describing PF and PFC and explaining its significance to electrical engineering systems.

# References

| [1] | J. C. Rawlins, “Preface,” in Basic AC Circuits, Second Edition, Newnes, 2000, p. VIII. |
| --- | --- |
| [2] | K. P. Wong, “Direct Current and Alternanting Current Systems,” in Electrical Engineering - Volume I, Singapore, EOLSS Publications, 2009, p. 108. |
| [3] | I. Torriente-García, F. M. Muñoz-Pérez, J. C. Castro-Palacio and J. S. Monsoriu, “The internal resistance of a non-ideal inductor in an RLC series circuit at resonance,” Physics Education, vol. 59, no. 4, 2024. |
| [4] | C. M. Coman, A. Florescu and C. D. Oancea, “Improving the Efficiency and Sustainability of Power Systems Using Distributed Power Factor Correction Methods,” 2020. |
| [5] | RapidTables, “Power Factor,” RapidTables, [Online]. Available: https://www.rapidtables.com/electric/Power_Factor.html. [Accessed 4 January 2026]. |
| [6] | M. R. Stoker, “Principles of pressure transducers, resonance, damping and frequency response,” Anaesthesia & Intensive Care Medicine, vol. 5, no. 11, pp. 371-375, 2004. |
| [7] | M. Gilbert, “Principles of pressure transducers, resonance, damping and frequency response,” Anaesthesia & Intensive Care Medicine, vol. 13, no. 1, pp. 1-6, 2012. |
| [8] | S. Roberts, “Conjugate-Image Impedances,” Proceedings of the IRE, vol. 34, no. 4, pp. 198-204, 1946. |
| [9] | M. Pinuela, D. C. Yates, S. Lucyszyn and P. D. Mitcheson, “Maximizing DC-to-Load Efficiency for Inductive Power Transfer,” IEEE Transactions on Power Electronics, vol. 28, no. 5, pp. 2437 - 2447, 2012. |
| [10] | J. Ware, “Power Factor Correction,” IET, 2006. |
| [11] | B. McDonald and B. Lough, “Power Factor Correction (PFC) Circuit Basics,” in Texas instruments power supply design seminar, Texas , 2020. |
| [12] | Nokian Capacitors, “Power Factor Correction,” Tampere, Nokian Capacitors, 2002, p. 3. |
| [13] | W. V. Lyon, Problems in electrical engineering, New York: McGraw, 1908. |
| [14] | S. Pajic and A. E. Emanuel, “Modern apparent power definitions: theoretical versus practical Approach-the general case,” IEEE Transactions on Power Delivery, vol. 21, no. 4, pp. 1787 - 1792, 2006. |
| [15] | R. Fetea and A. Petroianu, “Reactive Power: A Strange Concept?,” in Second European Conference on Physics Teaching In Engineering Education, Cape Town, 2000. |
| [16] | S. Kumar, A. Dutt, S. Hemraj, S. Bhat and B. Manipadybhim, “Phase Angle Measurement in Healthy Human Subjects through Bio-Impedance Analysis,” Iran J Basic Med Sci, vol. 15, no. 6, p. 1180–1184, 2012. |
| [17] | J. W. Kolar, F. Krismer, Y. Lobsiger, J. Muhlethaler, T. Nussbaumer and J. Minibock, “Extreme efficiency power electronics,” in 2012 7th International Conference on Integrated Power Electronics Systems (CIPS), Nuremberg, 2012. |
| [18] | A. Emanuel, “On the definition of Power factor and apparent power in unbalanced polyphase circuits with sinusoidal voltage and currents,” IEEE Transactions on Power Delivery, vol. 8, no. 3, pp. 841 - 852, 1993. |
| [19] | A. Bhatia, “Power Factor in Electrical Energy,” CEDengineering.com, [Online]. Available: https://www.cedengineering.com/userfiles/E04-014%20-%20Power%20Factor%20in%20Electrical%20Energy%20Management%20-%20US.pdf. [Accessed 23 December 2025]. |
| [20] | “Power Factor Correction,” Electronics Tutorials, [Online]. Available: https://www.electronics-tutorials.ws/accircuits/power-factor-correction.html. [Accessed 23 December 2025]. |
| [21] | Y. C. Arif, L. P. S. Raharja and S. Auladah, “Design and Implementation of Hybrid Filter for Harmonic Reduction and Damped Resonances in Non-Linear Loads,” IOP Conference Series: Materials Science and Engineering, vol. 1096, no. 1, 2021. |
| [22] | Fuseco, “Why is Resonance a Problem in Power Factor Correction?,” Fuseco, [Online]. Available: https://www.fuseco.com.au/articles/why-is-resonance-a-problem-in-power-factor-correction. [Accessed 31 December 2025]. |
| [23] | CUI INC, “Power Factor and Power Factor Correction,” Belfuse, [Online]. Available: https://www.belfuse.com/media/documents/whitepapers/power-factor-and-power-factor-correction.pdf. [Accessed 1 January 2026]. |
| [24] | S. Velickovic, “Ultimate Guide To Power Factor Correction,” 29 April 2025. [Online]. Available: https://infinitemathworld.com/ultimate-guide-to-power-factor-correction/. [Accessed 1 January 2026]. |
| [25] | Stanford University, “Energy Efficiency,” Stanford University, [Online]. Available: https://understand-energy.stanford.edu/energy-resources/renewable-energy/energy-efficiency. [Accessed 3 January 2026]. |
| [26] | M. Yang and X. Yu, Energy efficiency: Benefits for environment and society, Springer, 2015. |
| [27] | C. Russell, “Multiple Benefits of Business-Sector Energy Efficiency: A Survey of Existing and Potential Measures,” American Council for an Energy-Efficient Economy , 2015. |
| [28] | European Union, “The many benefits of energy efficiency,” European Union, [Online]. Available: https://build-up.ec.europa.eu/en/resources-and-tools/publications/many-benefits-energy-efficiency-report-IEA. [Accessed 3 January 2026]. |
| [29] | F. Godemel, “Why Energy Efficiency Is A Strategic Imperative For Modern Businesses,” Forbes Business Development Council, 2025. |
| [30] | Energy 2 Businesses, “UK Energy Regulations for Businesses: A Comprehensive Guide,” Energy 2 Businesses, [Online]. Available: https://www.e2b.uk/uk-energy-regulations-businesses-guide/. [Accessed 3 January 2026]. |
| [31] | Turn2Engineering, “Power System Efficiency,” Turn2Engineering, [Online]. Available: https://turn2engineering.com/electrical-engineering/power-systems-engineering/power-system-efficiency. [Accessed 3 January 2026]. |
| [32] | M. Howard, “Understanding The Importance Of PFC In Energy Efficiency,” 2025. |
| [33] | A. Snow, “The first National Grid,” Engineering Science and Education Journal, vol. 2, no. 5, 1993. |
| [34] | E. C. d. Santos, C. B. Jacobina, E. R. Cabral da Silva and N. Rocha, “Single-Phase to Three-Phase Power Converters: State of the Art,” Transactions on Power Electronics, vol. 27, no. 5, pp. 2437 - 2452, 2012. |
| [35] | S. Chimklai and J. R. Marti, “Simplified three-phase transformer model for electromagnetic transient studies,” IEEE Transactions on Power Delivery, vol. 10, no. 3, pp. 1316 - 1325, 1995. |
| [36] | J. Laghari, H. B. Mokhlid, A. H. Abu Bakar and H. Mohamad, “A comprehensive overview of new designs in the hydraulic, electrical equipments and controllers of mini hydro power plants making it cost effective technology,” Renewable and Sustainable Energy Reviews, vol. 20, pp. 279-293, 2013. |
| [37] | J. R. Stewart and D. D. Wilson, “High Phase Order Transmission--A Feasibility Analysis Part II--Overvoltages and Insulation Requirements,” IEEE Transactions on Power Apparatus and Systems, Vols. PAS-97, no. 6, pp. 2308-317, 1978. |
| [38] | S. W. Blume, Electric power system basics for the nonelectrical professional, 3rd Edition, IEEE Press Series on Power and Energy Systems, 2025. |
| [39] | Workbred, “Electrical Systems Engineer: Key Skills, Qualifications and Career Path,” Workbred, [Online]. Available: https://workbred.co.uk/career/electrical-systems-engineer. [Accessed 3 January 2026]. |
| [40] | R. Milsom, “What jobs can you do with an electrical engineering degree?,” Careers Advice and Planning, 2025. |
| [41] | G. E. Portal, “Power Factor: Concept, Importance, and Impacts,” Global Energy Portal, [Online]. Available: https://globalenergyportal.com/power-factor-concept-importance-and-impacts-2/. [Accessed 4 January 2026]. |