Momentum and Energy revision and Exercise Sheet 1

Revision:

Momentum =  mass  (kg) x velocity (ms-1)

Conservation of Momentum Law. Total linear momentum of two  bodies colliding is conserved.

M2

m1

M2

m1

u1

v2

u2

v1

Before:							After:

Conservation of Momentum :	 m1u1 + m2u2  =  m1v1  +m2v2

Newton’s Second Law:

Force =  mass  x acceleration

F= ma         can  be rewritten in terms of momentum change since acceleration is just rate pf change of velocity:

F=m(v-u)t

where u  is initial velocity and v is final velocity after time t

i.e.		   F= (mv-mu)t      or   Ft = mv-mu

Ft is called the Impulse  of the force

Exercise:

- A constant  force of 20N is applied for 2 s to  a body of mass 0.2 kg that is initially at rest. What is the final speed of the body?

- A person of mass 80kg jumps off a wall and lands on the ground with a velocity of 10 ms-1. Calculate the force on the person if:

  - They don’t flex their knees on landing and come to  a rest quickly  in 0.1 s

  - They flex their knees and come to a rest over a longer duration of 1 s.

- A gas-gun applies a constant excess pressure of 2 bar for 0.5 s  to  a 40g stationary projectile inside a barrel  of cross sectional area 12 cm2 .  What is the final velocity of the projectile?      (1 bar =105 Pa and remember all values in SI units.)

Revision:

Kinetic energy of mass  m with velocity  v is   12mv2

Elastic and Inelastic collisions:

In collisions the total momentum is always conserved. However, the kinetic energy might not be conserved if some of the collision energy is converted into heat or sound energy.

Elastic collisions :  kinetic energy is conserved  eg two  smooth snooker balls colliding is approximates an elastic collision.

Inelastic Collisions: some of the initial kinetic energy is converted into  non-recoverable heat/mechanical/sound energy.

Coefficient of restitution: In practice some energy is lost in a collision so that the velocity of separation does not equal  the velocity of approach.

Coeff of restitution=  e = velocity of separationvelocity of approach

e can vary from 0, where the two  bodies stick together , to  1 where the objects are very hard and the collision is perfectly elastic.

Exercise:

- A ball of mass 0.1 kg and velocity 30ms-1 hits a stationary ball of mass 0.2 kg head-on in an elastic collision. What are the velocities of the two balls after the collision. (Apply conservation of momentum law and conservation of energy law)

- A bullet of mass 50 g is fired from a rifle of mass 2kg with a velocity of 500 ms-1. Calculate:

  - the recoil velocity of the rifle.

  - The kinetic energy of the bullet

Work Done (by a force)

Work Done =  Force  x distance moved in direction of force

When force is 1 N and distance moved is 1 m then the energy transferred to the object is 1 J

Example:

- A bullet of mass 8 x 10-3 kg moving at 320 ms-1 penetrates  an object to  a depth of 10mm before coming to rest Find the resistive force  offered by the target assuming it is uniform.

Potential Energy of mass m at height h

WD to  raise mass m from ground to height h is force x distance =  mgh

Example:

- A  body of mass 200g is dropped vertically from a height of 1.5m . What is the velocity at time of impact? (apply Conservation of energy law) . Assume g=10ms-2

Further exercises:

- A body of mass 10kg is on an inclined plane 30o   to the horizontal. The coefficient of friction between the body and the plane is 0.7  . Determine:

  - The friction force between the body and the plane.

  - The net force on the body parallel to  the plane

  - The acceleration of the body parallel to the plane.

Assume g=10 ms-2

- Find the resultant force at the point P:

20N

45o

30o

10N

- Two masses are suspended under a beam as shown below. Determine the distance d to  maintain equilibrium of the beam.

d

30cm

1.5kg

7kg

- A vertically suspended  steel wire of cross section area 0.1 cm2   and original length 1m is loaded at the lower end  with a mass of 10kg. If the modulus of elasticity E = 50x 109  calculate:

  - The stress experienced by the wire

  - The new length of the wire.