Examination of Capacitive discharge curve

Download and run the Capacitor discharge curve proteus lab from Moodle.

- Use the circuit in Lab “Capacitive discharge curve”

- Close the switch and run the circuit for 1 sec by accessing the debug menu and clicking on run simulation (timed breakpoint) and enter 1 sec in the box and press ok.

- Take a reading and fill in the box below.

- Open the switch

- Access the debug menu and click on run simulation (timed breakpoint) and enter 10 sec in the box and press ok.

- Take a reading and fill in the box below.

- Repeat actions 5 and 6 until the reading is below 0.05V

- Transfer the numbers to a spread sheet and graph the values.

| 1 | 11 | 21 | 31 | 41 | 51 | 61 | 71 | 81 | 91 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 101 | 111 | 121 | 131 | 141 | 151 | 161 | 171 | 181 | 191 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 201 | 211 | 221 | 231 | 241 | 251 | 261 | 271 | 281 | 291 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 301 | 311 | 321 | 331 | 341 | 351 | 361 | 371 | 381 | 391 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Draw the graph in the box below

Questions

- What is the purpose of the 1K resistor in series with the capacitor?

- The voltage should drop to 37% of the initial voltage in one time constant. Calculate 37% of the initial voltage.

- Use your graph to estimate the time taken for the voltage to drop to 37% of the initial voltage.

- Use the time constant equation (t=CR) to calculate the resistance of the resistor the capacitor has discharged through. The capacitance of the capacitor is 100 *10-6 F.

- How does the calculated value of the resistance compare with the stated value of 470KΩ? Explain any discrepancy.