Engineering Science.

Friction Examples.

- It is found that a vehicle having a mass of 900kg can be moved slowly and steadily along a horizontal surface by a force of 5.8kN with the wheels locked.  Calculate the coefficient of friction.			 [0.66]

R

F

mg

- A metal block having a high friction lining and having a mass of 4.5kg requires a horizontal pull of 15.74N to move it at a steady speed along the horizontal steel surface.  What is the coefficient of friction for the high friction lining and Steel.					           [0.36]

R

F

mg

- A train wagon has a coefficient of friction of 0.35 and requires a force of 1500N to overcome friction and keep it moving at a steady speed.

- What is the mass of the wagon?        				    [436.87kg]

- If the number of wagons being pulled by a train is 8, what will the force exerted by the train need to be in order to overcome friction?

[11999.74N]

a)

b)

Total mass of 8 wagon’s=8x436.87=3494.98kg

v (m/s)

A 50 kg weight being pulled at a constant velocity up a plain angled at 30 degrees from the horizontal has a coefficient of friction of 0.25.  Calculate the total force acting down the plain.                         [351.45N]

R

θ

mgSinθ

θ

mg

mgCosθ

Resolving forces: