Conrod Dynamic Forces Report

Toby Marshall

Student No. 1702695

The aim of the report is to consider the forces which the conrod undergoes whilst the engine is in motion. The report will look in to how certain circumstances alters the velocity, height, acceleration and force on the conrod. The report will focus mainly on the maximum tensile force which the conrod is subject to. The information in the report has be sourced from research or calculated by a formula from an excel document which will be used in conjunction when completing and gathering evidence for the report.

Contents:

- Introduction										P3

- Research										P3

- Methodology									P3

- Analysis

4.1 Analysis of Standard Engine Values					P3

4.2 Analysis of Conrod when RPM is 2600					P4

4.3 Analysis of Conrod when Connecting Rod length is 140mm		P6

4.4 Analysis of Conrod when Piston Mass is 0.350kg 			P7

4.5 Analysis of Conrod when Crankshaft Radius is 38mm			P9

4.6 Analysis of Conrod when RPM is 2600, Conrod length is 140mm, Piston Mass is 0.350kg, and Crankshaft Radius is 38mm		          P10

- Conclusion								                     P12

- References 									          P12

Figure List

Figure 1 – Graph of velocity with Standard Engine Values	4

Figure 2 – Graph of height with Standard Engine Values	4

Figure 3 – Graph of acceleration and force with Standard Engine Values	4

Figure 4 – Graph of velocity when RPM is 2600	5

Figure 5 – Graph of height when RPM is 2600	5

Figure 6 – Graph of acceleration and force when RPM is 2600	6

Figure 7 – Graph of velocity when Connecting Rod length is 140mm	6

Figure 8 – Graph of height when Connecting Rod length is 140mm	7

Figure 9 – Graph of acceleration and force when Connecting Rod length is 140mm	8

Figure 10 – Graph of velocity when Mass is 0.350kg	8

Figure 11 – Graph of height when Mass is 0.350kg	9

Figure 12 – Graph of acceleration and force when Mass is 0.350kg	9

Figure 13 – Graph of velocity when Crankshaft Radius is 38mm	10

Figure 14 – Graph of height when Crankshaft Radius is 38mm	10

Figure 15 – Graph of acceleration and force when Crankshaft Radius is 38mm	11

Figure 16 – Graph of velocity when RPM is 2600, Conrod length is 140mm, Piston Mass is 0.350kg, and Crankshaft Radius is 38mm	11

Figure 17 – Graph of heght when RPM is 2600, Conrod length is 140mm, Piston Mass is 0.350kg, and Crankshaft Radius is 38mm	12

Figure 18 – Graph of acceleration and force RPM is 2600, Conrod length is 140mm, Piston Mass is 0.350kg, and Crankshaft Radius is 38mm	12

1. In this document, this report will look at the forces that a connecting rod (conrod) undergoes inside a Land Rover Discovery’s engine. Specifically looking at how the speed of the engine shown in Revs per minute (RPM), piston mass, length of the conrod and crank radius will affect the different forces which the conrod undergoes when in motion.

2. Given information researched about a Land Rover Discovery fitted with a 4.0L V8 engine which has a Crank radius of 29.21mm (wheeljack888, 2006 ), Conrod length of 155.067mm (transientmechanic, 2009), Piston mass of 581g (mgbloke, 2007), and a max RPM of 4750 (Anon., n.d.). I then used these values in a spreadsheet so I could calculate the force that the conrod endures whilst the engine is at a certain RPM.

3. Whilst the engine is under load, analysis considering the different forces and conditions which the conrod is subjected to, will be monitored and reviewed.  An excel document will be used to perform calculations, generating a set of graphs and results containing information about the force, acceleration, velocity and height of the conrod. The graphs and results will help compare the effect on the conrod when the values are altered, the outcomes to be compared.

4.1 The figures for the engine were inserted; a crank radius of 29.21mm, conrod length of 155.067mm, piston mass of 581g, and a max RPM of 4750. These values were calculated in the excel document to produce the following set of graphs and values.

Figure 1

The velocity of the conrod is clearly presented in the graph and the information clearly demonstrates the speed and position of the conrod. The graph indicates the conrod accelerating to -14.7572 m/s at its peak. The conrod decelerates back to 0, at this point the piston is at BDC (Bottom Dead Centre), where it stays momentarily. Furthermore, the conrod continues to accelerate to 14.77026 m/s at its peak, demonstrating the conrod is travelling up the cylinder in a positive velocity. According to the figures, the velocity reduces as the piston reaches TDC (Top Dead Centre) and the value of velocity becomes 0. Considering evidence, the graph demonstrates the velocity is identical throughout the strokes of the engine.

Figure 2

Height shown in this graph demonstrates difference in height between the top of the conrod and the crankshaft. The max height of 184.2727mm this will be at TDC as the top of the conrod is at the furthest point away from the crankshaft. And when the height was 125.865 the top of the conrod is as close to the crank as possible and this will be BDC. It can be seen that the travel of the conrod from its lowest point to is highest point is 58.4077 from BDC to TDC.

Figure 3

The acceleration shows the piston is accelerating at a slower speed when the piston is travelling up the cylinder this is because the piston is compressing the combustion mixture on the compression stroke which is slowing down the acceleration of the piston. When the piston is travelling down the cylinder it is on the power stroke and the ignition of the combustion mixture is pushing the piston down, therefore a greater force is happening to the conrod and increases the acceleration, the duration of the maximum acceleration is much shorter as there is no combustion mixture to slow the piston down.

As above the force on the conrod reacts similar to the acceleration of the conrod. On the compression stroke the force is at a plateau due to the conrod reaching its maximum acceleration as the compression of the mixture in the cylinder is slowing the conrod down. Whereas when the engine is on the power stroke the force on the conrod is greater. The force is greater but lasts for a shorter duration, this the maximum tensile force on the conrod, 4983.143.

4.2 The RPM value altered in the excel document to analyse effects of the forces it causes on the conrod during the engines rotation. The RPM value was changed from its maximum of 4750 to 2600, the RPM where maximum torque is produced by the engine.

Figure 4

According to the graph the velocity has altered with the change of the RPM, peak velocity is -8.09196 and happens between TDC and BDC where the value is 0. The velocity of the conrod then accelerates from TDC where it peaks at 8.0904 before decelerating back to BDC. The velocity of the conrod is lesser when the RPM is reduced to 2400 from 4750, the time between BDC and TDC is longer.

Figure 5

The height shown in this graph demonstrates difference between the top of the conrod and the crank shaft. The height of conrod has remained identical to the previous graph down to the fact that a change in RPM doesn’t have any effect on it.

Figure 6

The acceleration of the conrod has been greatly affected by the change in RPM, the graph shows that the acceleration of the conrod travelling up the cylinder has been reduced with the maximum acceleration only reaching 1757.27. This deceleration of the conrod is a result of the engine speed being slower due to the decrease in RPM. The acceleration of the conrod as it travels down the cylinder is greater, reaching -2571.53. The power stroke forces the conrod and piston down the cylinder resulting in extra inertia which accelerates the conrod faster than when it is traveling on the compression stroke.

The force still follows a similar wave form as the acceleration, but the force experiences different values. From examining the findings, the maximum tensile force on the conrod has altered with the change in Rpm, 1494.06 is only a fraction of the force which the conrod was experiencing when it was underload at 4750 RPM. Much like the acceleration the force on the conrod was smaller on the compression stroke than the force of the conrod when travelling down the cylinder on the power stroke. The force is less than it appears in the previous graph but it follows the same wave, this displays that the tensile forces are happening to the conrod at the same point in the engine cycle but the force is less due to the lower engine speed.

Figure 7

4.3 The conrod height value was altered in the excel document to analyse effects it causes to the forces on the conrod during the engines rotation. The conrod‘s height was changed from 155.067mm to 140mm.

The velocity is clearly represented in the graph, allowing for detailed analysis of the data for comparison. The velocity accelerates from BDC and its peak velocity reaches 14.8345. Whereas when compared to the velocity of the conrod in standard circumstances the peak is 14.83613. The small change is enough to alter the balance of the engine and the efficiently of its running. When the conrod travelled from TDC it displayed a similar velocity of 14.8183, whereas the conrod in standard circumstances is 14.77026.

Figure 8

This graph displays the distance between the crankshaft centre and the top of the conrod, monitoring how it changes as the engine rotates. The height has been heavily affected by the change in conrod length. The maximum distance the conrod reaches from crankshaft centre is 169.2056mm at TDC, with the minimum being 110.8103mm at BDC. When calculated it meant that the conrod in traveling 58.3953mm between TDC and BDC. The values for TDC and BDC differ from the original graph by the same amount that the conrod has been shortened by, but the distance it travels from its nearest point and it furthest point from the crank shaft has stayed identical to the one decimal place.

The acceleration of the conrod has been greatly affected by the change in conrod length, the graph shows that the acceleration of the conrod travelling up the cylinder is very similar to the acceleration seen in the original graphs, with the acceleration peak being 5716.99 and the original acceleration being peak 5862.599.  The acceleration has had little change as the conrod length has not alter the speed at which the conrod is travelling, but the acceleration is smaller than the original graph, this could be due the conrods shorter length not having as much inertia decreasing the acceleration. As with the original graph the acceleration is bigger when the piston is travelling down the cylinder on the power stroke as the combustion is forcing the piston and conrod down, this acceleration is -8722.85. When compared to the original graphs acceleration which was -8576.84, whilst very similar figures the acceleration of the shortened conrod has a larger peak

Figure 9

acceleration. The small difference could also be that the computer time interval on the excel document are too big and the exact point the acceleration peaks may be missed.

Same as the previous graphs produced the force has a similar waveform to the acceleration, this is as the conrod is accelerated, the greater the acceleration the greater the force. The force is at first traveling upwards then back down to 0 as it reaches TDC, whilst peaking at 3321.571. The force which the original conrod received was 3406.17. When travelling down negatively to BDC it peaked at -5067.98 this being the maximum tensile force on the conrod. Whilst the force from the original conrod was -4983.14. The forces are very similar to each other, showing that the length of the conrod does not have a dramatic effect on the force on the conrod.

4.4 Piston mass was altered in the excel document to analyse effects it causes the forces on the conrod during the engines rotation. Piston mass has been changed from 0.581kg to 350kg.

Figure 10

The velocity shown in the graph, represent the motion of the conrod moving up and down the cylinder. The velocity is not effected by the change in piston mass, with the exact same values being presented as the original conrod in the first set of graphs. This represents that piston mass has not effect on the velocity of the conrod.

The graph shows the height between the top of the conrod and the centre of the crankshaft and how it changes as the conrod moves when the engine is rotating. The graph represents that the maximum height of 184.2727mm, this will be at TDC as the top of the conrod is at the furthest point away from the crank. Minimum height was 125.865 the top of

Figure 11

the conrod is as close to the crank as possible and this will be BDC. This results in a distance of 58.4077 from BDC to TDC. These values are identical to graph above, this suggests that the piston mass has no effect on the piston height and how far it travels between TDC and BDC.

Figure 12

The acceleration is displayed in the graph, the acceleration demonstrates how the conrod is traveling up the cylinder when travelling to TDC. The conrod reaches peak acceleration at 5862.599 and when it travels down the cylinder to BDC it reaches a peak acceleration of 8576.84. The acceleration is greater when the piston is traveling to BDC as the inertia from the power stoke is pushing the piston down. The acceleration of the conrod is identical to the acceleration which the conrod experienced when under standard engine conditions.

The force shown in the graph represents the force the conrod is subject to when the engine is rotating. The conrods peak force when traveling up towards TDC was 2051.91. Maximum tensile force on the conrod was 3001.893, the peak force when the conrod is travelling down the cylinder toward BDC. In comparison, the force on the conrod is lesser than the force when the piston was at its standard mass, this suggests that a lighter piston will reduce the force on the conrod.

4.5 The crankshaft radius was altered in the excel document to analyse effects it causes on the forces the conrod experiences during the engines rotation. The radius was changed from 29.21mm and increased to 38mm.

Figure 13

The velocity of the conrod is clearly presented in the graph and the information clearly demonstrates the speed and position of the conrod. The graph indicates the conrod accelerating to 19.45086 at its peak whilst travelling towards TDC where the velocity becomes 0. The conrod then travels in the opposite direction, as the conrod travels towards BDC the peak velocity is -19.4507. The velocity is consistent with its wave forms for when the conrod is travelling to BDC and TDC when compared to the values presented in the first graph. The velocity when the crank radius is increased, is greatly affected, due to a large increase in peak velocity.

Figure 14

The height represents the distance between the centre of the crankshaft and top of the conrod. The graph allows you to see how the maximum and minimum distance between the two values. The minimum distance from the crankshaft to conrod is 117.0922, with the maximum the crankshaft is from the conrod being 193.0612. This means that the distance between the two point which the conrod travels is 75.969. When compared to the first graph, the minimum value is smaller and maximum value is larger, this has resulted in a larger distance between TDC and BDC.

The acceleration measures how fast the conrod moves between TDC and BDC. The peak acceleration on the conrod when it was travelling from BDC to TDC was 7097.072. The conrod then accelerates from TDC to BDC where it has a peak acceleration of -11689. The acceleration from TDC to BDC is larger as this is when the power stroke is pushing the piston and conrod down the cylinder. The

Figure 15

change in crankshaft radius has made the acceleration increase compared against the original values from the first graphs.

The force acting on the conrod is displayed clearly in the graph, presenting clear evidence. The peak force acting on the conrod when it is traveling from BDC to TDC is 4123.399. The maximum tensile force, 6791.295 which is present when the conrod is traveling between TDC to BDC.

Figure 16

4.6 In the excel document every value has been altered to match the values which were altered in previous graphs, therefore RPM is 2600, Conrod length is 140mm, Piston Mass is 0.350kg, and Crankshaft Radius is 38mm.

The velocity of the conrod is clearly presented in the graph and the information clearly demonstrates the speed and position of the conrod. The velocity can be seen to accelerated from TDC to BDC and peaks at -10.7199. When travelling from BDC to TDC the velocity peaks at 10.7177. The peak velocity has decreased dramatically when compared to the first graphs calculations, this is may be down to the reduction in rpm from 4750 to 2600.

Figure 17

The height represents the distance which the top of the conrod moves from the crankshaft. The minimum distance which top of the conrod is away from the crankshaft is 102.0039, the maximum distance is 177.9894. The change is values when compared to the original graphs is due to the alteration in conrod length and the alteration of the crank radius which has moved the position of where the top of the conrod is located when the engine is rotating.

Figure 18

The acceleration of the conrod can be seen in the graph, this allowing the acceleration to be monitored when it travel from TDC and BDC. The peak acceleration when the conrod was traveling from BDC to TDC was 2053.616. When the conrod was travelling from BDC to TDC the peak acceleration was -3578.96. The acceleration of the conrod when travelling from BDC to TDC shows the acceleration plateaued out, this mean that the conrod is reaching its peak acceleration and it can’t accelerate anymore. When the conrod is traveling from TDC to BDC the conrod reaches a larger peak acceleration is due the piston having extra downward force on it from the combustion on the power stroke. The conrod only reaches its peak acceleration momentarily as it is still accelerating up to the point where it starts to decelerate as it reaches BDC.

The force shown in the graph represent the force which the conrod undergoes when the engine rotates under load. The force changes as the conrod moves in the cylinder, the force follows a near identical wave form to the acceleration. When the conrod was travelling from BDC to TDC the maximum force which it peaked at was 718.7505, and when travelling from TDC to BDC it peaked at 1252.636 which is the maximum tensile force which the conrod experiences. This force is small when compared to results from previous graphs this is down to the decrease in RPM which has reduced the speed the conrod is travelling at.

5 In conclusion it seems that many different changes to the circumstance of the engine can alter the tensile force which the conrod experiences. The main reduction in force was due to the decrease in RPM, this also would help calculate the maximum RPM an engine could reach which out going over the maximum tensile force which the conrod can endure. The height of the conrod also changed the distance from the crankshaft which can help if building race engine the height from the crankshaft needed to be calculated for the engine block to be decked accurately to allow for a higher compression ratio. The change in height of the conrod had little effect on the maximum tensile force. The change in piston mass reduced the maximum tensile force on the conrod, it is common of a competition piston to be lighter, as a lighter piston reduces the force which allows a higher maximum RPM to be achieved. The change in crankshaft radius increase the maximum tensile force acting on the conrod, this means that an increase in diameter of the crank shaft is not beneficial to reducing the force acting on the concord.

# References

Anon., n.d. thecaryoudrive.com. [Online] 
Available at: http://www.thecaryoudrive.com/en/117329e079
[Accessed 20 11 2017].

mgbloke, 2007. v8 forum. [Online] 
Available at: http://www.v8forum.co.uk/forum/viewtopic.php?t=7731
[Accessed 20 11 2017].

transientmechanic, 2009. Private4x4.com. [Online] 
Available at: http://www.pirate4x4.com/forum/land-rover/760208-what-compression.html
[Accessed 20 11 2017].

wheeljack888, 2006 . Piston Heads. [Online] 
Available at: https://www.pistonheads.com/gassing/topic.asp?h=0&f=8&t=252839
[Accessed 20 11 2017].