| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 5 | LEFEL 5 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2022 | mai 2022 |

MODULE CODE: GOEE5000

Electrical Power

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator &Formula booklet

Question One (25 marks)

- The three-phase circuit shown in Figure 1 is unbalanced. Determine the current in the neutral line. 									[15 marks]

Figure 1

- The three-phase circuit shown in Figure 2 is balanced. Replace the circuit with its single-phase equivalent circuit, obtain the line current Ia from the single-phase analysis then determine the line currents Ib and Ic.

[10 marks]

Figure 2

Question Two (25 marks)

In Figure 3 shown below, determine:

- The total impedance of the circuit					          [10 marks]

- The total current of the circuit 					           [4 marks]

- The complex Power S 						           [5 marks]

- The total real power P							           [3 marks]

- The reactive power Q							           [3 marks]

Figure 3

Question Three (25 marks)

- A balanced Delta (Δ) connected load having impedance 20-j15 Ω is connected to a Delta (Δ) connected, positive sequence generator having  Vab=330 ∠ 0° , as shown in Figure 4 below. Calculate:

- The phase voltages Vab, Vbc, Vca

- The phase currents IAB, IBC,ICA

- The line currents Ia, Ib,Ic

[15 marks]

Figure 4

- Three impedance of 4+j3 Ω are Delta (Δ) connected and tied to a three-phase 208V power line as shown in Figure 5. Find the real power P, the reactive power Q, the apparent power S and the power factor of this load.

Figure 5

Question four (25 marks)

A load absorbs 100 kVA at lagging power factor of 0.6. Given that the voltage and the frequency of the source is 220V and 50Hz, compute the value of the capacitor that would correct the power factor to 0.85 if placed in parallel with the load.

End