University of Wales Trinity Saint Davids

Wales Institute of Science and art

Kinetic Energy Recovery

Alternative Propulsion Systems – ACHA5010

Cody Surridge

2304028

01/12/2024

Table of Contents

1	Introduction	2

2	Vehicle Selection	3

3	Dynamic Characteristics	3

4	WLTP and Tractive Forces	4

4.1	WLTP	4

4.2	Resistances	4

4.3	Kinetic Energy Recovery and Tractive Energy	4

4.3.1	Kinetic Energy Recovery	5

4.3.2	Tractive and breaking	5

5	KERS System	7

5.1	Largest Possible Power Input	7

5.2	Suitable KERS system	8

5.2.1	Errors within the system	8

5.3	Efficiency (70%)	9

6	Assumptions and Variations	10

6.1	Assumptions	10

7	References	11

Table 1- Properties of a Ferrari 599 GTB Fiorano	3

Table 2- Equations Used to Calculate the Resistances	4

Table 3- Some Points at Which Power < 0	5

Table 4-Showing Area of the above Curve in KW and KW/h	7

Table 5- Equation Used to find Area of Curve	7

Figure 1- Equation Used to Calculate Frontal Area	3

Figure 2- Graph showing a Class 3 WLTP	4

Figure 3- Graph Showing all of Rolling, Aerodynamic, Acceleration and Total Resistance Vs Time	4

Figure 4- Graph showing Power Vs Time	5

Figure 5- A graph Showing the Tractive Energy Vs Time	5

Figure 6- A Graph showing Braking Energy Vs Time	6

Figure 7- A graph comparing both Tractive and Braking Energy Vs Time	6

Figure 8- A Graph Showing Braking Energy Vs Time	7

Figure 9- A graph showing Braking Energy Vs Time @649s - 669s (Where KERs Power Input is the Maximum)	7

Figure 10- A Graph showing Braking energy output 70%	9

# Introduction

In this Report, I will be analysing the effects of introducing a Kinetic Energy Recovery System to reduce fuel consumption. This will be done by, selecting an appropriate vehicle so that I may research and identify its dynamic characteristics. The dynamic characteristics include the test mass, the frontal area, the coefficients of the rolling resistance and aerodynamic drag. From this data I will be able to select a suitable Worldwide harmonised Light vehicle Test Procedure (WLTP). I will then be producing spreadsheets that will allow me to plot graphs of both the cumulative braking and cumulative tractive energy required at the wheel over the WLTP. Next, I will be identifying the moments of the largest possible KERS. As a result, I can Specify a suitable KERS system and be able to size the components and therefore identify their efficiencies, allowing myself to plot the Kinetic Energy recovered due to wheel road contact. Finally, I will explain how the system is made and discuss any assumption I have made and any variations the driver will feel compared to conventional ICE and friction brakes vehicles.

# Vehicle Selection

I have Chosen a Ferrari GTB Fiorano. I have chosen this car because I believe that by adding a KERS it could help make it more fuel efficient by creating opportunities where it isn’t reliant on using its fuel. Additionally, by adding a KERS system it may help to create periods where the drive is smoother and more direct drive

# Dynamic Characteristics

| Test Mass ( (Specs, 2024) |  |
| --- | --- |
| Kerb Weight (kg) | 1690 |
| Passengers Weight (Kg) * | 78.45 |
| luggage (Kg) | 15 |
| Test Mass (Kg) | 1783.45 |
| Frontal Area (Specs, 2024) |  |
| Width (m) | 1.962 |
| Height (m) | 1.336 |
| Frontal Area (m^2) | 2.621232 |
| Rolling Resistance Coefficient |  |
| Rolling Resistance Coefficient | 0.013* |
| Coefficient of Aerodynamic Drag (Specs, 2024) |  |
| Coefficient of Aerodynamic Drag | 0.336 |

Table 1- Properties of a Ferrari 599 GTB Fiorano

*

(Ebrahimi, et al., 2018)

*In 2021, the mean weights of men and women based on adjustments to self-reported weight were 85.1kg and 71.8kg, respectively. (NHS, 2021)

| Purpose | Equation |
| --- | --- |
| Frontal Area (m^2) | Car Height X Width |

Figure 1- Equation Used to Calculate Frontal Area

# WLTP and Tractive Forces

## WLTP

I have Selected WLTP Class 3 for my Vehicle as the max speed for the Ferrari is 205Mph and the power this speed is 456KW (Specs, 2024). This then means that the ratio of rated power is 4560001783.85=260.67 W/Kg. This is Greater the 34 W/Kg required to be in Class 3.

Figure 2- Graph showing a Class 3 WLTP

## Resistances

Figure 3- Graph Showing all of Rolling, Aerodynamic, Acceleration and Total Resistance Vs Time

The Equations That were used are:

| Purpose | Equation |
| --- | --- |
| Rolling Resistance (N) | Mass X Gravity X Coefficient of Rolling Resistance |
| Aerodynamic Resistance (N) | 1/2 X Area X Density of Air X Aerodynamic Coefficient X (Velocity)^2 |
| Acceleration Resistance (N) | Force= Mass X Acceleration |
| Total Resistance (N) | The Sum of the Other Resistances |

Table 2- Equations Used to Calculate the Resistances

## Kinetic Energy Recovery and Tractive Energy

### Kinetic Energy Recovery

Figure 4- Graph showing Power Vs Time

| Purpose | Equation |
| --- | --- |
| Power (KW) | Force X Velocity / 1000 |

Kinetic Energy can be recovered when the Power is <0 Some Points Include:

Points Where Kinetic Energy Could be Recovered

35-55s

430-444s

888-907s

1117-1138s

1625-1642s

Table 3- Some Points at Which Power < 0

### Tractive and breaking

Tractive Energy happens when the power output is > 0

Figure 5- A graph Showing the Tractive Energy Vs Time

Braking energy happens when the Power output is < 0

Figure 6- A Graph showing Braking Energy Vs Time

This then allows me to compare both tractive and breaking force (by taking the braking force as a positive).

Figure 7- A graph comparing both Tractive and Braking Energy Vs Time

# KERS System

## Largest Possible Power Input

Figure 8- A Graph Showing Braking Energy Vs Time

Figure 9- A graph showing Braking Energy Vs Time @649s - 669s (Where KERs Power Input is the Maximum)

| Area | Units |
| --- | --- |
| 66.32482 | KWs or KJ |
| 0.018424 | KW/h |

Table 4-Showing Area of the above Curve in KW and KW/h

| Purpose | Equation |
| --- | --- |
| Area Under the Curve | 1/2 X Base X Height |
| Where Base = 2. Area = Height |  |

Table 5- Equation Used to find Area of Curve

## Suitable KERS system

The KERS system I have chosen for my vehicle is a flywheel. I have chosen a flywheel because it is a lightweight solution as “the flywheel is made from carbon filament wrapped round a steel hub and weighs 5kg” (Collins, 2011). This means that there will be very little change to the velocity and resistive forces due to a minimised change in weight. Additionally, the flywheel “can store up to 60kW” (Collins, 2011). Therefore, this is suitable for the Ferrari as the maximum braking energy it produces is 66.3kW. As a result, during the periods when the Ferrari cannot produce its maximum energy, the Fiorano should not produce any energy greater than 60kW and if it were to it would be a minimal quantity and should be easily dispersed by means such as heat transfer. Additionally, a flywheel braking system “has the efficiency of energy from braking to FES is 70% “ (Fui Tie & Wei Tan, 2013). This will affect the Power going back towards the car. This means that the fuel efficiency will slightly decrease compared to a flywheel that is 100% efficient. These losses will be caused by frictional losses throughout the system such as generating heat and sound.

### Errors within the system

Some problems with the system may be the size of the flywheel. This is an issue because I have chosen a very slim and compact car. This means that, I may struggle to comfortably fit a flywheel in the parameters of the Ferrari. To rectify this I would have to find a smaller way of storing energy such as a lithium ion battery which would be a more compact way of transferring and using the required energy as “a battery has a voltage of 12 volts and an ampere-hour rating of 50 Ah, its capacity would be 600 watt-hours (Wh) or 0.6 kWh (12V x 50Ah = 600Wh = 0.6 kWh).” (Sharma, 2024).  Despite this, a battery will be very heavy about “100kg to 550kg” (Sharma, 2024) for a full electric lithium-ion battery so it will be slightly light for a hybrid lower volt one. Moreover, the battery has a recharge rate is too large, this may cause an issue because the battery will not get to a full charge so either the battery will need to be significantly smaller or a different alternative will need to be found.

## Efficiency (70%)

Figure 10- A Graph showing Braking energy output 70%

# Assumptions and Variations

## Assumptions

- Road is Completely level with no gradient or curvature. Meaning that we are assuming that the car isn’t gaining any speed from gravitational potential energy. This also assumes that any time the car is braking isn’t for a corner but in a straight line.

- Taking the frontal area as Width X Height. The frontal area of a car is more complex than a square profile meaning that the area will be slightly smaller with the different shape. This will cause the aerodynamic resistance to decrease and therefore the overall resistance to decrease.

- The maximum energy point. This could be further along in the WLTP but by using visuals the point I had chosen looked to be the largest at a slower speed than it would be for a larger speed.

# References

Collins, S., 2011. Flywheel Hybrid Systems. [Online] 
Available at: https://www.racecar-engineering.com/articles/f1/flywheel-hybrid-systems-kers/#
[Accessed 29 November 2024].

Ebrahimi, K., Gao, Y., Longo, S. & Ehasani, M., 2018. Modern Electric, Hybrid Electric, and Fuel Cell Vehicles (Third Edition). Third Edition ed. Boca Raton: CRC Press.

Fui Tie, S. & Wei Tan, C., 2013. Flywheel Energy Storage. [Online] 
Available at: https://www.sciencedirect.com/topics/engineering/flywheel-energy-storage
[Accessed 29 November 2024].

NHS, 2021. Health Survey for England, 2021 part 1. [Online] 
Available at: https://digital.nhs.uk/data-and-information/publications/statistical/health-survey-for-england/2021/part-4-trends#:~:text=In%202021%2C%20the%20mean%20weights,For%20more%20information%3A%20Table%207.
[Accessed 26 November 2024].

Sharma, M., 2024. EV Battery Explained: Size, Weight, Power & Capacity. [Online] 
Available at: https://1charging.com/ev-battery-explained-size-weight-power-capacity/#:~:text=If%20it%E2%80%99s%20charged%20at%20a%201C%20rate%2C%20it%E2%80%99s,minutes%2C%20with%20a%20power%20output%20of%20100%20kW.
[Accessed 29 November 2024].

Specs, U., 2024. Ferrari 599 GTB Fiorano Specs. [Online] 
Available at: https://www.ultimatespecs.com/car-specs/Ferrari/1797/Ferrari-599-GTB-Fiorano.html
[Accessed 26 November 2024].

Staff, E., 2022. Michelin Pilot Sport 4 review. [Online] 
Available at: https://www.evo.co.uk/tyre-reviews/204161/michelin-pilot-sport-4-review
[Accessed 26 November 2024].