Formula Sheet

BEng.(Hons.) in Electrical and Electronics Engineering

ACDA4006

| Ohm’s Law |  | Series Resistors RT=R1+R2+…+Rn | Potential Divider V1=VR1R1+R2 |
| --- | --- | --- | --- |
|  |  | Parallel Resistors 1RT=1R1+1R2+…+1Rn | Current Divider I1=IR2R1+R2 |
| Capacitive Reactance XC=12πfC XC=-j1ωC | Inductive Reactance XL=2πfL XL=jωL | Series Capacitors 1CT=1C1+1C2+…+1Cn | Series Inductors LT=L1+L2+…+Ln |
| Angular Frequency ω=2πf | Capacitance C=ChargeVoltage | Parallel Capacitors CT=C1+C2+…+Cn | Parallel Inductors 1LT=1L1+1L2+…+1Ln |
| Frequency f=1T | Voltage across Inductor V=Ldidt |  |  |
| Voltage Equation V=Vmsin⁡(2πft) V=Vmsinωt V=Vmsinωt+ϕ | Current Equation I=Imsin⁡(2πft) I=Imsinωt I=Imsinωt+ϕ | RMS value of Current I=Irms=Im2 Irms=VrmsXC=VrmsXL=VrmsR | RMS value of Voltage V=Vrms=Vm2 Vrms=IrmsXC Vrms= IrmsXL=IrmsR |
| Active /Real Power P=VI Cos∅ | Apparent Power S=P2+Q2 S=VI | Total Power Loss PLoss=PIN-POUT | Efficiency Efficiency=POUTPIN |
| Peak Current Im=VmR Im=VmωL=VmXL Im=VmXC | Instantaneous Current I=VR I=1Lvdt I=CdVdt | Instantaneous Voltage V=IR V=Ldidt V=1CIdt | Instantaneous power P=V×I P=VmImsin2ωt P=-VmIm2sin2ωt P=VmIm2sin2ωt |
| Impedance Z=VI=VmIm=R Z=XL=ωL=2πfL Z=1ωC=XC=12πfC Z=R2+XL2 Z=R2+XC2 Z=R2+(XL-XC)2 | Resistivity ρ=RAL | Voltage Drop across a capacitor VC=IXC | Diode Circuit Equation VR=VS-VD |
|  | Phase Angle Phase angle=tT×360 ϕ=tan-1XLR ϕ=-tan-1XCR ϕ=tan-1XL-XCR ϕ1=tan-1VLVR ϕ1=Sin-1XLZLR ϕ1=Cos-1RZLR | Voltage Drop across an Inductor VL=IXL | From the Current triangle tanϕ=ILIR Cosϕ=IRI Sinϕ=ILI tanϕ=ICIR Cosϕ=IRI Sinϕ=ICI |
| Q Factor Q=XLR Q=XCR Q=fRfU-fL Q=1RLC Q=2πfrLR |  | Voltage V2=VR2+(VL-VC)2 |  |
|  |  | Average Power Pavg=VrmsIrmscosϕ |  |
| Current I=IR2+IL2 I=IR2+IC2 I=IL-IC | Inductor Current IL=VXL ILR=VZLR | Current at Resonance Ir=VRD | Resonant Frequency fr=12πLC |
|  | Capacitor Current IC=VXC | Peak-Peak Value Peak to Peak value= 2×Peak Value | Transformers Turns Ratio VSVP=NSNP |