| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| may 2021 | mai 2021 |

MODULE CODE: ACDB4002 / ACDA4006

Electrical & Electronic Principles

TIME:  2 HOURS

answer all questions

Equipment or materials required – Calculator

Adnoddau neu ddefnyddiau sydd eu hangen - Dim

ANSWER ALL QUESTIONS

- Name the specific circuit element represented by the following symbols:

switch

[1 Mark]

Diode

[1 Mark]

Battery

[1 Mark]

Resistor

[1 Mark]

Inductor

[1 Mark]

Capacitor

[1 Mark]

PNP Transistor

[1 Mark]

NPN Transistor

[1 Mark]

Ground

[1 Mark]

Lamp                                                                                      [1 Mark]

- Sketch the voltage – current relationship of a standard rectifier diode. Label the forward conduction voltage, the reverse breakdown voltage and the reverse leakage current.

[10 Marks]

- For the circuit shown in fig Q3. Calculate the following:

  - Total circuit resistance

R1 = 1k

1/R 2+3 = 1/5600 + 1/220

1/R 2+3 = 1/0.0472402597

1/R 2+3 = 211.6838488

1/R 4+5 = 1/4700 + 1/3300

1/R 4+5 = 1/0.00515

1/R 4+5 = 1938.75

RT = 1000 + 211.6838488 + 1938.75

RT = 3150.433849 ohms

RT 3150.434 ohms 3d.p.

[4 Marks]

  - Total circuit current

15 ÷ 3150.434849 = 4.761248996 mA

= 0.004761248996 A

[2 Marks]

  - Current flowing through R2

[4 Marks]

  - Total power delivered by the battery

[2 Marks]

  - Power dissipated by R4

15 ÷ 4700 = 0.00319 A

P = 15 x 0.00319

P = 0.0479 W

P = 47.9 mW

[4 Marks]

  - R4 is replaced by a wire link. What is the new circuit current?

[4 Marks]

Fig Q3

- An oscilloscope display shows the above voltage waveform.  The oscilloscope settings are shown in the box next to the waveform.

Oscilloscope settings:

50 mV/div

100 µs/div

Fig Q4

- Estimate the period of the waveform

4 div x 100 µs/div = 400 µs

[1 Mark]

- From the estimated period, determine its frequency

f = 1/0.0004

f = 2500Hz

[1 Mark]

- Determine its peak to peak amplitude

4 div x 50mV = 200mV

= 0.2 V

[1 Mark]

- Sketch the diagram shown in fig Q4 and indicate on it any two points between which the PERIOD (T) could be measured

[1 Mark]

- For a series RLC circuit where R=120Ω, L= 470mH, and C=1µF supplied from a 48V 400Hz supply,

  - calculate the voltage across:

    - The resistor

[2 Marks]

    - The capacitor

[2 Marks]

    - The inductor

[2 Marks]

  - Calculate the resonant frequency of the circuit

fres = 1/2π0.47x0.000001

fres = 232.513442

[2 Marks]

  - At the resonant frequency, calculate the circuit current

[2 Marks]

- A 12 V battery has an internal resistance of 3 Ω,

  - Calculate the power dissipated by various load resistances and complete the table below

Load resistance of 1

12V ÷ 4Ω = 3A

P = 12V x 3A

P = 36W

Load resistance of 2

12V ÷ 5Ω = 2.4A

P = 12V x 2.4A

P = 28W

Load resistance of 3

12V ÷ 6Ω = 2A

P = 12V x 2A

P = 24W

Load resistance of 4

12V ÷ 7Ω = 1.71A

P = 12V x 1.71A

P = 20.57W

Load resistance of 5

12V ÷ 8Ω = 1.5A

P = 12V x 1.5A

P = 18W

| Load resistance (Ω) | Load power (W) |
| --- | --- |
| 1 | 36 |
| 2 | 28.8 |
| 3 | 24 |
| 4 | 20.57 |
| 5 | 18 |

[10 Marks]

  - Plot a graph of the load power against load resistance

[6 Marks]

Diwedd/End

# Formulae Sheet

|  |  | RT=R1+R2+…+Rn 1RT=1R1+1R2+…+1Rn 1CT=1C1+1C2+…+1Cn CT=C1+C2+…+Cn LT=L1+L2+…+Ln 1LT=1L1+1L2+…+1Ln | VOut=VinRBRA+RB I=IinRBRA+RB |
| --- | --- | --- | --- |
|  |  |  | XC=12πfC XC=-j1ωC XL=2πfL XL=jωL |
| Z=R+jXL-XC Z=R2+XL-XC2 | ω=2πf XT=XL-XC | φ=tan-1XTR pf=cosφ | fres=12πLC |
| VC=Vs1-e-tτ IC=VRe-tRC VC=V0e-tτ IC=-VRe-tRC τ=RC | τ=LR VL=Ve-RtL VL=-Ve-RtL IL=VR1-e-RtL IL=VRe-RtL | f=12πRC AV=20logVoutVin AP=10logPoutPin | Q=XLR Q=XCR Q=fRfU-fL Q=1RLC |
|  |  | E=12CV2 E=12LI2 |  |
| VB =RB2(RB1 + RB2)× VCC VE=VB-0.7 IC=IE VC=VCC-IC×RC Av=vcvb VE=10%VCC RIN=11RB1+1RB2+1hFE×RE ROUT=RC AV≈-RCRE |  | Vout=-R2R1×Vin Vout=1+R2R1×Vin Vout=R2R1×V2-V1 Vout=-R2R1×V1+V2 Vout=-RR1×V1+RR2×V2+…+RRn×Vn |  |

Detach this page and place in answer book

Student Number ………………………….	Question Number ……………………….