Tutorial Sheet:  Signals in Noise

1.  The ratio of Energy Per Bit (Eb) to the Noise Power Spectral Density (No), or (Eb/No), is the digital equivalent of signal to noise ratio. Explain the Eb/No ratio using a simple rectangular pulse as an example.

2.  Our system works with the following characteristics:

Uni-polar signalling

Data rate = 2 Mbits/s

Received Voltage = 2 μV

Amplifier noise temperature = 2000K

What is the bit error rate?

2.  (Ans)

Eb = V2Tb = (4x10-12)(5x10-7) = 2x10-18

No = kTe = (1.38x10-23) x 2000 = 2.76x10-20

Eb/No = 72.46

(Eb/2No)1/2 =  6.02

Pe = Q(6.02) ≈ 1 x 10-9