University of Wales Trinity Saint David

Applied Computing

ACCD4001

Student Number 1806547

Rizwan Chishti

Lab - Building a Switch and Router Network

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.0.1 | 255.255.255.0 | N/A |
|  | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |
| PC-B | NIC | 192.168.0.3 | 255.255.255.0 | 192.168.0.1 |

Objectives

Part 1: Set Up the Topology and Initialize Devices

Part 2: Configure Devices and Verify Connectivity

Part 3: Display Device Information

Background / Scenario

This is a comprehensive lab to review previously covered IOS commands. In this lab, you will cable the equipment as shown in the topology diagram. You will then configure the devices to match the addressing table. After the configurations have been saved, you will verify your configurations by testing for network connectivity.

After the devices have been configured and network connectivity has been verified, you will use IOS commands to retrieve information from the devices to answer questions about your network equipment.

This lab provides minimal assistance with the actual commands necessary to configure the router. Test your knowledge by trying to configure the devices without referring to the content or previous activities.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). The switches used are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the correct interface identifiers.

Note: Ensure that the routers and switches have been erased and have no startup configurations. Consult with your instructor for the procedure to initialize and reload a router and switch.

Required Resources

1 Router (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

2 PCs (Windows 7 or 8 with terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Note: The Gigabit Ethernet interfaces on Cisco 1941 routers are autosensing and an Ethernet straight-through cable may be used between the router and PC-B. If using another model Cisco router, it may be necessary to use an Ethernet crossover cable.

Set Up Topology and Initialize Devices

Cable the network as shown in the topology.

Attach the devices shown in the topology diagram, and cable, as necessary.

Power on all the devices in the topology.

Initialize and reload the router and switch.

If configuration files were previously saved on the router and switch, initialize and reload these devices back to their basic configurations.

Configure Devices and Verify Connectivity

In Part 2, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords. Refer to the Topology and Addressing Table at the beginning of this lab for device names and address information.

Assign static IP information to the PC interfaces.

Configure the IP address, subnet mask, and default gateway settings on PC-A.

Configure the IP address, subnet mask, and default gateway settings on PC-B.

Ping PC-B from a command prompt window on PC-A.

Why were the pings not successful?

_this_will_not_happen_because_your_router_and_switches_are_not_configured_yet.______________________________________________________________________

Configure the router.

Console into the router and enable privileged EXEC mode.

Enter configuration mode.

Assign a device name to the router.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the clear text passwords.

Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

Configure and activate both interfaces on the router.

Configure an interface description for each interface indicating which device is connected to it.

Save the running configuration to the startup configuration file.

Set the clock on the router.

Note: Use the question mark (?) to help with the correct sequence of parameters needed to execute this command.

Ping PC-B from a command prompt window on PC-A.

Were the pings successful? Why?

____yes the ping was successful this time around because we managed to set up the router and switches to establish the connection correctly. Ip addresses were assigned properly and we managed to config the router properly so we were able to get a successful ping from PC-A to PC-B

________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

Display Device Information

In Part 3, you will use show commands to retrieve information from the router and switch.

Retrieve hardware and software information from the network devices.

Use the show version command to answer the following questions about the router.

What is the name of the IOS image that the router is running?

_ Cisco IOS Software, C1900 Software (C1900-UNIVERSALK9-M), Version 15.1(4)M4, RELEASE SOFTWARE (fc2)

Technical Support: http://www.cisco.com/techsupport

Copyright (c) 1986-2007 by Cisco Systems, Inc.

Compiled Wed 23-Feb-11 14:19 by pt_team___________________________________________________________________________________

How much DRAM memory does the router have?

____ DRAM configuration is 64 bits wide with parity disabled________________________________________________________________________________

____________________________________________________________________________________

How much NVRAM memory does the router have?

_________255 k of non volatile memory ___________________________________________________________________________

How much Flash memory does the router have?

__________________ 249856K bytes of ATA System CompactFlash 0 (Read/Write)__________________________________________________________________

Use the show version command to answer the following questions about the switch.

What is the name of the IOS image that the switch is running?

__ Cisco IOS Software, C2960 Software (C2960-LANBASE-M), Version 12.2(25)FX, RELEASE SOFTWARE (fc1)

Copyright (c) 1986-2005 by Cisco Systems, Inc.

Compiled Wed 12-Oct-05 22:05 by pt_team__________________________________________________________________________________

How much dynamic random access memory (DRAM) does the switch have?

____________________________________________________________________________________

How much nonvolatile random-access memory (NVRAM) does the switch have?

___ 63488K bytes of flash-simulated non-volatile configuration memory._________________________________________________________________________________

What is the model number of the switch?

Model number : WS-C2960-24TT

Display the routing table on the router.

Use the show ip route command on the router to answer the following questions.

What code is used in the routing table to indicate a directly connected network? C_____

How many route entries are coded with a C code in the routing table? __2_______

What interface types are associated to the C coded routes?

_ GigabitEthernet0/0    ______________ GigabitEthernet0/1________________________________________________________________________

Display interface information on the router.

Use the show interface g0/1 to answer the following questions.

What is the operational status of the G0/1 interface?

___ GigabitEthernet0/1 is up, line protocol is up (connected)____________________________________________________________________________________

What is the Media Access Control (MAC) address of the G0/1 interface?

____ address is 0000.0c0b.9202 (bia 0000.0c0b.9202)___________________________________________________________________________________

How is the Internet address displayed in this command?

_ Internet address is 192.168.1.1/24______________________________________________________________________________________

Display a summary list of the interfaces on the router and switch.

There are several commands that can be used to verify an interface configuration. One of the most useful of these is the show ip interface brief command. The command output displays a summary list of the interfaces on the device and provides immediate feedback to the status of each interface.

Enter the show ip interface brief command on the router.

R1# show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

Embedded-Service-Engine0/0 unassigned      YES unset  administratively down down

GigabitEthernet0/0         192.168.0.1     YES manual up                    up

GigabitEthernet0/1         192.168.1.1     YES manual up                    up

Serial0/0/0                unassigned      YES unset  administratively down down

Serial0/0/1                unassigned      YES unset  administratively down down

R1#

Enter the show ip interface brief command on the switch.

Switch# show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Vlan1                  unassigned      YES manual up                    up

FastEthernet0/1        unassigned      YES unset  down                  down

FastEthernet0/2        unassigned      YES unset  down                  down

FastEthernet0/3        unassigned      YES unset  down                  down

FastEthernet0/4        unassigned      YES unset  down                  down

FastEthernet0/5        unassigned      YES unset  up                    up

FastEthernet0/6        unassigned      YES unset  up                    up

FastEthernet0/7        unassigned      YES unset  down                  down

FastEthernet0/8        unassigned      YES unset  down                  down

FastEthernet0/9        unassigned      YES unset  down                  down

FastEthernet0/10       unassigned      YES unset  down                  down

FastEthernet0/11       unassigned      YES unset  down                  down

FastEthernet0/12       unassigned      YES unset  down                  down

FastEthernet0/13       unassigned      YES unset  down                  down

FastEthernet0/14       unassigned      YES unset  down                  down

FastEthernet0/15       unassigned      YES unset  down                  down

FastEthernet0/16       unassigned      YES unset  down                  down

FastEthernet0/17       unassigned      YES unset  down                  down

FastEthernet0/18       unassigned      YES unset  down                  down

FastEthernet0/19       unassigned      YES unset  down                  down

FastEthernet0/20       unassigned      YES unset  down                  down

FastEthernet0/21       unassigned      YES unset  down                  down

FastEthernet0/22       unassigned      YES unset  down                  down

FastEthernet0/23       unassigned      YES unset  down                  down

FastEthernet0/24       unassigned      YES unset  down                  down

GigabitEthernet0/1     unassigned      YES unset  down                  down

GigabitEthernet0/2     unassigned      YES unset  down                  down

Switch#

Reflection

If the G0/1 interface showed administratively down, what interface configuration command would you use to turn the interface up?

_____The no shutdown command __________________________________________________________________________________

What would happen if you had incorrectly configured interface G0/1 on the router with an IP address of 192.168.1.2?

- _The pc would not be able to send the ping as the configuration would not correctly done. The pings will get stuck at G0/1 gateway as it will not have the correct addresse to get through or correct direction to get through.______________________________________________________________________________________

- _______________________________________________________________________________________

Explain the operation of a Router and a Switch.

A router can connect multiple networks together while a switch can connect multiple computers within a network. _______________________________________________________________________________________

_______________________________________________________________________________________