Engineering Design & Materials

The Application of an Engineering Design Methodology – Dry Sump Design

By

Charles Sangster/ 1902452

Contents

Design specifications	4

QFC (Quality Function Deployment)	5

FMEA (Failure modes and effects analysis)	6

Wet and dry sump	6

Advantages	6

Disadvantages	6

Wet sump	6

Advantages	6

Disadvantages	6

Dry sump	6

anti flood system	7

Research	7

Materials	7

Aluminium	7

Low Carbon steel	7

existing products	8

construction	9

Cold chamber Die casting	9

press forming	9

CNC machined	9

Concept ideas	10

10

Final design	13

Description	13

Solid model and Technical drawings	15

References	19

Bibliography	19

List of figures

Figure 1:QFC	5

Figure 4: Opel/Vauxhall (https://www.turbosport.net/product/opelvauxhall-dry-sump-pan-c20xe-c20let-copy/	8

Figure 3:Ford Duratec (https://www.turbosport.net/product/ford-duratec-yb-dry-sump-pan/)	8

Figure 2: Ford Pinto/Cosworth (https://www.turbosport.net/product/ford-pintocosworth-yb-dry-sump-pan-copy/)	8

Figure 5:Concept one	10

Figure 6: Concept two	10

Figure 7:Concept three	10

Figure 8: Comparison of concept designees	12

Figure 9: Sump concept design	12

Figure 10: Final sump design	13

Figure 11: Assembled product	13

Figure 12:Final anti flood system	14

Figure 13:Final assembly schematic	15

Figure 14:Sump schematic	16

Figure 15:Anti flooding schematic	17

List of tables

Table 1:Customer Requirements	4

Table 2:Final design specification	4

Table 3:FMEA	6

Table 4:Analysis of existing products	8

Table 5:Concept ideas analysis	11

Table 6: Analysis of Final design	14

# Design specifications

An automotive manufacturer has requested a new dry sump and oil pick up system to be designed, this system must be able to fit on to a Ford Zetec engine. The dry sump system must replace the wet sump system that comes fitted to the standard production vehicle, and must be able to with stand the strain of being used in a motorsport application.

With further research into customer requirements the following was established

Table 1:Customer Requirements

| Requirements of the customer | Importance of requirement out of 10 |
| --- | --- |
| Light weight | 8/10 |
| Easy installation of product | 5/10 |
| Prevention of flooding when engine is rolled | 9/10 |
| Low cost | 7/10 |
| Fast and easy access for maintenance | 7/10 |

Harrison, A. (2020, p.2)

Using these points, a design specification was made to allow for a better end product.

Table 2:Final design specification

# QFC (Quality Function Deployment)

The QFC helps put the customer’s requirements for the product into characteristics that the design must have.

Morgan, R. Design Toolkit Quality Function Deployment (QFD).

Figure 1:QFC

Callister.W. D, JR. (2007) ALUMINIUM ALLOYS - YIELD STRENGTH AND TENSILE STRENGTH.

# FMEA (Failure modes and effects analysis)

Table 3:FMEA

Morgan, R. Failure Modes & Effects Analysis

# Wet and dry sump

### Advantages

- ran cheaply with little parts and maintenance

- Light weight

### Disadvantages

- oil cannot be cooled efficiently; cooling is reliant on the sump’s material radiating the heat.

- oil is affected by inclinations of the vehicle; this can course the oil pick up to become unsubmerged and start running dry.

- Engine must be fitted higher up in the vehicle

## Wet sump

### Advantages

- Better cooling of oil

- Engine can be fitted lower in the vehicle

- Oil is not affected by inclination.

### Disadvantages

- Expensive to run and maintain

- Adds wait to the vehicle

## Dry sump

There are two variations of sump, wet sump and dry sump. where a wet sump stores oil and acts as an oil drain, the dry sump only acts as an oil drain, storing oil in a separate tank located within the vehicle. Both sumps are located at the bottom of the crank housing. The wet sump system uses an oil pick up line which is ran by a pressured oil pump powered by the engine to distribute oil threw the engine, the dry sump uses the same oil distribution system but also uses a secondary pump to move oil between the collection pan and the oil tank.

Newton, Steeds and Garrett. (1989, p.415), Heisler (1999, p.470, 498).

anti flood system

Anti flood systems are fitted to sumps to prevent the engine from being flooded with oil when rolled, if this system was not in place the engine would become flooded coursing damage to the engine.

# Research

## Materials

Two materials have been researched for this product; the following materials are widely used with in the automotive industry.

### Aluminium

Aluminium is used commonly with higher end sumps and sump pumps.

#### Advantages

- high strength to wait ratio

- resistance to corrosion

- high thermal and electrical conductivity

- very durable

- accepts surface finishing

- recyclable

#### disadvantages

- expensive

- hard to machine and weld

THE PROS AND CONS OF ALUMINUM IN ENGINEERING (July 23, 2018).

### Low Carbon steel

Carbon steel is used within construction and is used in the manufacture of vehicle body structure.

#### Advantages

- strong

- cheap

#### disadvantages

- difficult to work with

- Easley deformed

- Affected by corrosion

Pros and Cons of Carbon Steel: What You Should Know (October 10, 2018)

Out of the two materials aluminium will be used for the manufacture of the sump, this is due to its strength, resistance to corrosion and its ability to be recycled. The cost of the aluminium will have an effect on the end price of the product.

## existing products

Figure 4: Opel/Vauxhall (https://www.turbosport.net/product/opelvauxhall-dry-sump-pan-c20xe-c20let-copy/

Figure 3:Ford Duratec (https://www.turbosport.net/product/ford-duratec-yb-dry-sump-pan/)

Figure 2: Ford Pinto/Cosworth (https://www.turbosport.net/product/ford-pintocosworth-yb-dry-sump-pan-copy/)

Looking at the following dry sump designs and prices an idea can be formed for what the final product must achieve.

Table 4:Analysis of existing products

## construction

### Cold chamber Die casting

This process forces molten metal in to a die cast, producing the product.

#### Advantages

- High levels of efficiency in mass production

- Good quality of surface finishes

- high levels of dimensional accuracy achieved

- Reusable die

#### Disadvantages

- Metals with high melting points cannot be used

- Large products cannot be casted

- High price of the die cast

Dr. Dmitri Kopeliovich, Die casting,

### press forming

press forming is placing a chosen material under a large amount of forcing till it takes the shape of a die (the product).

#### Advantages

- Suitable for bulk production

- High quality products can be made quickly

- Produces large quantities cheaply

#### Disadvantages

- Cost of manufacturing the die

- Longer lead times needed

- Inflexibility in product design due to die

When to Stamp Metal and When to Fabricate (October 30, 2014)

### CNC machined

CNC machining is when a chunk of material is machined using computer guided tools to manufacture the product.

#### Advantages

- The ability to reproduce product after product whilst being able to maintain a level of quality

- Extended running periods without human input

- Machine complex designs with ease

#### Disadvantages

- Expensive to run

- Cost of machinery

- Training of employs

Ricardo & Barbosa (January 24, 2019) The Pros and Cons of CNC Machining

# Concept ideas

Figure 5:Concept one

Figure 6: Concept two

Concept two

Concept one

Figure 7:Concept three

Concept three

Table 5:Concept ideas analysis

The following table compares and ranks each concept design on how well it performs within a specified category, this category relates back to the design specification. The ranking is shown by a number, 2 being the highest and 8 being the lowest. Once all scores have been added up, an overall total is produced; this total is than compared to the other designs. out of all the concepts, concept three has the lowest total score making it a more suited design for the final product. this means that concept three will be expanded upon till the final design is reached.

Figure 8: Comparison of concept designees

Morgan, R. Design Toolkit Systematic Idea Isolation Decision/Pugh Matrix

Sump design

Figure 9: Sump concept design

This is the first concept for the sump. Measurements were taken of the existing wet sump system, and material has been removed to reduce wait and cost of the final product. The design will be made from cold chamber die casted aluminium.

# Final design

## Description

Figure 10: Final sump design

Figure 11: Assembled product

C

Assembled product

B

Sump design

Figure 12:Final anti flood system

Anti flood system

A

Table 6: Analysis of Final design

## Solid model and Technical drawings

Figure 13:Final assembly schematic

Figure 14:Sump schematic

Figure 15:Anti flooding schematic

For further information on the final design use the following link:

https://uowtsd-my.sharepoint.com/:f:/g/personal/1902452_student_uwtsd_ac_uk/Emy2dlwWqw5JksQdasEF_qwBrpfsbS6I5MigM1I6PXPzrw?e=W3e4gD

# References

Newton, K, Steeds, W & Garrett, TK. (1989). The Motor Vehicle (Eleventh Edition). Sevenoaks: Butterworths.

Heisler, H. (1999). Vehicle and Engine Technology. (2nd edition). London: Hodder Headline Group.

July 23, 2018) THE PROS AND CONS OF ALUMINUM IN ENGINEERING, Available at: https://www.clintonaluminum.com/the-pros-and-cons-of-aluminum-in-engineering/(Accessed: 23rd February 2020).

(October 10, 2018) Pros and Cons of Carbon Steel: What You Should Know, Available at: https://monroeengineering.com/blog/pros-and-cons-of-carbon-steel-what-you-should-know/ (Accessed: 23rd February 2020).

Dr. Dmitri Kopeliovich () Die casting, Available at: https://www.substech.com/dokuwiki/doku.php?id=die_casting (Accessed: 23rd February 2020).

Ricardo & Barbosa (January 24, 2019) The Pros and Cons of CNC Machining, Available at: https://www.ricardo-barbosa.com/the-pros-and-cons-of-cnc-machining/(Accessed: 23rd February 2020).

Callister.W. D, JR. (2007) ALUMINIUM ALLOYS - YIELD STRENGTH AND TENSILE STRENGTH (Materials Science and Engineering: An Introduction. 7th edition), Available at: https://amesweb.info/Materials/Aluminum-Yield-Tensile-Strength.aspx(Accessed: 23rd February 2020).

(October 30, 2014) When to Stamp Metal and When to Fabricate – Selecting the optimum method for production, Available at: https://www.swiftmetal.com.au/latest-news/when-to-stamp-metal-and-when-to-fabricate/ (Accessed: 23rd February 2020).

Morgan, R. Design Toolkit Quality Function Deployment (QFD) (Accessed: 23rd February 2020)

Morgan, R. Design Toolkit Systematic Idea Isolation Decision/Pugh Matrix (Accessed: 23rd February 2020)

Harrison, A. (2020). The Application of an Engineering Design Methodology – Dry Sump Design.

Morgan, R. Failure Modes & Effects Analysis (Accessed: 28rd February 2020). School of Logistics & Manufacture Engineering

# Bibliography

Hillier, V.A.W. & Pittuck, F. (1981). Fundamentals of Motor Vehicle Technology (3rd edition). Tiptree: Wm Brendon & Son Ltd.

Morgan, R Design Toolkit Systematic Idea Isolation Decision/Pugh Matrix (Accessed: 23rd February 2020)

Morgan, R. Design Specification & The Kano Model of Quality (Accessed: 23rd February 2020)

Morgan, R. Design Process Fundamentals (Accessed: 23rd February 2020)