Lab - Building a Switch and Router Network

Topology

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.0.1 | 255.255.255.0 | N/A |
|  | G0/1 | 192.168.1.1 | 255.255.255.0 | N/A |
| PC-A | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 |
| PC-B | NIC | 192.168.0.3 | 255.255.255.0 | 192.168.0.1 |

Objectives

Part 1: Set Up the Topology and Initialize Devices

Part 2: Configure Devices and Verify Connectivity

Part 3: Display Device Information

Background / Scenario

This is a comprehensive lab to review previously covered IOS commands. In this lab, you will cable the equipment as shown in the topology diagram. You will then configure the devices to match the addressing table. After the configurations have been saved, you will verify your configurations by testing for network connectivity.

After the devices have been configured and network connectivity has been verified, you will use IOS commands to retrieve information from the devices to answer questions about your network equipment.

This lab provides minimal assistance with the actual commands necessary to configure the router. Test your knowledge by trying to configure the devices without referring to the content or previous activities.

Note: The routers used with CCNA hands-on labs are Cisco 1941 Integrated Services Routers (ISRs) with Cisco IOS Release 15.2(4)M3 (universalk9 image). The switches used are Cisco Catalyst 2960s with Cisco IOS Release 15.0(2) (lanbasek9 image). Other routers, switches, and Cisco IOS versions can be used. Depending on the model and Cisco IOS version, the commands available and output produced might vary from what is shown in the labs. Refer to the Router Interface Summary Table at the end of this lab for the correct interface identifiers.

Note: Ensure that the routers and switches have been erased and have no startup configurations. Consult with your instructor for the procedure to initialize and reload a router and switch.

Required Resources

1 Router (Cisco 1941 with Cisco IOS Release 15.2(4)M3 universal image or comparable)

1 Switch (Cisco 2960 with Cisco IOS Release 15.0(2) lanbasek9 image or comparable)

2 PCs (Windows 7 or 8 with terminal emulation program, such as Tera Term)

Console cables to configure the Cisco IOS devices via the console ports

Ethernet cables as shown in the topology

Note: The Gigabit Ethernet interfaces on Cisco 1941 routers are autosensing and an Ethernet straight-through cable may be used between the router and PC-B. If using another model Cisco router, it may be necessary to use an Ethernet crossover cable.

Set Up Topology and Initialize Devices

Cable the network as shown in the topology.

Attach the devices shown in the topology diagram, and cable, as necessary.

Power on all the devices in the topology.

Initialize and reload the router and switch.

If configuration files were previously saved on the router and switch, initialize and reload these devices back to their basic configurations.

Configure Devices and Verify Connectivity

In Part 2, you will set up the network topology and configure basic settings, such as the interface IP addresses, device access, and passwords. Refer to the Topology and Addressing Table at the beginning of this lab for device names and address information.

Assign static IP information to the PC interfaces.

Configure the IP address, subnet mask, and default gateway settings on PC-A.

Configure the IP address, subnet mask, and default gateway settings on PC-B.

Ping PC-B from a command prompt window on PC-A.

Why were the pings not successful?

The pings were not successful as_the_router,the switch_and_default_gateways_have_not_been_configured, no network traffic is_passing_through them.________________________________________________________________________

Configure the router.

Console into the router and enable privileged EXEC mode.

Enter configuration mode.

Assign a device name to the router.

Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names.

Assign class as the privileged EXEC encrypted password.

Assign cisco as the console password and enable login.

Assign cisco as the VTY password and enable login.

Encrypt the clear text passwords.

Create a banner that warns anyone accessing the device that unauthorized access is prohibited.

Configure and activate both interfaces on the router.

Configure an interface description for each interface indicating which device is connected to it.

Save the running configuration to the startup configuration file.

Set the clock on the router.

Note: Use the question mark (?) to help with the correct sequence of parameters needed to execute this command.

Ping PC-B from a command prompt window on PC-A.

Were the pings successful? Why?

The pings were successful as the router, switch and the default gateways are now set up properly. Enabling the traffic to pass_through_them.

Display Device Information

In Part 3, you will use show commands to retrieve information from the router and switch.

Retrieve hardware and software information from the network devices.

Use the show version command to answer the following questions about the router.

What is the name of the IOS image that the router is running?

_ "flash0:c1900-universalk9-mz.SPA.151-1.M4.bin" ___________________________________________________________________________________

How much DRAM memory does the router have?

_ 491520K / 32768K bytes of memory. Or 524,288k_bytes=512MB

How much NVRAM memory does the router have?

_there_is_255K_bytes_of_non-volatile_(NVRAM)_

How much Flash memory does the router have?

_ 249856K bytes of flash memory

Use the show version command to answer the following questions about the switch.

What is the name of the IOS image that the switch is running?

___ Cisco IOS Software, C2960 Software (C2960-LANBASE-M) _________________________________________________________________________________

How much dynamic random access memory (DRAM) does the switch have?

___ 21039K bytes of memory. _________________________________________________________________________________

How much nonvolatile random-access memory (NVRAM) does the switch have?

63488K bytes of flash-simulated non-volatile configuration memory.

What is the model number of the switch?

_The_model number = WS-C2960-24TT __________________________________________________________________________________

Display the routing table on the router.

Use the show ip route command on the router to answer the following questions.

What code is used in the routing table to indicate a directly connected network? __C_

How many route entries are coded with a C code in the routing table? ____2_____

What interface types are associated to the C coded routes?

___GigabitEthernet_0/0_&gigabitEthernet_0/1_________________________________________________________________________________

Display interface information on the router.

Use the show interface g0/1 to answer the following questions.

What is the operational status of the G0/1 interface?

_ GigabitEthernet0/1 is up, line protocol is up (connected) ______________________________________________________________________________________

What is the Media Access Control (MAC) address of the G0/1 interface?

_MAC_address_is_ address is 0060.3e2e.c102 ___________________________________________________________________________________

How is the Internet address displayed in this command?

Internet_address_is_displayed_like_this_= Internet address is 192.168.1.1/24 _________________________________________________________________________________

Display a summary list of the interfaces on the router and switch.

There are several commands that can be used to verify an interface configuration. One of the most useful of these is the show ip interface brief command. The command output displays a summary list of the interfaces on the device and provides immediate feedback to the status of each interface.

Enter the show ip interface brief command on the router.

R1# show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

Embedded-Service-Engine0/0 unassigned      YES unset  administratively down down

GigabitEthernet0/0         192.168.0.1     YES manual up                    up

GigabitEthernet0/1         192.168.1.1     YES manual up                    up

Serial0/0/0                unassigned      YES unset  administratively down down

Serial0/0/1                unassigned      YES unset  administratively down down

R1#

Enter the show ip interface brief command on the switch.

Switch# show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

Vlan1                  unassigned      YES manual up                    up

FastEthernet0/1        unassigned      YES unset  down                  down

FastEthernet0/2        unassigned      YES unset  down                  down

FastEthernet0/3        unassigned      YES unset  down                  down

FastEthernet0/4        unassigned      YES unset  down                  down

FastEthernet0/5        unassigned      YES unset  up                    up

FastEthernet0/6        unassigned      YES unset  up                    up

FastEthernet0/7        unassigned      YES unset  down                  down

FastEthernet0/8        unassigned      YES unset  down                  down

FastEthernet0/9        unassigned      YES unset  down                  down

FastEthernet0/10       unassigned      YES unset  down                  down

FastEthernet0/11       unassigned      YES unset  down                  down

FastEthernet0/12       unassigned      YES unset  down                  down

FastEthernet0/13       unassigned      YES unset  down                  down

FastEthernet0/14       unassigned      YES unset  down                  down

FastEthernet0/15       unassigned      YES unset  down                  down

FastEthernet0/16       unassigned      YES unset  down                  down

FastEthernet0/17       unassigned      YES unset  down                  down

FastEthernet0/18       unassigned      YES unset  down                  down

FastEthernet0/19       unassigned      YES unset  down                  down

FastEthernet0/20       unassigned      YES unset  down                  down

FastEthernet0/21       unassigned      YES unset  down                  down

FastEthernet0/22       unassigned      YES unset  down                  down

FastEthernet0/23       unassigned      YES unset  down                  down

FastEthernet0/24       unassigned      YES unset  down                  down

GigabitEthernet0/1     unassigned      YES unset  down                  down

GigabitEthernet0/2     unassigned      YES unset  down                  down

Switch#

Reflection

If the G0/1 interface showed administratively down, what interface configuration command would you use to turn the interface up?

_____The_”No_shutdown”_command_______________________________________________________________________________

What would happen if you had incorrectly configured interface G0/1 on the router with an IP address of 192.168.1.2?

- _The_PCs_would_not_be_able_to_send_pings(messages)_as_the_configuration_would_not_be_correct.____The_pings_would_get_stuck_at_G_0/1_gateway_as_it_would_not_have_a _path_to_get_to_its_ destination.

- _______________________________________________________________________________________

Explain the operation of a Router and a Switch.

_A_router_connects_multiple_networks_toghether  A_switch_connect_computers_within_a_network_together

What is a Default Gateway – A gateway from which information can be sent to another computer on another network, on a home network it is usually the router which is the default gateway. All packages go through the default gateway before exiting to the internet.