Marking Key

ANSWER ALL THE QUESTIONS

Question 1 [25 Marks]

- For the circuit shown in fig1, Calculate the following

- Total resistance in the circuit			[4 Marks]

- Total current in the circuit.			[2 Marks]

- Current across each resistor			[8 Marks]

- The voltage across R7 resistor			[2 Marks]

- Power delivered by the battery			[2 Marks]

Fig: 1

Answers:

- Total Resistance in the circuit

R1 and R2 are in parallel

Req1=11R1+1R2=116+13=10.5=2Ω						[1 Mark]

R4 and R5 are in parallel

Req2=11R4+1R5=115+110=10.3=3.33Ω						[1 Mark]

Req2 and R3 are in series

Req3=Req2+R3=4+3.33=7.33Ω 					[1 Mark]

R6+R7 in parallel with Req3

Req4=(R6+R7)×Req3R6+R7+Req3=2+4×7.332+4+7.33=3.3Ω

Req4 and Req1 are in series

Rt=Req4+Req1=3.3+2=5.3Ω					[1 Mark]

- Total current in the circuit

I=VRt=255.3									[1 Mark]

I=4.71A									[1 Mark]

- Current across each resistor

Current across R1

I1=IR2R1+R2= 4.7×36+3=1.57A							[1 Mark]

Current across R2

I2=IR1R1+R2= 4.7×66+3=3.14A							[1 Mark]

Current across R3

I3=I(R6+R7)R6+R7+Req3= 4.7×(2+4)2+4+7.33=2.1A						[1 Mark]

Current across R4

I4=I3R5R4+R5= 2.1×105+10=1.4A 							[1 Mark]

Current across R5

I5=I3R4R4+R5= 2.1×55+10=0.7A 							[2 /Marks]

Current across R6 and R7

I6=IReq3R6+R7+Req3= 4.7×7.332+4+7.33=2.55A						[2 Marks]

- Calculate the Voltage across R7 Resistor

V7=I6R7=2.44×4=10.2V 						[2 Marks]

- Power delivered by the battery

P=VI=25×4.71=117.75W			[2 Marks]

- A solar yard light has a solar collector that can provide 1.0W of power for charging the 3.0V batteries. What is the maximum charging current the solar collector could supply to the fully discharged 3.0V batteries?							[2 Marks]

Answers:

I=PV=13 									[2 Marks]

I=0.33A 									[1 Mark]

- Calculate the current flowing when a charge of 60C flows past a point over a period of 2 minutes.										[2 Marks]

Answers:

I=Qt 											[1 Mark]

I=60120=0.5A									[1 Mark]

- What length of German silver wire, diameter 0.050 cm, is needed to make a 28 resistor, if the resistivity of German silver is 2.2×10-7  m?				[3 Marks]

Answers:

A=π r2

r=2.50×10-4m

A=π×( 2.50×10-4)2=1.96×10-7m2					[1 Mark]

L=RAρ											[1 Mark]

L=28×1.96×10-72.2×10-7=24.95m     							[1 Mark]

Question 2 [25 Marks]

- Calculate the current flowing when a charge of 60C flows past a point over a period of 2 minutes.										[2 Marks]

Answers:

I=Qt 											[1 Mark]

I=60120=0.5A									[1 Mark]

- Determine the current across each resistor in the circuit shown in fig.2, using mesh analysis (Kirchhoff’s Voltage Law)	[13 Marks]

Fig.2

Answers:

Apply KVL in ABEFA

-5I1-3I1-I2-5-2I1+20=0 						[2 Marks]

-10I1+3I2=-15 …………(1)						[2 Marks]

Apply KVL in BCDEB

-6I2-7I2-2I2+5+3I1-I2=0 						[2 Marks]

-3I1-18I2=-5 …………(2)							[2 Marks]

Simplifying Equation (1) and (2)

I1=1.67A									[2 Marks]

Substitute, I1 in equation (1)

-10 1.67+3I2=-15

-16.7+3I2=-15

3I2=-15+16.7

I2=1.73=0.567A									[2 Marks]

Current across branch BE

I1-I2=1.67-0.567=1.1A							[1 Mark]

Current Across R1and R3=I1=1.67A

Current Across R2=I1-I2=1.1A

Current Across R4 R5 and R6=I2=0.567A

- Differentiate Step- up Transformer and Step-Down Transformer (minimum of two differences)									[4 Marks]

Answers:

| Sl. No | Step-up Transformer | Step-down Transformer |
| --- | --- | --- |
| 1 | Input voltage is lower than the output voltage | Input voltage is higher than the output voltage |
| 2 | Number of Secondary winding is higher than the primary winding | Number of Secondary winding is less than the primary winding |
| 3 |  |  |

[Each difference two marks, Maximum of 4 marks]

- Explain the following terms:

- Resistance									[2 Marks]

- Reactance									[2 Marks]

- Impedance									[2 Marks]

Answers:

- Resistance

Opposition to flow of current. Unit is Ohm - DC circuit property		[2 Marks]

- Reactance

Opposition to change of flow of current. Unit is Ohm - AC circuit property	[2 Marks]

- Impedance

Opposition to change of flow of current. Unit is Ohm- AC circuit property 	[2 Marks]

Question 3 [25 Marks]

- The alternating current is given by i=40 sin314t A. Determine the following

- Peak Value of current			[1 Mark]

- RMS Value of Current			[2 Marks]

- Frequency			[2 Marks]

- Time period			[1 Mark]

Answers:

- Peak Value of current

From the given equation, Im=40A						[1 Mark]

- RMS Value of Current

IRMS=Im2=402									[1 Mark]

IRMS=28.28A									[1 Mark]

- Frequency

From the equation, ω=314

We know that, ω=2πf

314=2πf 									[1 Mark]

f=3142π=50Hz									[1 Mark]

- Time

T=1f=150=0.02sec								[1 Mark]

- An oscilloscope display shows the below voltage waveform. The oscilloscope settings are shown in the box next to the waveform.

Oscilloscope settings:

2V/div

10ms/div

Fig:3

- Estimate the period of the waveform.					[1 mark]

- From the estimated period, determine its frequency.			[2 marks]

- Determine its peak-peak amplitude					[2 marks]

- Determine the r.m.s. value of the waveform.				[2 marks]

Answers:.

- Estimate the period of the waveform.

Period=4 div ×10 ms=40 ms				[1 mark]

- From the estimated period, determine its frequency.

f=1t

f= 140 ms=25 Hz						[2 marks]

- Determine its peak-peak amplitude

Vpk-pk=4 div ×2 V=8 V				[2 marks]

- Determine the r.m.s. value of the waveform.

VRMS=Vpk2=42=2.828 V					[2 marks]

- An RLC circuit is shown in figure 4 below. Calculate the following

- The total circuit impedance 						[4 Marks]

- Total current flowing in the circuit.					[2 Marks]

- Determine the voltage drop across each component in the circuits.	[6 Marks]

Fig: 4

Answers:

- Total impedance

XL=2πfL=2×π×50×1 mH=0.3142 Ω					[1 Mark] XC=12πfC=12×π×50×470 μF=6.773 Ω  						[1 Mark]

Z=R2+XL-XC2=1002+0.3142-6.7732=100.208 Ω		[2 Marks]

- Circuit Current

I=VZ=120100.208=1.198 A								[2 Marks]

- Determine the voltage drop across each component in the circuits.

VR=I×R=1.198×100=119.8 V					[2 Marks]

VL=I×XL=1.198×0.3142=0.376 V				[2 Marks]

VC=I×XC=1.198×6.773=8.114 V				[2 Marks]

Question 4 [25 Marks]

- State Maximum power transfer theorem.					[2 Marks]

Answers:

Maximum power is transferred from a source to a load when the load resistance​ is equal to the internal resistance of the source network as seen from the load terminals.	[3 Marks]

- A transformer has 300 turns in its secondary coil and is used to step up a 12v AC voltage to 200V.

  - How many turns would there be in the primary coil.			[3 Marks]

  - Under full load conditions, the current demand in the secondary circuit is 2.5A.  Determine the Power output rating of this transformer.		[2 Marks]

  - Determine the maximum load resistance that this transformer could supply.											[2 Marks]

  - we assume that the transformer is 97.5% efficient, estimate the current flow in the primary circuit for the conditions present in part b).		[2 Marks]

Answers:

- NSVS=NPVP									[1 Mark]

NS=205×110=440 turns 							[1 Mark]

Therefore, turns ratio = 20:440 = 1:22					[1 Mark]

- Power Rating = VS× IS							[1 Mark]

Power Rating= 110 x 3 = 330W						[1 Mark]

- RL max= VSIS								[1 Mark]

RL max = 1103 = 37Ω							[1 Mark]

- PS=PP×0.93

PP=PS0.93  = 3300.93 = 355W							[1 Mark]

IP full load=PPVP

IP full load=3555=71A 						[1 Mark]

- Describe the functions of the following components used in DC machines

- Field System								[2 Marks]

- Armature									[2 Marks]

Answers:

  - Field System

It consists of number of poles bolted inside the circular frame. It provide uniform magnetic field.										[1 Mark]

The mmf (magnetomotive force) produced by the field windings provides a magnetic flux that passes through the pole pieces, the air gap, the armature and the frame.														[2 Marks]

  - Armature

The armature consists of coils of wire wound around a metal core. When the armature rotates within the magnetic field created by the stationary field magnets, an electromagnetic force (EMF) is induced in the armature coils due to electromagnetic induction. 													[3 Marks]

- In the transient circuit, a Capacitor of Value 1μF and a resistor of 5.45MΩ are connected in series across a 220V power supply through a switch. Calculate the time by which the capacitor will be charged to 60 percentage of the supply voltage.													[10 Marks]

Answers:

When the Capacitor is charging

VC=V(1-e-tτ)									[1 mark]

VC=V(1-e-tRC)

VC=V(1-e-t5.45×106×1×10-9) 							[1 mark]

As the capacitor will be discharging to 60% of the Voltage

220100×60=220(1-e-t5.45) 							[1 mark]

132=220(1-e-t5.45)

132220=(1-e-t5.45) 								[1 mark]

0.6=(1-e-t5.45)

e-t5.45=1-0.6

e-t5.45=0.4 									[1 mark]

Taking Log on both the side

-t5.45=ln0.4 									[1 mark]

-t5.45=-0.916									[2 marks]

t=0.398×5.45

t=4.99 Sec 									[2 marks]

The Time taken by the capacitor to charge to 60% of the supply voltage is 2.17Sec.