### Addressing Table

| Device | Interface | IPv6 Address/Prefix | Default Gateway |
| --- | --- | --- | --- |
| R1 | G0/0 | 2001:db8:1:1::1/64 | N/A |
| R1 | G0/0 | fe80::1 | N/A |
| R1 | G0/1 | 2001:db8:1:2::1/64 | N/A |
| R1 | G0/1 | fe80::1 | N/A |
| R1 | S0/0/0 | 2001:db8:1:a001::2/64 | N/A |
| R1 | S0/0/0 | fe80::1 | N/A |
| Sales | NIC | 2001:db8:1:1::2/64 | fe80::1 |
| Billing | NIC | 2001:db8:1:1::3/64 | fe80::1 |
| Accounting | NIC | 2001:db8:1:1::4/64 | fe80::1 |
| Design | NIC | 2001:db8:1:2::2/64 | fe80::1 |
| Engineering | NIC | 2001:db8:1:2::3/64 | fe80::1 |
| CAD | NIC | 2001:db8:1:2::4/64 | fe80::1 |
| ISP | S0/0/0 | 2001:db8:1:a001::1 | fe80::1 |

### Objectives

Part 1: Configure IPv6 Addressing on the Router

Part 2: Configure IPv6 Addressing on Servers

Part 3: Configure IPv6 Addressing on Clients

Part 4: Test and Verify Network Connectivity

### Background

In this activity, you will practice configuring IPv6 addresses on a router, servers, and clients. You will also practice verifying your IPv6 addressing implementation.

## Part 1:  Configure IPv6 Addressing on the Router

### Step 1:  Enable the router to forward IPv6 packets.

- Click R1 and then the CLI tab. Press Enter.

Open a configuration window

- Enter privileged EXEC mode.

- Enter the ipv6 unicast-routing global configuration command. This command must be entered to enable the router to forward IPv6 packets.

R1(config)# ipv6 unicast-routing

### Step 2:  Configure IPv6 addressing on GigabitEthernet0/0.

- Enter the commands necessary to move to interface configuration mode for GigabitEthernet0/0.

- Configure the IPv6 address with the following command:

R1(config-if)# ipv6 address 2001:db8:1:1::1/64

- Configure the link-local IPv6 address with the following command:

R1(config-if)# ipv6 address fe80::1 link-local

- Activate the interface.

R1(config-if)# no shutdown

### Step 3:  Configure IPv6 addressing on GigabitEthernet0/1.

- Enter the commands necessary to move to interface configuration mode for GigabitEthernet0/1.

- Refer to the Addressing Table for the correct IPv6 address.

- Configure the IPv6 address, the link-local address and activate the interface.

### Step 4:  Configure IPv6 addressing on Serial0/0/0.

- Enter the commands necessary to move to interface configuration mode for Serial0/0/0.

- Refer to the Addressing Table for the correct IPv6 address.

- Configure the IPv6 address, the link-local address and activate the interface.

### Step 5:  Verify IPv6 addressing on R1.

It is good practice to verify addressing when it is complete by comparing configured values with the values in the addressing table.

- Exit configuration mode on R1.

- Verify the addressing configured by issuing the following command:

R1# show ipv6 interface brief

- If any addresses are incorrect, repeat the steps above as necessary to make any corrections.

Note: To make a change in addressing with IPv6, you must remove the incorrect address or else both the correct address and incorrect address will remain configured on the interface.

Example:

R1(config-if)# no ipv6 address 2001:db8:1:5::1/64

- Save the router configuration to NVRAM.

Close a configuration window

## Part 2:  Configure IPv6 Addressing on the Servers

### Step 1:  Configure IPv6 addressing on the Accounting Server.

- Click Accounting and click the Desktop tab > IP Configuration.

- Set the IPv6 Address to 2001:db8:1:1::4 with a prefix of /64.

- Set the IPv6 Gateway to the link-local address, fe80::1.

### Step 2:  Configure IPv6 addressing on the CAD Server.

Configure the CAD server with addresses as was done in Step 1. Refer to the Addressing Table for the addresses to use.

## Part 3:  Configure IPv6 Addressing on the Clients

### Step 1:  Configure IPv6 addressing on the Sales and Billing Clients.

- Click Billing and then select the Desktop tab followed by IP Configuration.

- Set the IPv6 Address to 2001:db8:1:1::3 with a prefix of /64.

- Set the IPv6 Gateway to the link-local address, fe80::1.

- Repeat Steps 1a through 1c for Sales. Refer to the Addressing Table for the IPv6 address.

### Step 2:  Configure IPv6 Addressing on the Engineering and Design Clients.

- Click Engineering and then select the Desktop tab followed by IP Configuration.

- Set the IPv6 Address to 2001:db8:1:2::3 with a prefix of /64.

- Set the IPv6 Gateway to the link-local address, fe80::1.

- Repeat Steps 2a through 2c for Design. Refer to the Addressing Table for the IPv6 address.

## Part 4:  Test and Verify Network Connectivity

### Step 1:  Open the server web pages from the clients.

- Click Sales and click the Desktop tab. Close the IP Configuration window, if necessary.

- Click Web Browser. Enter 2001:db8:1:1::4 in the URL box and click Go. The Accounting website should appear.

- Enter 2001:db8:1:2::4 in the URL box and click Go. The CAD website should appear.

- Repeat steps 1a through 1c for the rest of the clients.

### Step 2:  Ping the ISP.

- Click on any client.

- Click the Desktop tab > Command Prompt.

- Test connectivity to the ISP by entering the following command:

PC> ping 2001:db8:1:a001::1

- Repeat the ping command with other clients until full connectivity is verified.