James Cawte (2313853)

Catapult experiment

Taguchi Experimentation

Contents

Table of Tables	2

Table of Figures	2

Section 1 – Information	3

1.1	Background information	3

1.2 Example Array	3

Section 2 – Experiment	4

2.1 – Objective	4

2.2 – Method & Apparatus	4

2.2.1 – Apparatus	4

2.2.2 – Method	4

2.2.3 – Variables	4

2.3 – Experimental Data	6

2.3.1 – Populated Array	6

2.3.2 – Experiment Results	6

2.3.3 – Graphs	7

2.3.4 – Results 2	10

2.3.5 – Mid-point Calculation	10

2.3.6 – Mid-point Experiment Result	10

2.4 – Interpretation of Results	10

2.5 – Conclusion	11

2.6 – Reflection	11

References	12

# Table of Tables

Table 1 – Experiment Array	3

Table 2 – Independent Variables	4

Table 3 – Dependent Variables	5

Table 4 – Control Variables	5

Table 5 – Populated Array	6

Table 6 – Results 1	6

Table 7 – Results 2	10

Table 8 – Mid-point calculation	10

# Table of Figures

Figure 1 – Front Position	7

Figure 2 – Anchor Position	7

Figure 3 – Front Position to Anchor Position interaction	8

Figure 4 – Stop Position	8

Figure 5 – Pull Back	9

Figure 6 – Anchor Position to Stop Position interaction	9

Figure 7 – Operator	10

# Section 1 – Information

## Background information

Genichi Taguchi was a Japanese engineer and statistician concerned with the quality of manufactured goods. To aid in this, he devised the Taguchi method [1] for use in the design of experiments when finding relationships between factors to achieve a robust optimum relationship between factors.

## 1.2 Example Array

For an experiment involving 5 variables, the array of experiments would take the following form:

Table 1 – Experiment Array

| Array |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Factor | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | A | B | AB | C | D | BC | E |
| Expt 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Expt 2 | 1 | 1 | 1 | 2 | 2 | 2 | 2 |
| Expt 3 | 1 | 2 | 2 | 1 | 1 | 2 | 2 |
| Expt 4 | 1 | 2 | 2 | 2 | 2 | 1 | 1 |
| Expt 5 | 2 | 1 | 2 | 1 | 2 | 1 | 2 |
| Expt 6 | 2 | 1 | 2 | 2 | 1 | 2 | 1 |
| Expt 7 | 2 | 2 | 1 | 1 | 2 | 2 | 1 |
| Expt 8 | 2 | 2 | 1 | 2 | 1 | 1 | 2 |

The first column lists which number experiment each row is. Column A and B each represent independent variables with 2 levels each (meaning they can be 2 different values). Column AB shows whether variables A and B are at the same level or not. Column C and D represent another 2 independent variables at 2 levels. Column BC shows whether B and C are at the same level. Column E represents a fifth independent variable.

If you were to complete every possible combination of variable levels would take 16 experiments but by utilizing the relationship between variables A and B as well as B and C allows you to know if either pair of variables being different from each other makes a difference to the result. It provides a more efficient means to get meaningful data.

# Section 2 – Experiment

## 2.1 – Objective

Carry out a practical experiment utilizing a catapult with many alterable characteristics to determine the key features to adjusting the range of the system.

## 2.2 – Method & Apparatus

### 2.2.1 – Apparatus

- Lean 6 Sigma Catapult [2]

- Projectile

- Tape measure

- Video recording device

### 2.2.2 – Method

- Fasten catapult to surface

- Extend tape measure from the base of catapult along firing direction

- Consult experiment table to set all variables on the catapult and decide which operator will conduct the test

- Load the projectile into the basket

- Pull the catapult to the intended angle

- Release promptly after reaching required angle

- Make note of rough impact zone along tape measure

- Begin video recording of impact zone and repeat steps 4 to 6

- View the recording in slow motion to accurately determine the distance

- Repeat steps 8 and 9 twice to gain a total of 3 results

- Take an average of the distances

- Return to step 3 for the next experiment until all intended experiments have been carried out

- Use the graphs produced in the spreadsheet to determine the key features of the system. Use this to determine the optimum settings for long distance.

### 2.2.3 – Variables

Independent:

Table 2 – Independent Variables

| Variable | Description |
| --- | --- |
| Band front position | Influences how far the band stretches therefore altering the stored energy before release |
| Band anchor position | Where the band is held onto the catapult |
| Stop position | The physical obstruction which prevents the catapult moving forward thereby releasing the projectile |
| Pull back | How far backwards the catapult is pulled before release |
| Operator | Person operating the catapult |

Dependant:

Table 3 – Dependent Variables

| Variable | Description |
| --- | --- |
| Horizontal distance | The distance travelled by the projectile after release from the catapult |

Control:

Table 4 – Control Variables

| Variable | Description |
| --- | --- |
| Method of measurement | The measurements were taken from slow motion video to remove the noise of human reactions when watching the projectile impact with the tape measure |
| Position of catapult | The catapult was fixed to the surface in the same position for all tests |
| Environment | All tests were conducted in the same room |
| Projectile | The projectile being used in the experiment |
| Catapult | The catapult being used in the experiment |

## 2.3 – Experimental Data

### 2.3.1 – Populated Array

Table 5 – Populated Array

| Array Populated |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Experiment | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | Band front position | Band Anchor position | AB Interaction | Stop Position | Pull back | BC Interaction | Operator |
| Expt 1 | 1 | 2 | 1 | 1 | 140 | 1 | A |
| Expt 2 | 1 | 2 | 1 | 3 | 170 | 2 | B |
| Expt 3 | 1 | 4 | 2 | 1 | 140 | 2 | B |
| Expt 4 | 1 | 4 | 2 | 3 | 170 | 1 | A |
| Expt 5 | 3 | 2 | 2 | 1 | 170 | 1 | B |
| Expt 6 | 3 | 2 | 2 | 3 | 140 | 2 | A |
| Expt 7 | 3 | 4 | 1 | 1 | 170 | 2 | A |
| Expt 8 | 3 | 4 | 1 | 3 | 140 | 1 | B |

### 2.3.2 – Experiment Results

Table 6 – Results 1

|  | Trial 1 | Trial 2 | Trial 3 | Average |
| --- | --- | --- | --- | --- |
| Experiment 1 | 83 | 76 | 87 | 82 |
| Experiment 2 | 239 | 248 | 183 | 223.3333 |
| Experiment 3 | 63 | 74 | 86 | 74.33333 |
| Experiment 4 | 174 | 174 | 179 | 175.6667 |
| Experiment 5 | 103 | 108 | 104 | 105 |
| Experiment 6 | 43 | 43 | 30 | 38.66667 |
| Experiment 7 | 78 | 74 | 79 | 77 |
| Experiment 8 | 51 | 53 | 51 | 51.66667 |

### 2.3.3 – Graphs

Figure 1 – Front Position

Figure 2 – Anchor Position

Figure 3 – Front Position to Anchor Position interaction

Figure 4 – Stop Position

Figure 5 – Pull Back

Figure 6 – Anchor Position to Stop Position interaction

Figure 7 – Operator

### 2.3.4 – Results 2

Table 7 – Results 2

|  | KF1 | KF2 |  | 1 | 2 | 3 | Ave |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Firing 1 | 1 | 140 | range | 40 | 40 | 40 | 40 |
| Firing 2 | 3 | 140 | range | 25 | 25 | 23 | 24.33333 |
| Firing 3 | 1 | 170 | range | 184 | 205 | 194 | 194.3333 |
| Firing 4 | 3 | 170 | range | 100 | 112 | 108 | 106.6667 |

### 2.3.5 – Mid-point Calculation

Table 8 – Mid-point calculation

| KF 1 :1 | KF2 :1 |
| --- | --- |
| 117.1667 | 32.16667 |
| KF 1 :2 | KF2 :2 |
| 65.5 | 150.5 |
| mid each factor |  |
| 91.33333 | 91.33333 |
| mid combined |  |
| 91.33333333 |  |

### 2.3.6 – Mid-point Experiment Result

Result for the final test was 90cm.

## 2.4 – Interpretation of Results

As seen in Figure 1 – Front Position and Figure 5 – Pull Back, the 2 critical factors in determining the range of the catapult were front position and pull back. This is visually displayed by the steep gradient between level 1 and 2 for these variables. This was then used to run a second set of tests to determine which combination of these levels would result in an optimum performance.

The next experiments were planned as seen in Table 7 – Results 2. The 2 levels for front position and pull back were to be tested in every possible combination with all other variables in the configuration that produced the better results according to the figures in 2.3.3 – Graphs. This added the following as control variables: Anchor position at 2, stop position at 3 and operator B carrying out the tests.

To ascertain if the relationship between the variables and distance was linear, a mid-point was calculated and then all variables were set to a value at the mid-point between the upper and lower level. This provided a target value, which if achieved would support a linear relationship between the variables and distance.

## 2.5 – Conclusion

The experiment correctly identified the critical factors in achieving a maximum distance without requiring exhaustive testing of every possible combination of settings. The calculated mid-point was very close to the measured distance in the final trial indicating that the data extracted from the experiments was accurate.

The optimum settings for the model of catapult used were as follows:

- Front Position: 1

- Anchor Position: 2

- Stop Position: 3

- Pull Back: 170°

- Operator: Operator B

## 2.6 – Reflection

The experiment could have been improved. One example is the use of human operators to set the pull back distance instead of using some form of mechanical release. A mechanical release would guarantee a consistent pull back distance therefore removing the noise of human inaccuracy. Though the measurements were taken via a slow-motion video, the measurements were not perfectly accurate as the projectile was fast moving and it was not always possible to ascertain the point of impact with the surface.

# References

| [1] | G. Taguchi, “Quality engineering in japan,” Communications in Statistics - Theory and Methods, vol. 14, 1985. |
| --- | --- |
| [2] | “Lean 6 Sigma Training Aids and Downloads,” Lean 6 Sigma, 03 07 2016. [Online]. Available: https://www.lean6-sigma.co.uk/training-aids/. [Accessed 10 04 2026]. |

Note: Band front position, anchor position and stop position were all a choice of 4 settings while the pull back was measured using a scale attached to the catapult in degrees.