| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 6 | LEFEL 6 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| May 2023 | MAI 2023 |

MODULE CODE: ACDA6001

Electronics II

TIME:  2 HOURS

ANSWER ANY FIVE QUESTIONS

Equipment or materials required – Calculator

Adnoddau neu ddefnyddiau sydd eu hangen - Cyfrifiannell

QUESTION 1

- Implement a  4:1 Multiplexer  for the function f=f(w, x, y, z)  and  represent the following.

  - Combinational Diagram					              [2 Marks]

  - Truth Table								[4 Marks]

  - Logic Diagram							[4 Marks]

- Write a VHDL code for 4:1 Mux using Data flow Model.	           		[10 Marks]

QUESTION 2

- Determine the reduced state diagram for the state diagram given below.

[6 Marks]

- The reduced state diagram, obtained from Question2(i), Determine the following

  - its state Assignment.						   	[1 Mark]

  - State Table.                                                                                 		 [3 Marks]

  - Excitation Table using any Flip- flop.				   [3 Marks]

  - Output Expressions.							   [3 Marks]

  - Draw its Logic Circuit						   [4 Marks]

QUESTION 3

- Consider two signed numbers -5 (Multiplicand) and 4 (Multiplier).

  - Multiply the above given numbers using 2’s Compliment method.    [4 Marks]

- Draw the Booths Algorithm flow chart for a 4 bit Multiplier.	     [5 Marks]

- Use the Booths Multiplier operation table  to verify the resultant of the product from question (a).						   	     [6 Marks]

- Write a VHDL code for a 2 X 2 Multiplier.			    		     [5 Marks]

QUESTION 4

- Design a serial Adder using D Flip-flop and draw its corresponding logic circuit. Consider any two 4- bit binary numbers which are stored in two shift registers A and B respectively. The result (sum) of the adder must be stored in register A. Use a Full adder to add the two numbers. Answer the following.

  - Block Diagram							[4 Marks]

  - State Diagram							[4 Marks]

  - Present state, Next state, output Table.				[4 Marks]

  - Excitation Table and  Simplified K- Map Expression.		[4 Marks]

  - Logic Diagram.							[4 Marks]

QUESTION 5

- For the functions  the F1 (x, y, z)=∑m  (0, 3, 5, 6)   and F2 (x, y, z)=∑m  (0, 1, 2, 4, 7).

Answer the following.

  - Draw  the truth tables for the functions F1 and F2			[5 Marks]

  - Logic  Expressions							[3 Marks]

  - Show how F1 and F2 can be implemented using one Decoder.	[6 Marks]

  - Design them using Programmable Logic Array (PLA).		[6 Marks]

QUESTION 6

- What is LUT in  FPGA?, explain the operation of two input  LUT seen in FPGA, with a help of a neat diagram.							[12 Marks]

- For a  3- input LUT of an FPGA Logic block and  determine the bits to be stored in its storage cell by using   the function  f= ABC+ABC+ABC+ABC to realize by answering the following

- Truth Table								[2 Marks]

- Design a  3- input LUT of an FPGA by assuming the value of inputs to be 001 respectively.								[6 Marks]

Diwedd/End