Computer Architecture and networks assignment 1

James Long

# Contents

Contents

Contents	1

Abstract:	3

Section 1: PC Specifications	4

1.1	CEO	4

1.2	HR Administrator	4

1.3	Sales Marketing	4

1.4	Finance Assistant	4

1.5	Film Editors	4

1.6	Media Engineers	4

1.7	Graphics/Animation Engineers	5

1.8	Art Designers	5

1.9	Web Designers / Developers	5

1.10 Remote Users	5

1.11 Media Technicians	5

1.12 Web and Graphics Design Interns	5

1.13 Total Cost	5

Section 2: Office Equipment	6

2.1 Servers	6

2.1.1 Web Server & Media Server	6

2.1.2 Email / Print Server	6

2.2 VoIP System	6

2.3 Video Conferencing System	6

2.4 Secure Wired/Wireless Connections	6

2.5 High Speed Internet Connection	7

2.6 Printers	7

2.7 Required Software	7

Total Cost	7

Section 3: Packet Tracer	8

Logical Overview	8

Physical Overview	9

Floor 1	10

Logical	10

Physical	11

Floor 2	12

Logical	12

Physical	13

Floor 3	14

Logical	14

Physical	15

Section 4: Appendices	16

Section 1:	16

PC Build 1	16

PC Build 2	18

Section 2	20

Headset	20

TV	20

Router	21

Switch	21

Broadband	21

Web & Media Server	22

Email / Print Server	26

Software	29

Bibliography	32

# Abstract:

In this document I will be suggesting hardware and software for the company known as DigiProd. The guidance in this document should help the company create a decision based on the technology I have suggested to them. This document will cover PC’s for 48 members of staff, web and print servers, any software which I feel would be beneficial to the company and screenshots of a packet tracer program showing that what I have suggested would work in a real-life environment.

# Section 1: PC Specifications

## CEO

For the CEO of the company I am suggesting using PC Build 1 (listed in appendices). This is because I believe that the CEO of the company won’t be needing that much power to complete his everyday tasks. He is likely to be using email, browsing the web and possibly creating or viewing reports, nothing too strenuous on the PC. His computer costs £479.79.

1 x PC Build 1 = £479.79

## HR Administrator

I also believe that the HR administrator won’t be doing anything too strenuous on the PC, so they have been suggested the same PC as the CEO (PC Build 1). They will likely be using email, browsing the web, creating spreadsheets and possibly editing databases. They will also need to use Sage Payroll meaning they need a good PC, but nothing to the specifications of PC Build 2 which is why I have given them PC Build 1.

1 x PC Build 1 = £479.49

## Sales Marketing

The Sales Marketing team would also be best suited to PC Build 1 because they will most likely be using email, Microsoft Office, browsing the web and possibly creating posters. They will not need a high spec machine like the film editors or the media engineers and that is why I have given them PC Build 1.

2 x PC Build 1 = £958.98

## Finance Assistant

Much like the HR Administrator there isn’t much specialist software the finance assistant will need to run. Programs like Sage Payroll and Microsoft Office programs don’t take much computing power, so they are suited well to PC Build 1.

1 x PC Build 1 = £479.49

## Film Editors

Unlike the previous members of staff, the film editors will need a high spec machine to run specialist software. Some of the software that the film editors will need to run includes Photoshop, Premier Pro and Sony Vegas Pro. These pieces of software require high spec machines with around 4-8GB of RAM, a high power dedicated graphics card along with a good processor to stop any lag while editing. Therefore, the film editors have been suggested PC Build 2.

4 x PC Build 2 = £3171.32

## Media Engineers

The media engineers are mostly gonging to be out of the office filming but when they are back inside the office they will need high specification machines to run any specialist software such as Premier Pro to check any footage and make sure it is useable. Therefore, I have suggested them PC Build 2

5 x PC Build 2 = £3964.30

## Graphics/Animation Engineers

The graphics / animation engineers will also need high spec machines because of the software they will be using. This includes Photoshop, Illustrator and possibly even Adobe Flash. These programs require high specification machines for them to run smoothly without any lag so that the user can complete their work without any problems. Therefore, I am suggesting PC Build 2.

4 x PC Build 2 = £3171.32

## Art Designers

The art designers will be using software such as Photoshop and Illustrator and while you can run these on a lower specification machine it will lag and stutter, therefore it makes more sense to give the art designers a higher specification machine to make their lives easier and so they can work more efficiently. Therefore, I am suggesting PC Build 2.

4 x PC Build 2 = £3171.32

## Web Designers / Developers

The web designers will be using specialist software such as Dreamweaver along with potentially creating scripts via Visual Studio or having remote access to the server using Microsoft Server Essentials. These programs will require a high spec machine, so I am suggesting PC Build 2.

10x PC Build 2 = £7928.30

## 1.10 Remote Users

The remote users won’t need to have a machine built for them because they will be using their own machine. Whether it is their own laptop, desktop or tablet they will be able to connect to the internet so they can access the cloud via Ethernet or Wi-Fi in the office or Wi-Fi/4G outside of the office.

## 1.11 Media Technicians

The media technicians in the office would be using specialist equipment and software on their machines to be checking the quality of the footage which was shot by the media engineers. They will also be looking after the film equipment. One program they would be using to check the footage would be Adobe Premier Pro, because of this I am suggesting PC Build 2. You could probably afford to go slightly higher spec on PC build 2

2 x PC Build 2 = 1585.66

## 1.12 Web and Graphics Design Interns

Just because the interns aren’t full members of staff does not mean they deserve lower spec machines than those who are full time members, therefore because they will be using the same programs as the full-time members of staff I am suggesting PC Build 2.

4 x PC Build 2 = £3171.44

## 1.13 Total Cost

33 x PC Build 2 = £26164.38

5 x PC Build 1 = £2398.95

Total Cost = £28563.33

# Section 2: Office Equipment

## 2.1 Servers

### 2.1.1 Web Server & Media Server

I am suggesting a custom-built server so that it can be built for purpose for what I think would be best for the company. Due to the nature of the company I am suggesting a server with a very high storage capacity. The server I have specified has 150TB of storage space so that users can be assigned around 500GB for personal files each, along with the hard drives in their machine. This would then leave around 120TB available for HD video and any other files. The server has very high specifications to “future-proof” it so that there is no need to upgrade the server for around 5 years. One example of this is providing the server with 192GB of RAM along with a 16 core 2.40GHz Intel Xeon processor. The full specifications have been placed in the appendix.

Price of server (Inc. VAT) = £17084.74

### 2.1.2 Email / Print Server

For the email server I am suggesting a standard off the shelf server from Dell. This is because a standalone server just for dealing with email and print requests won’t take much strain, but I wouldn’t want to slow down the original server with extra load. The specifications for the server are listed in the appendices.

Price of server (Inc. VAT) = £2578.39

## 2.2 VoIP System

There are several VoIP systems out there, but I think the simplest one to use would be Skype. This is extremely simple to set up for each user because all they need is a PC / Mobile and an email address. Once they set up they can simply call other people who they’ve added as friends or if they want to call a phone number they can add Skype Credit to do so. To make the call quality more consistent I have priced up 48 headsets (1 for each member of staff) to be added to the total bill.

48 x Headset = £911.52

## 2.3 Video Conferencing System

For the video conference room, I am suggesting a 42” TV which has information below in the specifications. Also, for this room I am suggesting buying a separate PC so that you wouldn’t need to keep moving a PC back and forth of one of the offices. I am suggesting using PC Build 1 simply because the PC won’t be used for anything other than running Skype and basic Office software and it should not be used for any specialist software.

1 x PC Build 1 = £479.79

1 42” TV = £249.00

## 2.4 Secure Wired/Wireless Connections

I am suggesting using wireless routers because it will save money when it comes to setting them up and there won’t need to be cables littered across the office. The routers specifications are in the appendices section. The reason I have chosen these routers is because they support Dual Band connection so that employees can connect via the 2.4GHz band and the 5GHz band. Along with this I am suggesting a network switch for the web designer’s office because they will be the ones spending most of the time on the internet, so this will give them a more stable connection. 

4 x Router = £719.96

1 x Switch = £56.99

## 2.5 High Speed Internet Connection

For a secure high-speed internet connection, I am suggesting a contract with Virgin Media. This is because they provide the fastest UK connection with up to 350Mbps which would be enough for the office, even if there are several users online and downloading at the same time. On top of this they are offering up to 20Mbps upload speed which is good for uploading and backing up files to the cloud along with making the conference calls and using Skype smoother. Virgin Media are charging £42 per month for 12 months. Therefore, I have put the cost for the 12-month contract including the setup fees below.

12 Months Virgin Media broadband = £524

## 2.6 Printers

The printer I have suggested is an all-in-one printer meaning it can copy, print and faxing. This will make it easier other than having to have a different machine for each purpose. Along with this it also has network connectivity, so you can link them all up to the network to print from any machine in the office.

4 x Printer = £679.96

## 2.7 Required Software

The required software for all the machines includes a subscription to office 365 for 48 users. It is £9.40 per month for each user. Alongside this you will need 49 Windows 10 licenses which are priced at £19.99 each. The most expensive software would be Adobe Creative Cloud which is priced at £1947.00 per month for 33 users. 15 Licenses of Sony Vegas Pro which would be used by the film editors, media engineers etc. costs £186.30 per month

48 x Office 365 (£9.40) = £451.20 pm

49 x Windows 10 License = £979.51

33 x Adobe Creative Cloud = £1947.00 pm

15 x Sony Vegas Pro = £186.30

## Total Cost

48 x Office 365 (£9.40) = £451.20 pm

49 x Windows 10 License = £979.51

33 x Adobe Creative Cloud = £1947.00 pm

15 x Sony Vegas Pro = £186.30pm

12 Months Virgin Media broadband = £524

1 x PC Build 1 = £479.79

1 42” TV = £249.00

48 x Headset = £911.52

Price of server (Inc. VAT) = £2578.39

Price of server (Inc. VAT) = £17084.74

33 x PC Build 2 = £26164.38

5 x PC Build 1 = £2398.95

4 x Printer = £679.96

4 x Router = £719.96

1 x Switch = £56.99

Total Cost £55411.30

# Section 3: Packet Tracer

[12]

## Logical Overview

I have sectioned off the 3 floors into clusters inside packet tracer to make it easier to manage the 3 floors in the logical overview. Each router is connected to another via a copper cross over cable.  The servers are then connected to the third-floor router inside the Server/Printer room

## Physical Overview

This is the Physical overview which shows the 3 floors as separate buildings and is showing the connections are successful between routers and buildings. This shows a working network in a real-life situation.

## Floor 1

### Logical

This is the logical display showing that all the machines are connected wirelessly to the router. The only wire is a copper cross-over cable which is connecting to another router off-screen.

### Physical

This photo is showing that the router is wired up to another which is off screen as it is in another building/floor along with all the PC’s and Printers in a physical location throughout the office. All these machines are wirelessly connected to the router.

## Floor 2

### Logical

On the second floor there are 2 routers and a switch. The reason I have done this instead of using just one router is to stop any overload of traffic coming through the router. Alongside this the connection wouldn’t be very stable across this floor with the amount of PC’s on one floor. DigiProd 2 is connected to a switch which then has several ethernet cables going into each PC in the Web Developer / Designer’s PC’s. Everything else on this floor other than the printer is connected wirelessly.

### Physical

This is the physical view of the second floor, as you can see there is a lot of ethernet cables connected to a switch which is connected to a router in the Web Designers / Developer’s office while everything else other than the printer is all connected wirelessly.

## Floor 3

### Logical

These devices on the third floor are all wirelessly connected to the DigiProd 4 router. These devices might not be in the office always because there is a remote user office on-site. This is a viewpoint where all remote users are in the office working on projects with their own devices such as laptops, tablets and smartphones.

### Physical

As you can see in the above screenshot the Web Servers are hooked up to DigiProd 4 and everything else is wirelessly connected to the same router.

# Section 4: Appendices

## Section 1:

### PC Build 1

| Job Role: CEO |  |
| --- | --- |
| Location: On-site |  |
| Component | Recommendation |
| Processor: (Manufacturer, Model, Speed) | AMD A9-9410 Dual-Core 2.9GHz / 3.5GHz (Turbo Boost) 1MB Cache |
| Memory: (Type, Amount) | 8GB DDR4 (2133MHz) |
| Hard Drive: (Type, Capacity) | 1TB 7200rpm |
| CD-ROM/DVD: (Read, Read/Write, Speed) | DVD RW |
| USB Ports: (Number, Location) | USB 3.0 x 2 USB 2.0 x 3 |
| Video Card: (Manufacturer, Model, Video RAM) | Integrated Graphics |
| Sound Card: (Manufacturer, Model) | Dolby Advanced Audio |
| Modem: (Internal/External, Speed, Standard) | N/A |
| Network Card(s): (Ethernet: speed, Wireless, Standard) | Wireless-AC |
| Operating System: (Manufacturer, Version, Compatibility) | Windows 10 |
| Monitor: (Size, Resolution, Refresh Rate) | 21.5” 60Hz 1920x1080 |
| Printer: (Manufacturer, Model, Type, Speed) | N/A |
| Speakers: (Manufacturer, Model, Type) | 3 W x 2 |
| Internet Connection: (USB, Ethernet, Wireless) | Wireless-AC Gigabit Ethernet 10/100/1000 |

.

### PC Build 2

| Job Role: Web and Graphics Design Interns |  |
| --- | --- |
| Location: On-site |  |
| Component | Recommendation |
| Processor: (Manufacturer, Model, Speed) | Intel - Core i5-8600K 3.6GHz 6-Core Processor |
| Memory: (Type, Amount) | G.Skill - Ripjaws 4 series 8GB (2 x 4GB) DDR4-2400 Memory |
| Hard Drive: (Type, Capacity) | Western Digital - WD Blue 750GB 2.5" 5400RPM Internal Hard Drive |
| CD-ROM/DVD: (Read, Read/Write, Speed) | Lite-On - iHAS124-04 DVD/CD Writer |
| USB Ports: (Number, Location) | USB 3.0 x 4 USB 2.0 x 2 |
| Video Card: (Manufacturer, Model, Video RAM) | MSI - GeForce GTX 1050 2GB GAMING X 2G Video Card |
| Sound Card: (Manufacturer, Model) | On Board Audio |
| Modem: (Internal/External, Speed, Standard) | N/A |
| Network Card(s): (Ethernet: speed, Wireless, Standard) | TP-Link - TG-3468 PCI-Express x1 10/100/1000 Mbps Network Adapter |
| Operating System: (Manufacturer, Version, Compatibility) | Windows 10 |
| Monitor: (Size, Resolution, Refresh Rate) | AOC - G2260VWQ6 21.5" 1920x1080 75Hz Monitor |
| Printer: (Manufacturer, Model, Type, Speed) | N/A |
| Speakers: (Manufacturer, Model, Type) | N/A |
| Internet Connection: (USB, Ethernet, Wireless) | Wireless-AC Gigabit Ethernet 10/100/1000 |

[1]

## Section 2

### Headset

[2]

### TV

[3]

### Router

### Switch

[5]

### Broadband

[6]

### Web & Media Server

[7]

### Email / Print Server

[8]

### Software

[9]

[10]

[11]

[13]

# Bibliography

[1]

“PCPartPicker,” System Builder - Core i5-8600K 3.6GHz 6-Core, GeForce GTX 1050 2GB GAMING X 2G, TESSERACT SW ATX Mid Tower - PCPartPicker United Kingdom. [Online]. Available: https://uk.pcpartpicker.com/list/LQGCzY. [Accessed: 03-May-2018].

[2]

“Mpow PC Headset, Multi-Use USB Headset & 3.5mm Skype Headset: Amazon.co.uk: Computers & Accessories,” Child And Family Assessment in Social Work Practice (Social Work in Action): Amazon.co.uk: Sally Holland: 9780761949022: Books. [Online]. Available: https://www.amazon.co.uk/Mpow-Headset-Multi-Use-3-5mm-Skype/dp/B06XX7SYP2/ref=sr_1_1?ie=UTF8&qid=1525374014&sr=8-1&keywords=mpow pc headset. [Accessed: 03-May-2018].

[3]

“JVC LT-42C550 42’ LED TV,” currys. [Online]. Available: https://www.currys.co.uk/gbuk/tv-and-home-entertainment/televisions/televisions/jvc-lt-42c550-42-led-tv-10142379-pdt.html. [Accessed: 03-May-2018].

[4]

“TP-LINK Archer VR2800 Wireless Modem Router - AC 2800, Dual-band,” pcw. [Online]. Available: https://www.pcworld.co.uk/gbuk/computing-accessories/networking/network-routers-and-switches/routers/tp-link-archer-vr2800-wireless-modem-router-ac-2800-dual-band-10165478-pdt.html. [Accessed: 03-May-2018].

[5]

“TP-LINK TL-SG1016D Network Switch - 16 port,” currys. [Online]. Available: https://www.currys.co.uk/gbuk/computing-accessories/networking/network-routers-and-switches/network-switches/tp-link-tl-sg1016d-network-switch-16-port-21429272-pdt.html. [Accessed: 03-May-2018].

[6]

“Broadband only,” Broadband Only Deals | No Landline Required | Virgin Media. [Online]. Available: http://www.virginmedia.com/shop/broadband/broadband-only.html. [Accessed: 03-May-2018].

[7]

“CyberStore 316S 12GB/s Storage Server ®,” 3U, 16 Drive Bays, SAS / SATA Rackmount Storage Server. [Online]. Available: http://www.broadberry.co.uk/storage-storage-servers/cyberstore-316. [Accessed: 03-May-2018]

[8]

“PowerEdge R530 2-socket Rack Server | Dell UK,” Trusted Platform Module (TPM) Overview - KB Article - 274113 | Dell US. [Online]. Available: http://www.dell.com/en-uk/work/shop/servers-storage-and-networking/smart-value-flexi-r530-8x35-e5-2620v4-1x16gb-2x1tb-72k-sata-h330-idrac8-express/spd/poweredge-r530/per53003?selectionState=eyJGUHJpY2UiOjIxNDguNjYsIk9DIjoicGVyNTMwMDMiLCJRdHkiOjEsIk1vZHMiOlt7IklkIjoxNTYwLCJPcHRzIjpbeyJJZCI6IjMyR0JNTSIsIlByaWNlIjo3MTQuMH1dfSx7IklkIjoxNjUwLCJPcHRzIjpbeyJJZCI6IjU0MDc3NSJ9XX0seyJJZCI6MTY1MiwiT3B0cyI6W3siSWQiOiI1MzUyMjcifV19LHsiSWQiOjE2NTEsIk9wdHMiOlt7IklkIjoiNTM1NzA1IiwiUHJpY2UiOjkwMC4wfV19LHsiSWQiOjE1NzAsIk9wdHMiOlt7IklkIjoiNFRBNzIiLCJQcmljZSI6NDAxLjB9XX0seyJJZCI6MTUzNCwiT3B0cyI6W3siSWQiOiJVRUZJQiJ9XX0seyJJZCI6MTYyMSwiT3B0cyI6W3siSWQiOiI1MTg5NTEiLCJQcmljZSI6MTMuMH1dfSx7IklkIjoxNTIwLCJPcHRzIjpbeyJJZCI6Ikk4RVhQIiwiUHJpY2UiOjE1NC4wfV19LHsiSWQiOjE1MTQsIk9wdHMiOlt7IklkIjoiT0JOSUMifV19XX0=. [Accessed: 03-May-2018].

[9]

“Start your video production journey with VEGAS Pro 365,” VEGAS Pro 365. [Online]. Available: https://www.vegascreativesoftware.com/gb/sem/vegas-pro-365/?cHash=88ba0c657c723b2ee0292a583b04da6e. [Accessed: 03-May-2018]

[10]

“Discover the Creative Cloud experience,” Adobe Captivate - Welcome to the world of smart eLearning authoring. [Online]. Available: https://www.adobe.com/uk/creativecloud/plans.html?plan=team&step=2&store_code=gb&promoid=BJ4T3KF8&mv=other. [Accessed: 03-May-2018].

[11]

“WINDOWS 10 PROFESSIONAL [LICENSE] – Softwaregeeks.co.uk,” Softwaregeeks.co.uk. [Online]. Available: https://softwaregeeks.co.uk/product/windows-10-pro/?gclid=CjwKCAjwlIvXBRBjEiwATWAQIlLQSClvdfzDv7roTOpbrvFrDrWJVXJXcYtOziTKZ7Wv5S_DVsYvHRoCwzAQAvD_BwE. [Accessed: 03-May-2018].

[12]

DIPEN618, “configuring wireless network in packet tracer,” YouTube, 26-Apr-2011. [Online]. Available: https://www.youtube.com/watch?v=uB6iBSlAkIc. [Accessed: 03-May-2018].

[13]

“Get the most from Office with Office 365,” Project Management Software | Microsoft Project. [Online]. Available: https://products.office.com/en-gb/compare-all-microsoft-office-products?tab=2. [Accessed: 03-May-2018].