MODULE CODE: ACCA5024

Advance Networking Final Exam Answer Sheet

Student Name: Florin-Cristian Matic

Student ID: 2029128

Date: 21/11/2022

- I- IPv4 Addressing Table

| Specification | Subnet A | Subnet B |
| --- | --- | --- |
| Subnet IP address | 192.168.202.0 | 192.168.202.128 |
| Subnet mask (slash notation) | /26 | /25 |
| Subnet mask (dotted decimal) | 255.255.255.192 | 255.255.255.128 |
| 1st Host IP address | 192.168.202.1 | 192.168.202.129 |
| Last Host IP address | 192.168.202.62 | 192.168.202.254 |
| Number of usable hosts per subnet | 62 | 126 |
| Broadcast address | 192.168.202.63 | 192.168.202.255 |

- II- IPv4 Address assignment

| Device | IP address | Subnet Mask | Gateway |
| --- | --- | --- | --- |
| PC-A | 192.168.202.62 | 192.168.202.192 | 192.120.2.1 |
| R1-G0/0/0 | 192.168.202.129 |  | 192.120.2.129 |
| R1-G0/0/1 | 192.168.202.1 | 255.255.255.192 | 192.120.2.1 |
| S1 | 192.168.202.2 | 255.255.255.192 | 192.120.2.1 |
| PC-B | 192.168.202.254 | 255.255.255.128 | 192.168.202.129 |

- III- IPv6 Addressing Table

| Assigned to Interface | IPv6 Subnet Address | Prefix Length |
| --- | --- | --- |
| R1-G0/0/1 | 2001:da8:acbd:0001::0 | /64 |
| R1-G0/0/0 | 2001:da8:acbd:0002::0 | /64 |

- IV-IPv6 Address assignment

| Device | IPv6 address | Prefix Length | Gateway |
| --- | --- | --- | --- |
| R1-G0/0/0 | 2001:da8:acbd:0002::1 | /64 | FE80::1 |
| R1-G0/0/1 | 2001:da8:acbd:0001::1 | /64 | FE80::1 |
| S1 | 2001:da8:acbd:0001::2 | /64 | 2001:da8:acbd:0001::1 |

# Task 2: Device Configuration

- Hostname

- Encrypted privileges EXEC password

- Secure the console access password

S1#configure terminal<OR>config t

Enter configuration commands, one per line. End with CNTL/Z.

S1(config)#line console 0

S1(config-line)#password ccnaconpass

S1(config-line)#login

S1(config-line)#exit

S1(config)#exit

- Minimum length passwords

R1(config)# security passwords min-length 10

R1(config)#

- Create an administrative user

R1(config)#username admin password admin1pass

R1(config)#

- Configure an MOTD banner

R1(config)#ba

R1(config)#banner motd “ No Unauthorised Access!”

- Encrypt all passwords

R1(config)#service pass

R1(config)# service password-encryption

- Disable IP domain lookup

R1(config)#no ip do

R1(config)#no ip domain-lookup

R1(config)#

- Enable synchronous logging

R1 (config-line) #logging sy

R1 (config-line) #logging synchronus

R1 (config-line) #exit

S1 (config-line) #logging sy

S1 (config-line) #logging synchronus

S1 (config-line) #exit

- Configure router’s interfaces

R1 (config) #int gigabitEthernet 0/0/0

RI (config-if)fip add 192.168.202.129 255.255.255.128

R1 (config-if)fno shut

R1 (config-if) #

SLINK-5-CHANGED: Interface GigabitEthernet0/0/0, changed state to up

SLINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0, changed state to up

R1 (config-if) fint gi

RI (config-if) texit

(RI (config) tint g

RI (config) fint gigabitEthernet 0/0/1

RI (config-if)fip add 192.168.202.1 255.255.255.192

R1 (config-if)|no shut

RI (config-if)‡

BLINK-5-CHANGED: Interface GigabitEthernet0/0/1, changed state to up

SLINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/1, changed

state to up

RI (config-if) #

R1 (

- Configure switch’s management (SVI) interface

S1 (config-if)#ip add 192.168.202.2 255.255.255.192

S1 (config-if) #no shu

S1 (config-if) #no shutdown

S1 (config-if) #

SLINK-5-CHANGED: Interface Vlan1, changed state to up

SLINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

S1 (config-if) #

# Task 3: Verify Network connectivity

| From | To | Protocol | IP Address | Ping Results |
| --- | --- | --- | --- | --- |
| PC-A | R1 G0/0/0 | IPv4 |  |  |
| tftttttttPC-A | R1 G0/0/0 | IPv6 |  |  |
| PC-A | R1 G0/0/1 | IPv4 |  |  |
| PC-A | R1 G0/0/1 | IPv6 |  |  |
| PC-A | S1 VLAN 1 | IPv4 |  |  |
| PC-A | S1 VLAN 1 | IPv6 |  |  |
| PC-A | PC-B | IPv4 |  |  |
| PC-A | PC-B | IPv6 |  |  |
| PC-B | R1 G0/0/0 | IPv4 |  |  |
| PC-B | R1 G0/0/0 | IPv6 |  |  |
| PC-B | R1 G0/0/1 | IPv4 |  |  |
| PC-B | R1 G0/0/1 | IPv6 |  |  |
| PC-B | S1 VLAN1 | IPv4 |  |  |
| PC-B | S1 VLAN1 | IPv6 |  |  |