ACDA6020 LAB SESSION #4

DETERMINATION OF MELTING POINT

AIM OF THE EXPERIMENT: Determine the melting point of four methyl-3-nitrobenzoate samples synthesised in Cranfield to evaluate the purity.

INTRODUCTION

The melting point is defined as a temperature at which a given compound changes from a solid to a liquid and it is one of physical properties by which such compound can be identified. Melting point can also be used to determine the purity or indicate the changes in crystal structure.

Pure organic solids melt at single temperature and presence of any (even very tiny) amount of impurity can cause the melting point to change and /or spread out.

It is common though, that even for pure compound the melting point is in the range of 2-3 °C around the literature value. The first sign of melting is when the crystal structure of the sample begins to form liquid drops, and ends when the entire sample is liquid.

Recrystallisation is a technique used to purify solid compounds. It involves dissolving the compound in a suitable hot solvent to form a solution and then slowly cooling it to allow the pure compound to crystallise out. Impurities either remain dissolved in the solvent or are filtered off during the process. This method takes advantage of differences in solubility between the desired product and its impurities.

In the case of methyl-3-nitrobenzoate, recrystallisation from hot ethanol removes impurities such as unreacted starting materials or side products formed during synthesis. By dissolving the product in a hot solvent and cooling it, pure crystals of methyl-3-nitrobenzoate form, leaving impurities behind.

In this experiment you will measure the melting point of four samples of methyl-3-nitrobenzoate synthesised in Cranfield to determine the purity of the final product of nitration.

PROCEDURE

Put on the lab coat, safety goggles, and gloves. Make sure that long hair is tied.

IMPORTANT: Record all your observations, calculations, and conclusions in the data sheets at the end of this script.

Lab tools and supplies:

Cole-Parmer M200 Melting Point Microscope (MPM)

Glass capillaries

Samples of methyl-3-nitrobenzoate synthesised and purified in Cranfield.

Operating procedure will be discussed in the lab however the main points are:

- Set up the MPM device by placing it on a stable surface, connecting to power, and switching it on. The display will show the block temperature, and the sample chamber light will activate.

- Prepare samples for the analysis bu filling a capillary tube with each sample .

- Set Plateau Temperature:

  - Press and hold the 'Plateau Set' button; adjust the temperature with arrow keys to 10°C below the expected melting point.

  - Release the button to confirm the setting.

- Place the capillary tube into a sample hole in the block and make sure that it is visible through the lens.

- Press 'Start' to heat quickly to the plateau temperature. The plateau light will turn on once stabilized.

- To observe melting point, press 'Start' again to begin slow ramping (2°C/min). Observe through the lens and record the temperature at the first sign of melting.

- Press 'Stop' after recording. Allow the device to cool before the next use.

Safety Notes:

- Handle with care; the block becomes very hot.

- Use protective gear and ensure good ventilation.

POTENTIAL HAZARD

|  | Hazards |
| --- | --- |
| ### Health Hazards | Methyl-3-nitrobenzoate - may be harmful if swallowed, inhaled, or absorbed through the skin. - can cause irritation to the skin, eyes, and respiratory tract. - prolonged exposure may lead to sensitisation or allergic reactions in susceptible individuals. |
| Flammability and Combustibility | While not highly flammable, methyl-3-nitrobenzoate is combustible and can release toxic fumes, such as carbon monoxide, carbon dioxide, and nitrogen oxides, when burned |
| Environmental Hazards | It may pose a risk to aquatic environments if released improperly, as nitroaromatic compounds can be toxic to aquatic life. |
| Reactivity | Under certain conditions, nitro compounds can act as oxidising agents, potentially posing a hazard when in contact with strong reducing agents or combustible materials. |

DATA SHEET to RECORD OBSERVATIONS, CALCULATIONS and RESULTS

STUDENT’S NAME: ____________________________________________________ PARTNERS’ NAMES: ____________________________________________________

REPORT IS DUE DECEMBER 13TH, 2024

submitted via MOODLE as ONE .doc file

PROCEDURE

Put on the lab coat, safety goggles, and gloves. Make sure that long hair is tied.

IMPORTANT: Record all your observations, calculations, and conclusions in the data sheets at the end of this script.

Lab tools and supplies:

Cole-Parmer M200 Melting Point Microscope (MPM)

Glass capillaries

Samples of methyl-3-nitrobenzoate synthesised and purified in Cranfield.

Operating procedure will be discussed in the lab however the main points are:

- Set up the MPM device by placing it on a stable surface, connecting to power, and switching it on. The display will show the block temperature, and the sample chamber light will activate.

- Prepare samples for the analysis bu filling a capillary tube with each sample .

- Set Plateau Temperature:

    - Press and hold the 'Plateau Set' button; adjust the temperature with arrow keys to 10°C below the expected melting point.

    - Release the button to confirm the setting.

- Place the capillary tube into a sample hole in the block and make sure that it is visible through the lens.

- Press 'Start' to heat quickly to the plateau temperature. The plateau light will turn on once stabilized.

- To observe melting point, press 'Start' again to begin slow ramping (2°C/min). Observe through the lens and record the temperature at the first sign of melting.

- Press 'Stop' after recording. Allow the device to cool before the next use.

Safety Notes:

- Handle with care; the block becomes very hot.

- Use protective gear and ensure good ventilation.

REPEAT THE MEASUREMENTS TWO TIMES FOR EACH SAMPLE.

Literature values of melting points:

|  | melting point, °C |
| --- | --- |
| methyl benzoate |  |
| methyl-3-nitrobenzoate |  |

Experimental values of melting points

| Sample | appearance | trial 1 | trial 2 |
| --- | --- | --- | --- |
|  |  | mp, °C | mp, °C |
| A |  |  |  |
| B |  |  |  |
| C |  |  |  |
| D |  |  |  |

OBSERVATIONS AND OTHER RELEVANT INFORMATION

Guidance for lab report:

- Title page, TOC, etc

- Purpose (Aim) of the experiment

- Hypothesis: based on the experience and knowledge, what do you expect to observe? Why?

- Variables: independent, dependent, controlled

- Theory (background information)

  - Basic concepts, principles and laws, definitions

  - Chemical equation and formulas for calculations  (if relevant)

  - Examples of applications/importance of the method in OME supported by literature

- Observations/Data (as of the Data Sheet provided)

- Calculations (where relevant)

- Results and Discussion: How do the observations compare to your hypothesis  — do they match? How do the observations compare to the literature values?  What are primary sources of experimental errors?

- Conclusion

- References (IEEE)

- Appendix

  - Chemicals and equipment

  - Procedure

END OF LAB REPORT