Formula Sheet

BEng.(Hons.) in Electrical and Electronics Engineering

ACDA5001

| Ohms Law |  | Series Resistors RT=R1+R2+…+Rn | Potential Divider V1=VR1R1+R2 |
| --- | --- | --- | --- |
|  |  | Parallel Resistors 1RT=1R1+1R2+…+1Rn | Current Divider I1=IR2R1+R2 |
| Capacitive Reactance XC=12πfC XC=-j1ωC=-j12πfC | Inductive Reactance XL=2πfL XL=jωL XL=j2πfL | Series Capacitors 1CT=1C1+1C2+…+1Cn | Series Inductors LT=L1+L2+…+Ln |
| Angular Frequency ω=2πf | Capacitance C=ChargeVoltage | Parallel Capacitors CT=C1+C2+…+Cn | Parallel Inductors 1LT=1L1+1L2+…+1Ln |
| Frequency f=1t | Voltage across Inductor V=Ldidt VL=I×jXL |  |  |
| Voltage Equation V=Vmsin2πft V=Vmsinωt V=Vmsinωt+ϕ | Current Equation I=Imsin2πft I=Imsinωt I=Imsinωt+ϕ | RMS value of Current I=Irms=Im2 Irms=VXC=VXL=VR | RMS value of Voltage V=Vrms=Vm2=VZ Vrms=IXC=IXL=IR |
| Form Factor = RMS valueAverage value | Peak factor =Maximum valueRMS value | Power Factor PF=cosϕ Cosϕ=RZ | Peak to Peak Voltage Peak to Peak voltage = 2×Amplitude Peak Current Im=VmR Im=VmωL=VmXL=VmXC |
| Active /Real Power P=VI Cos∅ | Reactive power Q=VI Sin∅ Q=SSin∅ Q=I2X=V2X | Apparent Power S=P+jQ S=P2+Q2 S=VI S=VI* |  |
| Transformer Efficiency η=PSPP |  |  |  |
| Transformer VPVS=NPNS= ISIP IP=PPVP | Instantaneous Current I=VR=VZ I=1Lvdt I=CdVdt | Instantaneous Voltage V=IR=IZ V=Ldidt V=1CIdt | Instantaneous power P=V×I PL=IL2RL P=VmImsin2ωt P=-VmIm2sin2ωt P=VmIm2sin2ωt |
| Impedance Z=VI=VmIm=R Z=XL=ωL=2πfL Z=1ωC=XC=12πfC Z=R+jX Z=R2+XL2 Z=R2+XC2 Z=R2+(XL-XC)2 | Voltage Drop across capacitor VC=IXC VC=I×(-jXC) | Phase Angle Phase angle=tT×360 ϕdeg=ϕrad×180π ϕ=tan-1XLR ϕ=tan-1(-XCR) ϕ=tan-1(XC-XLR) ϕ=tan-1(XL-XCR) ϕ=∠V-∠I ϕ1=tan-1VLVR ϕ1=Sin-1XLZLR ϕ1=Cos-1RZLR ϕ=tan-1(ILRsinϕ1∼ICILRCosϕ1) ϕ=Cos-1(ILRCosϕ1I) |  |
|  |  |  | From the Current triangle tanϕ=ILIR Cosϕ=IRI Sinϕ=ILI tanϕ=ICIR Cosϕ=IRI Sinϕ=ICI Sinϕ=ICIL |
|  | Voltage Drop across an Inductor VL=IXL |  |  |
|  | Voltage V2=VR2+(VL-VC)2 V2=VR2+(VC-VL)2 |  |  |
| From the Voltage triangle Voltage =VR2+VL2 tanϕ =VLVR Sinϕ =XLZ | Average Power Pavg=VrmsIrms Pavg=VmIm2cosϕ |  |  |
| Current I=IR2+IL2 I=IR2+IC2 I=IC-IL I=IC+IL I=ILRCosϕ12+ (ILRsinϕ1∼IC)2 | Inductor Current IL=VXL ILR=VZLR |  |  |
|  |  |  | Resonant Frequency fr=12πLC fr=12π1LC-R2L2 |
|  |  | Current at Resonance Ir=VRD=VRCL |  |
|  | Capacitor Current IC=VXC | Dynamic Resistance RD=VIr=LRC | Power in three phase P=3VLILCosϕ P=3IP2RP |
| Line Current IL=3Ip | Apparent Power -3phase S=3VLIL | Line Voltage VL=3Vp |  |
| Capacitive Transients Time Constant τ=RC Steady state current I0=VR Charging VC=V1-e-tτ i=VRe-tRC Discharging VC=V0e-tτ i=-VRe-tRC | Inductive Transients Time Constant τ=LR Steady state current I0=VR Charging VL=Ve-RtL i=I01-e-RtL Discharging i=VRe-RtL | Q Factor Q=XLR Q=XCR Q=fRfU-fL Q=1RLC Q=ICIr Q=2πfrLR | Phase Current IP=VPR |
|  |  |  | Efficiency =Power OutputPower Input Efficiency =Output Input |
|  |  |  | Terminal Voltage V=ILRL V=VL+Vdrop |
|  |  |  | Generated EMF E=V+IaRa |
| Area of Triangle A=bh2 | Area of Rectangle A=bh | Average value Unsymmetrical =Area-One cycleT Symmetrical =Area-half cycleT | Armature Current Ia=If+I |
| Polar to Rectangular Coordinate X component =rcosϕ and Y component =rsinϕ Rectangular representation =X+jY =rcosϕ + jrsinϕ |  | Rectangular to Polar Coordinates r=X2+Y2 ϕ=tan-1(YX) Polar Representation= r∠ϕ =X2+Y2∠tan-1(YX) |  |
| Rectangular Co-ordinates If P=X1+jY1 and Q=X2+jY2 Addition R=P+Q=X1+X2+j(Y1+Y2) Subtraction R=P-Q=X1-X2+j(Y1-Y2) |  | Multiplication of polar co-ordinates If P=r1∠ϕ1 and Q=r2∠ϕ2 P×Q=r1∠ϕ1×r2∠ϕ2 =[r1×r2]∠ϕ1+ϕ2 Division of Polar Co-ordinates PQ=r1∠ϕ1r2∠ϕ2=r1r2∠ ϕ1 -ϕ2 |  |
| Complex Number can be expressed as :rejϕ ejϕ=cosϕ+jsinϕ e-jϕ=cosϕ-jsinϕ rejϕ = r∠±ϕ |  | Polar Co-ordinates If I=Imsinωt+ϕ, r=Im , ϕ=ϕ | Rectangular Co-ordinates If I=Imsinωt+ϕ, X- Component = Imcosϕ Y- Component = ImSinϕ |

Trigonometric Formula

Sinθ=Opposite sideHypotenuse

Cosθ=Adjacent sideHypotenuse

tanθ=Opposite sideAdjacent side

Differentiation Formula