| LOCATION: Swansea | LLEOLIAD: abertawe |
| --- | --- |
| LEVEL 4 | LEFEL 4 |
| EXAMINATION PERIOD | CYFNOD ARHOLIADAU |
| JANUARY 2023 | IONAwR 2023 |

MODULE CODE: ACDB4007

Physics

TIME:  2 HOURS

answer ALL QUESTIONS

Equipment or materials required – Calculator, formula sheet

Adnoddau neu ddefnyddiau sydd eu hangen – cyfrifiannell , taflen fformiwla

Q1

What are the essential assumptions made in Gurney’s theory of terminal velocities of an explosive charge?						(6 marks)

Q2 (20 marks)

- A body is rotating at a speed of 5 revolutions per minute.  Determine to 2 D.P. the angular velocity in radians per sec. Give your answer to 2 D.P.		(4 marks)

- Find the resultant force for the system of forces shown in Figure 1. Give your answers to 2 D.P.

9 N

11 N

30°

30°

60°

8  N

Figure 1

(16 marks)

Q2

- A body of mass 200 g is dropped from a vertical height of 7.5 m. Calculate the speed at impact on ground. Assume acceleration due to gravity g = 9.8 ms-2. Express your answer to two decimal places.				(5 marks)

- A projectile of mass 25 g penetrates a target to a depth of 0.4 m. If the speed at initial impact is 800 ms-1 determine the resistive force that the body exerts on the projectile, assuming uniform resistance					(5 marks)

Q4 (20 marks)

- Using simple sketches, compare the stress- strain relationship for a ductile metal and a brittle material.							(6 marks)

- An applied force stretches a uniform wire. If the change in length is 5x10-3 m and the strain was 0.03, calculate the original length of the wire.		(4 marks)

- A vertical uniform wire has Modulus of Elasticity 150 x 109 pascals, cross-sectional area 0.2 cm2 and natural length 2.5 m. Calculate the stress and strain values when a mass of 80 kg is suspended from one end. Express your answer to two decimal places.	 Assume acceleration due to gravity g = 9.8 ms-2.

(10 marks)

Q5 (20 marks)

- Describe the different ways thermal energy can be transported													(6 marks)

- How much energy is required to raise the temperature of a body with specific heat 300 J kg-1 K-1 and mass 1.5 kg through a temperature increase of 30 K? Express your answer to two decimal places.					(7 marks)